3GPP TSG-RAN WG4 Meeting #114                                                              R4-2501967

Athens, GR, 17th – 21st February, 2025

Source:	Samsung

Title:	Further discussion on general aspects for SBFD system

Agenda item:			7.23.1

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly, from RAN4 perspective, it is tasked to specify BS RF, necessary RRM and demodulation requirements.

In this contribution, we would like to provide our further analysis and viewpoints on the general aspects including RAN4 aspects for SBFD system parameters.

# 2. RAN4 aspects for SBFD system parameters

#### 2.1 Specification implementation of operating bands support for SBFD

Based on the discussion in RAN4#111, SBFD was recognized as “a feature with requirements of which can potentially be applied to all TDD band”, and the declaration-based method for the SBFD-support in a particular TDD band was agreed in RAN4#112. As follow-up, in RAN4#112bis, the supported channel bandwidth shall be no less than 40 MHz, except 20MHz for band n39 [9]. Furthermore, it is confirmed that SBFD operation shall not be applied in NR-U bands, i.e., n46, n96 and n102 [11].

For the specification implementation of operating bands support for SBFD, the agreements have been reached [11]:

<From RAN4#113> Agreement: - The following TP is adopted for clause 5.2 in TS 38.104 - Subband full duplex (SBFD) is designed to operate in NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96, n101 and n102, and NR operating bands in FR2-1 which are defined in Table 5.2-2. - FFS a note in the band table 5.2-1 to indicate the NR TDD bands not applicable to SBFD operation.

From last meeting’s agreement, it is supposed to FFS a note in the band table 5.2-1 to indicate the NR TDD bands not applicable to SBFD operation. Accordingly, the following proposal is provided:

Proposal 1: RAN4 shall adopt the following Note in band table 5.2-1 to indicate the NR TDD bands not applicable to SBFD operation:

- “Note 12: This TDD band is not applicable to SBFD operation.”

Accordingly, the following text proposal can be adopted:

Proposal 2: RAN4 shall adopt the following text proposal to section 5.2 for SBFD operation:

=================== Start of Text Proposal ===================

## 5.2	Operating bands

NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

NR operating band n1, which is defined in Table 5.2-1, can be applied for HAPS operation.

NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation.

NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n31,  n41, n65, n66, n70, n71, n72, n74, n85, n90, n106 which are defined in Table 5.2-1.

Subband full duplex (SBFD) is designed to operate in NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96, n101 and n102, and NR operating bands in FR2-1 which are defined in Table 5.2-2.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Notes |
| --- | --- | --- | --- | --- |
| n1 | 1920 – 1980 | 2110 – 2170 | FDD |  |
| n2 | 1850 – 1910 | 1930 – 1990 | FDD |  |
| n3 | 1710 – 1785 | 1805 – 1880 | FDD |  |
| n5 | 824 – 849 | 869 – 894 | FDD |  |
| n7 | 2500 – 2570 | 2620 – 2690 | FDD |  |
| n8 | 880 – 915 | 925 – 960 | FDD |  |
| n12 | 699 – 716 | 729 – 746 | FDD |  |
| n13 | 777 – 787 | 746 – 756 | FDD |  |
| n14 | 788 – 798 | 758 – 768 | FDD |  |
| n18 | 815 – 830 | 860 – 875 | FDD |  |
| n20 | 832 – 862 | 791 – 821 | FDD |  |
| n24 | 1626.5 – 1660.5 | 1525 – 1559 | FDD | Note 7 |
| n25 | 1850 – 1915 | 1930 – 1995 | FDD |  |
| n26 | 814 – 849 | 859 – 894 | FDD |  |
| n28 | 703 – 748 | 758 – 803 | FDD |  |
| n29 | N/A | 717 – 728 | SDL |  |
| n30 | 2305 – 2315 | 2350 – 2360 | FDD |  |
| n31 | 452.5 – 457.5 | 462.5 – 467.5 | FDD |  |
| n34 | 2010 – 2025 | 2010 – 2025 | TDD | Note 12 |
| n38 | 2570 – 2620 | 2570 – 2620 | TDD |  |
| n39 | 1880 – 1920 | 1880 – 1920 | TDD |  |
| n40 | 2300 – 2400 | 2300 – 2400 | TDD |  |
| n41 | 2496 – 2690 | 2496 – 2690 | TDD |  |
| n46 | 5150 – 5925 | 5150 – 5925 | TDD | Note 3, Note 12 |
| n48 | 3550 – 3700 | 3550 – 3700 | TDD |  |
| n50 | 1432 – 1517 | 1432 – 1517 | TDD |  |
| n51 | 1427 – 1432 | 1427 – 1432 | TDD | Note 12 |
| n53 | 2483.5 – 2495 | 2483.5 – 2495 | TDD | Note 12 |
| n54 | 1670 – 1675 | 1670 – 1675 | TDD | Note 12 |
| n65 | 1920 – 2010 | 2110 – 2200 | FDD |  |
| n66 | 1710 – 1780 | 2110 – 2200 | FDD |  |
| n67 | N/A | 738 – 758 | SDL |  |
| n70 | 1695 – 1710 | 1995 – 2020 | FDD |  |
| n71 | 663 – 698 | 617 – 652 | FDD |  |
| n72 | 451 – 456 | 461 – 466 | FDD |  |
| n74 | 1427 – 1470 | 1475 – 1518 | FDD |  |
| n75 | N/A | 1432 – 1517 | SDL |  |
| n76 | N/A | 1427 – 1432 | SDL |  |
| n77 | 3300 – 4200 | 3300 – 4200 | TDD | Note 11 |
| n78 | 3300 – 3800 | 3300 – 3800 | TDD |  |
| n79 | 4400 – 5000 | 4400 – 5000 | TDD |  |
| n80 | 1710 – 1785 | N/A | SUL |  |
| n81 | 880 – 915 | N/A | SUL |  |
| n82 | 832 – 862 | N/A | SUL |  |
| n83 | 703 – 748 | N/A | SUL |  |
| n84 | 1920 – 1980 | N/A | SUL |  |
| n85 | 698 – 716 | 728 – 746 | FDD |  |
| n86 | 1710 – 1780 | N/A | SUL |  |
| n89 | 824 – 849 | N/A | SUL |  |
| n90 | 2496 – 2690 | 2496 – 2690 | TDD |  |
| n91 | 832 – 862 | 1427 – 1432 | FDD | Note 2 |
| n92 | 832 – 862 | 1432 – 1517 | FDD | Note 2 |
| n93 | 880 – 915 | 1427 – 1432 | FDD | Note 2 |
| n94 | 880 – 915 | 1432 – 1517 | FDD | Note 2 |
| n95 | 2010 – 2025 | N/A | SUL | Note 1 |
| n96 | 5925 – 7125 | 5925 – 7125 | TDD | Note 3, Note 4, Note 12 |
| n97 | 2300 – 2400 | N/A | SUL | Note 5 |
| n98 | 1880 – 1920 | N/A | SUL | Note 5 |
| n99 | 1626.5 – 1660.5 | N/A | SUL | Note 6 |
| n100 | 874.4 – 880 | 919.4 – 925 | FDD | Note 10 |
| n101 | 1900 – 1910 | 1900 – 1910 | TDD | Note 10, Note 12 |
| n102 | 5925 – 6425 | 5925 – 6425 | TDD | Note 3, Note 4, Note 12 |
| n104 | 6425 – 7125 | 6425 – 7125 | TDD | Note 8 |
| n105 | 663 – 703 | 612 – 652 | FDD |  |
| n106 | 896 – 901 | 935 – 940 | FDD |  |
| n109 | 703 – 733 | 1432 – 1517 | FDD | Note 2 |
| NOTE 1: This band is applicable in China only. NOTE 2: Variable duplex operation does not enable dynamic variable duplex configuration by the network, and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band. NOTE 3: This band is restricted to operation with shared spectrum channel access as defined in TS 37.213 [20]. NOTE 4: This band is applicable only in countries/regions designating this band for shared-spectrum access use subject to country-specific conditions. NOTE 5: The requirements for this band are applicable only where no other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area. For scenarios where other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 6: UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48 [25]. NOTE 7: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order 20-51 [24]. NOTE 8: This band is applicable only in countries/regions designating this band for IMT licensed operation subject to country-specific conditions. NOTE 9: Operating band n200 is a reserved value. NOTE 10: This band is applicable to CEPT countries subject to ECC Decision (20)02 [21], for the RMR application. NOTE 11: In the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. In Canada this band is restricted to 3450 – 3650 MHz and 3650 – 3980 MHz. NOTE 12: This TDD band is not applicable to SBFD operation. |  |  |  |  |

Table 5.2-2: NR operating bands in FR2

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high (MHz) | Duplex mode |
| --- | --- | --- |
| n257 | 26500 – 29500 | TDD |
| n258 | 24250 – 27500 | TDD |
| n259 | 39500 – 43500 | TDD |
| n260 | 37000 – 40000 | TDD |
| n261 | 27500 – 28350 | TDD |
| n262 | 47200 – 48200 | TDD |
| n263 | 57000 – 71000 | TDD |

=================== End of Text Proposal ===================

#### 2.2 Specification implementation of BS CBW support for SBFD

For the specification implementation of BS channel bandwidth support, the following agreements have been reached [11]:

<From RAN4#113> Agreement: - The following note is introduced in Table 5.3.2-1 and Table 5.3.5-1 in TS 38.104: - In the bands applicable to SBFD operation, the channel bandwidth for SBFD operation shall be no less than 20MHz for band n39 and no less than 40MHz otherwise.

Because the option 2b is adopted for SBFD DL/UL subband configurations support, the UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided Table 5.3.2-2 for FR2-1. Accordingly, the minimum transmission bandwidth configuration for SBFD UL subband is 50MHz, which means the minimum BS channel bandwidth supported for SBFD shall be 100MHz, i.e, larger than 50MHz. Therefore, the following proposal can be provided straightforwardly:

Proposal 3: In FR2-1 bands, the BS channel bandwidth for SBFD operation shall be no less than 100MHz.

Accordingly, based on the previous agreement and above Proposal 3, the following text proposal can be adopted for section 5.3.2 in TS38.104:

Proposal 4: RAN4 shall adopt the following text proposal to Table 5.3.2-1 and Table 5.3.2-2 for SBFD operation:

=================== Start of Text Proposal ===================

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |
| Note: In FR1 bands applicable to SBFD operation, the BS channel bandwidth for SBFD operation shall be no less than 20MHz for band n39 and no less than 40MHz otherwise. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |
| Note: The BS channel bandwidth for SBFD operation shall be no less than 100MHz. |  |  |  |  |

=================== End of Text Proposal ===================

For the text proposal to section 5.3.5 in TS38.104, accrodingly, the following proposal is provided:

Proposal 5: Same as the Notes in previous proposal, RAN4 shall adopt the same Notes for Table 5.3.5-1 and Table 5.3.5-2 respectively. Take the table for FR2-1 as example:

=================== Start of Text Proposal ===================

Table 5.3.5-2: BS channel bandwidths and SCS per operating band in FR2-1

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n257 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n258 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n259 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n260 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n261 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n262 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| Note 1: For this bandwidth, it is not applicable for SBFD operation. |  |  |  |  |  |

=================== End of Text Proposal ===================

#### 2.3 DL/UL subband configurations support for SBFD

As requested in WID, to support SBFD operation at gNB side within a TDD carrier, the mechanism for semi-static indication of frequency location of SBFD subbands to UE shall be specified. Based on the agreement given in RAN4#113, the option 2b is adopted to specify DL/UL subband configurations support for SBFD, with the following restrictions on SBFD configuration provided [11]:

Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

And the option 2b is given in RAN4#112:

Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB.

Here we would like to further discuss the remaining issues on the restrictions on SBFD configuration.

Restriction-1: For DUD case, the UL subband should be placed in the middle of channel bandwidth

During last meeting, for DUD case, it is agreed that the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD, in which DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB. There is no remaining issue for this restriction except how to implement this agreement into the specification.

Restriction-2: Restriction on the number of UL subband sizes supported

Based on the agreement in RAN4#113, for a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. Furthermore, it is still FFS different sizes for specific UL subband for DUD and DU/UD, and FFS the restriction is also applied to FR1 MR BS.

As the supported UL subband sizes are declaration based, and we see no harm to support maximum 3 specific UL subband sizes. Since the restriction of maximum UL subband sizes comes from the limited number of RF subband filtering implementations, we think the specific UL subband sizes should be applicable for DUD and DU/UD if supported by BS. Furthermore, based on the possible digital filtering design for FR1 MR BS, the restriction shall not be applied to FR1 MR BS.

Proposal 6: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

- The specific UL subband sizes supported shall be the same for DUD and DU/UD if BS supports.

- This restriction shall be applied to FR1 WA BS only.

Restriction-3: Restriction on the number of DL subband sizes supported

Based on the agreement in RAN4#113, for a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case. It is still FFS the restriction is also applied to FR1 MR BS. Similar as previous restriction, based on the possible digital filtering design for FR1 MR BS, the restriction shall not be applied to FR1 MR BS.

Proposal 7: For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case.

- This restriction shall be applied to FR1 WA BS only.

#### 2.4 Restriction on guardband size

Since Option-2b is agreed in which it enable flexible DL subband size(s) and guardband size(s) depending on BS implementation, there is no need to impose restrictions on guardband size, including defining minimum or maximum RB limits.

Proposal 8: For guardband size, no restriction shall be introduced on maximum/minimum number of RBs.

# 3. PUSCH with resource muting for CLI handling

During RAN1#119 (Nov. 2024), the LS on impact on transmit signal quality/MPR requirement is approved [12]. Based on the information provided by RAN1, RAN1 would like to ask RAN4 to check impact on transmit signal quality/MPR.

From RAN4 perspective, one of comparable waveform is almost contiguous allocation, for which extra maximum power reduction is allowed, if NRB_gap / (NRB_alloc + NRB_gap ) ≤ 0.25 and NRB_alloc + NRB_gap is larger than 106, 51 or 24 RBs for 15 kHz, 30 kHz or 60 kHz respectively where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs. But it should also be noted that the difference exists in (1) the number of muted REs is half of the total REs; (2) the muting is RE-based granularity, rather than RB-based granularity for unallocated resources in almost contiguous allocation. By keeping the above differences in mind, considering PUSCH with resource muting is similar as “almost contiugous allocation”, the detailed MPR should be needed.

Furthermore, to evaluate the transmit signal quality/MPR impact, RAN4 should consider the following cases:

Proposal 9: The following scenarios for transmit signal quality/MPR impact evaluation can be considered:

- DFT-s-OFDM waveform:

  - FR1: MPR evaluation at least for power class 3 UE

  - FR2: discussion on the necessity of MPR evaluation

- CP-OFDM waveform:

  - FR1: MPR evaluation at least for power class 3 UE, with (1) contiguous allocation and (2) almost contiguous allocation

  - FR2: discussion on the necessity of MPR evaluation

# 4. Applicability of Existing Requirement for SBFD

Similar as the clause 4.6 in TS38.104 for applicability of requirements for different BS types, we expect the similar table should be introduced for SBFD capable BS. According to the agreement from RAN4#113, i.e., “For SBFD, BS type 1-C is precluded.”, the applicability table is drafted for BS type 1-H, 1-O and 2-O below.

Proposal 10: The applicability of RAN4 requirement for SBFD capable BS shall be introduced, with the following table used as baseline for further discussion:

Table x.x: Requirement set applicability for SBFD-capable BS

| Requirement | Requirement Set for non-SBFD symbols |  |  | Requirement Set for SBFD symbols |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BS type 1-H | BS type 1-O | BS type 2-O | BS type 1-H | BS type 1-O | BS type 2-O |
| BS output power | 6.2 |  |  | TBD |  |  |
| Output power dynamics | 6.3 |  |  | TBD |  |  |
| Transmit ON/OFF power | 6.4 |  |  | TBD |  |  |
| Transmitted signal quality | 6.5 |  |  | TBD |  |  |
| Occupied bandwidth | 6.6.2 |  |  | TBD |  |  |
| ACLR | 6.6.3 |  |  | TBD |  |  |
| Operating band unwanted emissions | 6.6.4 |  |  | TBD |  |  |
| Transmitter spurious emissions | 6.6.5 |  |  | TBD |  |  |
| Transmitter intermodulation | 6.7 | NA | NA | TBD | NA | NA |
| Reference sensitivity level | 7.2 |  |  | TBD |  |  |
| Dynamic range | 7.3 |  |  | TBD |  |  |
| In-band selectivity and blocking | 7.4 |  |  | TBD |  |  |
| Out-of-band blocking | 7.5 |  |  | TBD |  |  |
| Receiver spurious emissions | 7.6 |  |  | TBD |  |  |
| Receiver intermodulation | 7.7 |  |  | TBD |  |  |
| In-channel selectivity | 7.8 |  |  | TBD |  |  |
| Performance requirements | 8 |  |  | TBD |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 | TBD | TBD | TBD |
| OTA base station output power |  | 9.3 | 9.3 |  | TBD | TBD |
| OTA output power dynamics |  | 9.4 | 9.4 |  | TBD | TBD |
| OTA transmit ON/OFF power |  | 9.5 | 9.5 |  | TBD | TBD |
| OTA transmitted signal quality |  | 9.6 | 9.6 |  | TBD | TBD |
| OTA occupied bandwidth |  | 9.7.2 | 9.7.2 |  | TBD | TBD |
| OTA ACLR | NA | 9.7.3 | 9.7.3 | NA | TBD | TBD |
| OTA out-of-band emission |  | 9.7.4 | 9.7.4 |  | TBD | TBD |
| OTA transmitter spurious emission |  | 9.7.5 | 9.7.5 |  | TBD | TBD |
| OTA transmitter intermodulation |  | 9.8 | NA |  | TBD | NA |
| OTA sensitivity | 10.2 | 10.2 | NA | TBD | TBD | NA |
| OTA reference sensitivity level |  | 10.3 | 10.3 |  | TBD | TBD |
| OTA dynamic range |  | 10.4 | NA |  | TBD | NA |
| OTA in-band selectivity and blocking |  | 10.5 | 10.5 |  | TBD | TBD |
| OTA out-of-band blocking | NA | 10.6 | 10.6 | NA | TBD | TBD |
| OTA receiver spurious emission |  | 10.7 | 10.7 |  | TBD | TBD |
| OTA receiver intermodulation |  | 10.8 | 10.8 |  | TBD | TBD |
| OTA in-channel selectivity |  | 10.9 | 10.9 |  | TBD | TBD |
| Radiated performance requirements |  | 11 | 11 |  | TBD | TBD |

# 5. BS RF specification structure for SBFD requirements

In previous RAN4 discussion, two options, i.e., creating new sub-clauses and embedding the SBFD RF requirement in corresponding section, were listed on how to incorporate SBFD requirements in the BS RF core specification (i.e., TS 38.104). Based on the discussion outcome from RAN4#112bis, the following way forward is achieved:

<From RAN4#112bis> Way Forward: - FFS options of how to introduce BS RF new requirements for SBFD-capable BS: - Option-2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option-3: A new specification for SBFD BS RF requirements

To proceed the work, the following agreement is achieved:

Agreement: - Draft CR’s endorsed against 38.104 does not imply that a new specification could not be agreed.

By considering the readability of the specification for SBFD-capable BS, we think the way of creating new and standalone sub-clauses (e.g., with a suffix) in TS 38.104 can be followed for SBFD-specific existing or new gNB RF requirements. As mentioned in previous RAN4 discussion by some companies, one comparable precedent is using suffix-B for Mobile IAB-node in the core and conformance specification for IAB nodes (TS 38.174, TS 38.176-1 and TS 38.176-2).

Observation 1: Setting new sub-clauses with a suffix for a new feature is not only a common practice used in UE RF requirement, but also in the specification for network nodes like IAB.

Furthermore, in the updated WID [1], the impacted existing TS (including TS 38.104 and TS 38.141-1/2 for BS requirements) are listed while there is no new specification listed for BS RF requirement dedicated for SBFD operation or SBFD-capable BS.

Observation 2: In WID of Rel-19 duplex evolution, there is no new specification expected, and TS 38.104 is listed as impacted existing TS by SBFD feature introduced.

Another drawback for setting up the new specification shall be noted: it could bring confusion to the reader the scope of this new specification, whether it is the specification for new SBFD-capable gNB or the specification for SBFD operation within SBFD symbols. If the former one is followed, we expect the specification should copy/paste lots of existing BS RF requirements which bring lots of specification implementation issues to sync-up future text proposals on the similar requirements in two specifications. If the latter is followed, we also expect lots of referring to TS 38.104 are required, which greatly impact the specification readability.

Considering all the above reasons, RAN4 shall adopt the way of creating new and standalone sub-clauses in TS 38.104 by setting up new sub-clauses with suffix-B, for both new and modified existing requirements.

Proposal 11: For introducing BS RF requirements for SBFD-capable BS, RAN4 shall adopt the way of creating new and standalone sub-clauses in TS 38.104 by setting up new sub-clauses with suffix-B, i.e.,

- For the SBFD BS RF requirements specified by modifying existing requirements:

  - Set up new sub-clause with suffix-B

- For the new BF RF requirements dedicated to SBFD BS

  - Set up new sub-clause with suffix-B

# 6. Conclusion

In this contribution, we would like to provide our analysis and viewpoints on the general aspects including RAN4 aspects for SBFD system parameters, accordingly with the following observations and proposals obtained:

RAN4 aspects for SBFD system parameters

Proposal 1: RAN4 shall adopt the following Note in band table 5.2-1 to indicate the NR TDD bands not applicable to SBFD operation:

- “Note 12: This TDD band is not applicable to SBFD operation.”

Proposal 2: RAN4 shall adopt the following text proposal to section 5.2 for SBFD operation:

=================== Start of Text Proposal ===================

## 5.2	Operating bands

NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

NR operating band n1, which is defined in Table 5.2-1, can be applied for HAPS operation.

NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation.

NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n31,  n41, n65, n66, n70, n71, n72, n74, n85, n90, n106 which are defined in Table 5.2-1.

Subband full duplex (SBFD) is designed to operate in NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96, n101 and n102, and NR operating bands in FR2-1 which are defined in Table 5.2-2.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Notes |
| --- | --- | --- | --- | --- |
| n1 | 1920 – 1980 | 2110 – 2170 | FDD |  |
| n2 | 1850 – 1910 | 1930 – 1990 | FDD |  |
| n3 | 1710 – 1785 | 1805 – 1880 | FDD |  |
| n5 | 824 – 849 | 869 – 894 | FDD |  |
| n7 | 2500 – 2570 | 2620 – 2690 | FDD |  |
| n8 | 880 – 915 | 925 – 960 | FDD |  |
| n12 | 699 – 716 | 729 – 746 | FDD |  |
| n13 | 777 – 787 | 746 – 756 | FDD |  |
| n14 | 788 – 798 | 758 – 768 | FDD |  |
| n18 | 815 – 830 | 860 – 875 | FDD |  |
| n20 | 832 – 862 | 791 – 821 | FDD |  |
| n24 | 1626.5 – 1660.5 | 1525 – 1559 | FDD | Note 7 |
| n25 | 1850 – 1915 | 1930 – 1995 | FDD |  |
| n26 | 814 – 849 | 859 – 894 | FDD |  |
| n28 | 703 – 748 | 758 – 803 | FDD |  |
| n29 | N/A | 717 – 728 | SDL |  |
| n30 | 2305 – 2315 | 2350 – 2360 | FDD |  |
| n31 | 452.5 – 457.5 | 462.5 – 467.5 | FDD |  |
| n34 | 2010 – 2025 | 2010 – 2025 | TDD | Note 12 |
| n38 | 2570 – 2620 | 2570 – 2620 | TDD |  |
| n39 | 1880 – 1920 | 1880 – 1920 | TDD |  |
| n40 | 2300 – 2400 | 2300 – 2400 | TDD |  |
| n41 | 2496 – 2690 | 2496 – 2690 | TDD |  |
| n46 | 5150 – 5925 | 5150 – 5925 | TDD | Note 3, Note 12 |
| n48 | 3550 – 3700 | 3550 – 3700 | TDD |  |
| n50 | 1432 – 1517 | 1432 – 1517 | TDD |  |
| n51 | 1427 – 1432 | 1427 – 1432 | TDD | Note 12 |
| n53 | 2483.5 – 2495 | 2483.5 – 2495 | TDD | Note 12 |
| n54 | 1670 – 1675 | 1670 – 1675 | TDD | Note 12 |
| n65 | 1920 – 2010 | 2110 – 2200 | FDD |  |
| n66 | 1710 – 1780 | 2110 – 2200 | FDD |  |
| n67 | N/A | 738 – 758 | SDL |  |
| n70 | 1695 – 1710 | 1995 – 2020 | FDD |  |
| n71 | 663 – 698 | 617 – 652 | FDD |  |
| n72 | 451 – 456 | 461 – 466 | FDD |  |
| n74 | 1427 – 1470 | 1475 – 1518 | FDD |  |
| n75 | N/A | 1432 – 1517 | SDL |  |
| n76 | N/A | 1427 – 1432 | SDL |  |
| n77 | 3300 – 4200 | 3300 – 4200 | TDD | Note 11 |
| n78 | 3300 – 3800 | 3300 – 3800 | TDD |  |
| n79 | 4400 – 5000 | 4400 – 5000 | TDD |  |
| n80 | 1710 – 1785 | N/A | SUL |  |
| n81 | 880 – 915 | N/A | SUL |  |
| n82 | 832 – 862 | N/A | SUL |  |
| n83 | 703 – 748 | N/A | SUL |  |
| n84 | 1920 – 1980 | N/A | SUL |  |
| n85 | 698 – 716 | 728 – 746 | FDD |  |
| n86 | 1710 – 1780 | N/A | SUL |  |
| n89 | 824 – 849 | N/A | SUL |  |
| n90 | 2496 – 2690 | 2496 – 2690 | TDD |  |
| n91 | 832 – 862 | 1427 – 1432 | FDD | Note 2 |
| n92 | 832 – 862 | 1432 – 1517 | FDD | Note 2 |
| n93 | 880 – 915 | 1427 – 1432 | FDD | Note 2 |
| n94 | 880 – 915 | 1432 – 1517 | FDD | Note 2 |
| n95 | 2010 – 2025 | N/A | SUL | Note 1 |
| n96 | 5925 – 7125 | 5925 – 7125 | TDD | Note 3, Note 4, Note 12 |
| n97 | 2300 – 2400 | N/A | SUL | Note 5 |
| n98 | 1880 – 1920 | N/A | SUL | Note 5 |
| n99 | 1626.5 – 1660.5 | N/A | SUL | Note 6 |
| n100 | 874.4 – 880 | 919.4 – 925 | FDD | Note 10 |
| n101 | 1900 – 1910 | 1900 – 1910 | TDD | Note 10, Note 12 |
| n102 | 5925 – 6425 | 5925 – 6425 | TDD | Note 3, Note 4, Note 12 |
| n104 | 6425 – 7125 | 6425 – 7125 | TDD | Note 8 |
| n105 | 663 – 703 | 612 – 652 | FDD |  |
| n106 | 896 – 901 | 935 – 940 | FDD |  |
| n109 | 703 – 733 | 1432 – 1517 | FDD | Note 2 |
| NOTE 1: This band is applicable in China only. NOTE 2: Variable duplex operation does not enable dynamic variable duplex configuration by the network, and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band. NOTE 3: This band is restricted to operation with shared spectrum channel access as defined in TS 37.213 [20]. NOTE 4: This band is applicable only in countries/regions designating this band for shared-spectrum access use subject to country-specific conditions. NOTE 5: The requirements for this band are applicable only where no other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area. For scenarios where other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 6: UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48 [25]. NOTE 7: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order 20-51 [24]. NOTE 8: This band is applicable only in countries/regions designating this band for IMT licensed operation subject to country-specific conditions. NOTE 9: Operating band n200 is a reserved value. NOTE 10: This band is applicable to CEPT countries subject to ECC Decision (20)02 [21], for the RMR application. NOTE 11: In the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. In Canada this band is restricted to 3450 – 3650 MHz and 3650 – 3980 MHz. NOTE 12: This TDD band is not applicable to SBFD operation. |  |  |  |  |

Table 5.2-2: NR operating bands in FR2

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high (MHz) | Duplex mode |
| --- | --- | --- |
| n257 | 26500 – 29500 | TDD |
| n258 | 24250 – 27500 | TDD |
| n259 | 39500 – 43500 | TDD |
| n260 | 37000 – 40000 | TDD |
| n261 | 27500 – 28350 | TDD |
| n262 | 47200 – 48200 | TDD |
| n263 | 57000 – 71000 | TDD |

=================== End of Text Proposal ===================

Proposal 3: In FR2-1 bands, the BS channel bandwidth for SBFD operation shall be no less than 100MHz.

Proposal 4: RAN4 shall adopt the following text proposal to Table 5.3.2-1 and Table 5.3.2-2 for SBFD operation:

=================== Start of Text Proposal ===================

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |
| Note: In FR1 bands applicable to SBFD operation, the BS channel bandwidth for SBFD operation shall be no less than 20MHz for band n39 and no less than 40MHz otherwise. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |
| Note: The BS channel bandwidth for SBFD operation shall be no less than 100MHz. |  |  |  |  |

=================== End of Text Proposal ===================

Proposal 5: Same as the Notes in previous proposal, RAN4 shall adopt the same Notes for Table 5.3.5-1 and Table 5.3.5-2 respectively. Take the table for FR2-1 as example:

=================== Start of Text Proposal ===================

Table 5.3.5-2: BS channel bandwidths and SCS per operating band in FR2-1

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n257 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n258 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n259 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n260 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n261 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n262 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| Note 1: For this bandwidth, it is not applicable for SBFD operation. |  |  |  |  |  |

=================== End of Text Proposal ===================

Proposal 6: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

- The specific UL subband sizes supported shall be the same for DUD and DU/UD if BS supports.

- This restriction shall be applied to FR1 WA BS only.

Proposal 7: For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case.

- This restriction shall be applied to FR1 WA BS only.

Proposal 8: For guardband size, no restriction shall be introduced on maximum/minimum number of RBs.

PUSCH with resource muting for CLI handling

Proposal 9: The following scenarios for transmit signal quality/MPR impact evaluation can be considered:

- DFT-s-OFDM waveform:

  - FR1: MPR evaluation at least for power class 3 UE

  - FR2: discussion on the necessity of MPR evaluation

- CP-OFDM waveform:

  - FR1: MPR evaluation at least for power class 3 UE, with (1) contiguous allocation and (2) almost contiguous allocation

  - FR2: discussion on the necessity of MPR evaluation

Applicability of Existing Requirement for SBFD

Proposal 10: The applicability of RAN4 requirement for SBFD capable BS shall be introduced, with the following table used as baseline for further discussion:

Table x.x: Requirement set applicability for SBFD-capable BS

| Requirement | Requirement Set for non-SBFD symbols |  |  | Requirement Set for SBFD symbols |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BS type 1-H | BS type 1-O | BS type 2-O | BS type 1-H | BS type 1-O | BS type 2-O |
| BS output power | 6.2 |  |  | TBD |  |  |
| Output power dynamics | 6.3 |  |  | TBD |  |  |
| Transmit ON/OFF power | 6.4 |  |  | TBD |  |  |
| Transmitted signal quality | 6.5 |  |  | TBD |  |  |
| Occupied bandwidth | 6.6.2 |  |  | TBD |  |  |
| ACLR | 6.6.3 |  |  | TBD |  |  |
| Operating band unwanted emissions | 6.6.4 |  |  | TBD |  |  |
| Transmitter spurious emissions | 6.6.5 |  |  | TBD |  |  |
| Transmitter intermodulation | 6.7 | NA | NA | TBD | NA | NA |
| Reference sensitivity level | 7.2 |  |  | TBD |  |  |
| Dynamic range | 7.3 |  |  | TBD |  |  |
| In-band selectivity and blocking | 7.4 |  |  | TBD |  |  |
| Out-of-band blocking | 7.5 |  |  | TBD |  |  |
| Receiver spurious emissions | 7.6 |  |  | TBD |  |  |
| Receiver intermodulation | 7.7 |  |  | TBD |  |  |
| In-channel selectivity | 7.8 |  |  | TBD |  |  |
| Performance requirements | 8 |  |  | TBD |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 | TBD | TBD | TBD |
| OTA base station output power |  | 9.3 | 9.3 |  | TBD | TBD |
| OTA output power dynamics |  | 9.4 | 9.4 |  | TBD | TBD |
| OTA transmit ON/OFF power |  | 9.5 | 9.5 |  | TBD | TBD |
| OTA transmitted signal quality |  | 9.6 | 9.6 |  | TBD | TBD |
| OTA occupied bandwidth |  | 9.7.2 | 9.7.2 |  | TBD | TBD |
| OTA ACLR | NA | 9.7.3 | 9.7.3 | NA | TBD | TBD |
| OTA out-of-band emission |  | 9.7.4 | 9.7.4 |  | TBD | TBD |
| OTA transmitter spurious emission |  | 9.7.5 | 9.7.5 |  | TBD | TBD |
| OTA transmitter intermodulation |  | 9.8 | NA |  | TBD | NA |
| OTA sensitivity | 10.2 | 10.2 | NA | TBD | TBD | NA |
| OTA reference sensitivity level |  | 10.3 | 10.3 |  | TBD | TBD |
| OTA dynamic range |  | 10.4 | NA |  | TBD | NA |
| OTA in-band selectivity and blocking |  | 10.5 | 10.5 |  | TBD | TBD |
| OTA out-of-band blocking | NA | 10.6 | 10.6 | NA | TBD | TBD |
| OTA receiver spurious emission |  | 10.7 | 10.7 |  | TBD | TBD |
| OTA receiver intermodulation |  | 10.8 | 10.8 |  | TBD | TBD |
| OTA in-channel selectivity |  | 10.9 | 10.9 |  | TBD | TBD |
| Radiated performance requirements |  | 11 | 11 |  | TBD | TBD |

BS RF specification structure for SBFD requirements

Observation 1: Setting new sub-clauses with a suffix for a new feature is not only a common practice used in UE RF requirement, but also in the specification for network nodes like IAB.

Observation 2: In WID of Rel-19 duplex evolution, there is no new specification expected, and TS 38.104 is listed as impacted existing TS by SBFD feature introduced.

Proposal 11: For introducing BS RF requirements for SBFD-capable BS, RAN4 shall adopt the way of creating new and standalone sub-clauses in TS 38.104 by setting up new sub-clauses with suffix-B, i.e.,

- For the SBFD BS RF requirements specified by modifying existing requirements:

  - Set up new sub-clause with suffix-B

- For the new BF RF requirements dedicated to SBFD BS

  - Set up new sub-clause with suffix-B

# 7. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[4] R4-2409763, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#111.

[5] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[6] R4-2411637, “Discussion on SBFD general aspects”, Samsung, RAN4#112.

[7] R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112.

[8] R4-2416497, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#112bis.

[9] R4-2416584, “Way forward for [112bis][306] NR_duplex_evo_General”, Samsung, RAN4#112bis.

[10] R4-2418097, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#113.

[11] R4-2419907, “Way Forward for [113][306] NR_duplex_evo_General”, Samsung, RAN4#113.

[12] R4-2500006 (R1-2410889), “LS on impact on transmit signal quality/MPR requirement”, Huawei, RAN1#119.