3GPP TSG-RAN WG4 Meeting #114									R4-2501974

Athen, GR, 17th – 21nd February, 2025

Agenda item:	7.24.3

Source:	CAICT

Title:	Discussion on OTA test method for A-IoT device

Document for:	Approval

1	Introduction

In RAN#106 meeting, a new WI Solutions for Ambient IoT (Internet of Things) in NR was approved [1]. A key objective of this WI is to investigate and develop OTA test methodologies for A-IoT devices. In this contribution, we present our general perspectives on this topic.

2	General views on OTA test method for A-IoT

Unlike conventional device types, A-IoT devices lack conducted interfaces, necessitating full OTA-based testing for all RF (RRM and Demod, if any) test cases, similar to mmwave device testing methodologies. Given the complexity and compatibility requirements of test systems, it is imperative to maximize reuse of existing test infrastructure. This alignment with the WID's stated principle to use test methods specified in TR 38.870 as starting point.

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

In alignment with the framework of TR 38.870, the study of OTA test methodologies for A-IoT devices shall address the following key aspects:

- Test Configurations and use scenarios

- Performance Metrics

- Positioning Guidelines

- Chamber Method

- Testing Time Reduction Method (if needed)

- Measurement Uncertainty Analysis

- Test Configurations and Use Scenarios

Unlike smartphones or wearable devices where OTA testing typically employs body phantoms to simulate real-world usage scenarios, A-IoT devices1 with Deployment scenario 1 are deployed in indoor environments and interactions with human bodies are minimal. To simplify test complexity, Free Space testing configurations are proposed as the foundational condition for A-IoT device OTA test.

Proposal 1: Adopt Free Space testing configuration for A-IoT device OTA testing.

- Performance Metrics

The definition of performance metrics for A-IoT devices should align with RF (RRM/demodulation, if any) requirements and leverage existing OTA test methodologies. Currently, our key considerations include:

- Power-related metrics: Focus on backscattering power characterization (e.g., EIRP-like for directional patterns or TRP-like for omnidirectional coverage).

- Rx metric: Reference sensitivity

Proposal 2: Clarify the applicability of EIRP-like vs. TRP-like metrics in A-IoT OTA testing through further analysis of deployment scenarios, test efficiency, and other related aspects.

- Positioning guidelines

If FS testing condition is adopted for A-IoT devices, positioning guidelines can directly reuse existing definitions from TR38.870 as a baseline. This approach maximizes the reuse of established OTA test method and simplifies implementation.

Proposal 3: Reuse TR38.870 positioning guidelines as the baseline for A-IoT FS testing.

- Test Methodology

The chamber setup and test methodology for A-IoT devices should balance baseline reuse and A-IoT-specific adaptations. Key considerations include:

Baseline Reuse:

- Quiet Zone/Calibration/Minimum Range Length: Adopt requirements defined in TR 38.870.

System Setup Adaptations:

For ultra-low power A-IoT devices (e.g., ~1 µW peak power), amplifiers in the test system UL/DL maybe necessary to ensure measurable signal-to-noise ratio. In addition, the test system may need to support device charging functionality due to limited energy storage, address the unique energy supply mechanism of A-IoT Device.

Test Procedure:

Study the impact of charging time and energy release patterns on test steps (e.g., pre-charging duration, synchronization between energy availability and measurement windows).

Proposal 4: Reuse Quiet Zone/Calibration/Minimum Range Length in TR 38.870 as the baseline, while further investigating system setup and test procedure adaptions for energy charging/discharging management.

- Testing Time Reduction Method (if needed) and MU Analysis

- FFS based on the outcome of above topics.

3	Conclusion

This contribution makes the following observations and proposals.

Proposal 1: Adopt Free Space testing configuration for A-IoT device OTA testing.

Proposal 2: Clarify the applicability of EIRP-like vs. TRP-like metrics in A-IoT OTA testing through further analysis of deployment scenarios, test efficiency, and other related aspects.

Proposal 3: Reuse TR38.870 positioning guidelines as the baseline for A-IoT FS testing.

Proposal 4: Reuse Quiet Zone/Calibration/Minimum Range Length in TR 38.870 as the baseline, while further investigating system setup and test procedure adaptions for energy charging/discharging management.

References

- RP-243312, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR