3GPP TSG-RAN WG4 Meeting #114	R4-2501978

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	Discussion on EMR Core / Performance requirements for rel-18 NR_Mob_Enh2

Agenda item:	5.23.1

Document for:	Discussion

Introduction

In the RAN4#112 meeting, there was a discussion on whether

Discussion

Performance part test case

In our view the Rel-16 test case does not sufficiently cover the impact of validityDuration, because validityDuration applies even if T331 is still running. In Rel-16, the UE is expected to report measurement results if T331 is running. If the duration of T331 is longer than validityDuration, the UE would be allowed to report results from the time before validityDuration, which is not correct UE behavior according to Rel-18. Therefore, we would propose to consider adding a test case for EMR-based measurements, where the UE reports valid measurements from the duration of validityDuration, similar to the test cases that have been defined for cell reselection measurement based early reporting.

Currently rel-19 is adding support for EMR with Scell activation. Therefore, there needs to be a rel-18 test case that addresses the idle-mode EMR when the validityDuration-r18 is configured. Otherwise the rel-18 feature remains incomplete.

Add Test case for Idle mode fast CA/DC eEMR measurement for FR1 with valid reporting

Core-part – About ambiguity between RAN4 and RAN2

In the RAN4#112 meeting, companies (R4-2412384) brought up an issue of mismatch between RAN4 and RAN2 requirements.

RAN2 Specifies currently:

TS 38.331 clause 5.7.10.3 1> if the reselectionMeasurementReq is included in the UEInformationRequest: 2> if validatedMeasurementsReq is included in the UEInformationRequest and measReselectionValidityDuration is included in VarMeasReselectionConfig; 3> if measReselectionCarrierListNR is present in VarMeasReselectionConfig: 4> set the measResultReselectionNR in the UEInformationResponse message the valid NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig for each reported measurement; 3> else: 4> set the measResultReselectionNR in the UEInformationResponse message to any valid NR measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig; 2> else: 3> if measReselectionCarrierListNR is present in VarMeasReselectionConfig: 4> set the measResultReselectionNR in the UEInformationResponse message the NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig; 3> else: 4> set the measResultReselectionNR in the UEInformationResponse message to any NR measurement results, if available;

When considering the  above, RAN4 requirements are very clear, if the network wants UE to perform and report measurements, the network will include these either in the RRC or SIB configuration via corresponding measurement list.

TS 38.133 clause 4.7.1 UE is not required to report for fast CA/DC setup from any measurements during IDLE mode for carriers which are not configured by higher layers.

We believe that the ambiguity in RAN2 specification comes from the agreement in RAN2#125bis (R2-2404006), which states

Agreement in RAN2#125bis (R2-2404006) For reselection measurements: Confirm that the freq is optionally configured by the network, applicable to UE availability indication and UE reporting (doesn’t impact the actual measurements). If not configured, the UE reports what is has.

As UE is not required to report for fast CA/DC setup from any measurements during IDLE mode for carriers which are not configured by higher layers.

No need to update RAN4 specification due to ambiguity in RAN2 specification. Discuss if we can close this issue.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Add Test case for Idle mode fast CA/DC eEMR measurement for FR1 with valid reporting

Proposal 2: No need to update RAN4 specification due to ambiguity in RAN2 specification.