3GPP TSG-RAN WG4 Meeting #114	R4-2502008

# Athens, Greece, 17 – 21 February 2025

Agenda item:		7.10.2.3.1

Source: 		Qualcomm Inc.

Title: 		IoT NTN high power UE Tx requirements

Document for:		Approval

## 1	Introduction

The work item for enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved [1]. One of the objectives is high power UE operation for NTN. In this contribution UE RF requirements and feasibility of NTN HPUE are discussed.

## 2	Discussion

For UE RF architecture, NR TN HPUE conclusions need further consideration. For NR, PC2 is allowed to use dual Tx (tx diversity) implementation, but PC1.5 is specified to always apply dual Tx. For NB-NTN dual Tx operation may not be necessary for high output powers, though it still is one possible option for implementation. For NTN UL-MIMO has not been considered, as UL-MIMO typically needs good SNR to give benefits. For NB-IoT and cat M1 UL-MIMO has never been specified.

Also the signal characteristics are very different between NR-NTN and NB-NTN, as e.g. NB-IoT transmission bandwidth is only up to 180 kHz. This gives opportunities to take advantage of high power PAs similar to GSM PAs, which can be able to drive high power signal from single Tx.

Handheld satellite phones which have very high output power exist. For example, phones for Iridium system [6]. From the technical specifications it an be seen that this handheld device can provice 4 hours of talk time, while the EU type examination certification shows 37.7 dBm output power.

Figure 1: Snippets of materials available in [1]

The produce regulatory and safety card which is linked to the same web page confirms that this is conducted power provided to the antenna connector. The device weight is not different from a smartphone with a large display.

Observation 1: Handheld battery powered satellite phones with much higher conducted output power than PC1 are available today.

It should be also noted that PC1.5 is not available for IoT NTN. Therefore it is even more important that some higher power option than PC2 is specified. It is not necessary to fully align with TN PC1 requirements but further accommodations can be made to help the feasibility. Possible approaches can be e.g.

- Extending lower power tolerance for PC1 handheld devices

- Considering MPR0 waveform only for narrow allocation in the middle of the band as NB-IoT single tone has lower PAR than multi-tone transmission

- Considering lower nominal output power for IoT NTN PC1

Observation 2: Final requirements can be defined in a manner which facilitate easier feasibility.

Higher than PC2 power level would be extremely useful for IoT NTN, as it improves the link budget. This can make a huge difference in whether connection is possible only in perfect open-sky conditions, or whether connection is also possible e.g. in forest where trees cause additional path loss.

Observation 3: Link budget improvements beyond PC2 are desirable.

Finally, when it comes to component availability and performance, it is perfectly expected that not all components are available and supporting PC1 operation today – there has been no market for them as even standard does not allow PC1 operation in handheld UEs. Therefore, solving the chicken-egg problem can start by enabling the higher power in standards, after which component development is more likely to accelerate.

Observation 4: Component performance and availability is likely to improve after standards enable using said higher power. Therefore, it is perfectly expected that all components are not commercially available today.

Proposal 1: PC1, including single Tx PC1, is considered feasible for NB-IoT handheld devices.

Proposal 2: RF requirements are defined for PC1 handheld devices.

Currently, NB-IoT does not support power class 2. Overall in LTE PC2 requirements have been defined only for channel bandwidths of 5 MHz and above, for example E-UTRA ACLR requirements for power class 2 have not been defined for 1.4 and 3 MHz channel bandwidths, and therefore there is no clear baseline ACLR value for category M1 UE either.

Based on the co-existence evaluations, PC3 ACLR would be sufficient to fulfil co-existence needs. Based on our evaluations the difference in absolute emission level is approximately the same 1 dB as the difference between the usual PC3 ACLR of 30 dBc and PC2 ACLR of 31 dBc. In some cases this difference is enough to justify an increase in MPR and/or A-MPR. Given that the link budget is already tight for NTN, additional power reduction is not good. Therefore, it is proposed to specify 31 dBc ACLR for cat M1 UEs.

Proposal 3: Specify PC2 ACLR of 31 dB for cat M1 UEs.

In cases where PC2 cat M1 is supported by the UE, it is likely that the same PA is used for also other more wideband waveforms (NR or LTE). As the hardware is similar, it would be logical to assume the same PA calibration for cat M1 as used for NR and LTE to be used in A-MPR evaluations.

Proposal 4: For Cat M1, in simulations calibrate PA to just meet ACLR with 31 dBc ACLR and 1 dB MPR with 100 RB DFT-s-OFDM waveform.

When PC3 requirements for NB-IoT has been discussed, there was no common alignment on how PA is calibrated, what impairment levels are used and what are the assumptions for the baseband waveform filtering before it is put through the PA. This has resulted in large divergence in results, as observed e.g. for A-MPR proposals for B254, where A-MPR is not yet concluded. Therefore, it is necessary to align the assumptions for NB-IoT evaluations.

Observation 5: RAN4 needs to agree evaluation assumptions for NB-IoT

Based on initial evaluations, same MPR can be used for PC3 and PC2 and impairment levels can be assumed to be aligned with minimum requirements of 25 dBc, and CIM3 at 60 dBc. However, for PC1, using 25 dBc IQ-image, there is a risk of needing high MPR/A-MPR to meet general requirements like NB-IoT mask. Therefore, for PC1 we propose to assume in simulation [30] dBc IQ-image and LO-leakage instead of 25 dBc. Design striving towards PC1 power level could be assumed to be slightly better than lower power classes.

Proposal 5: For NB-IoT PC2, as a starting point consider re-use of PC3 MPR.

Proposal 6: For NB-IoT PC2, for A-MPR evaluations assume 25 dBc IQ-image and LO-leakage and 60 dBc CIM3.

Proposal 7: For NB-IoT PC1, for MPR and A-MPR evaluations assume [30]dBc IQ-image and LO-leakage and 60 dBc CIM3.

These proposals still leave open how to address the baseband filtering. This is not crucial for deciding MPR, but is needed for A-MPR as there are some additional emission requirements immediately outside the channel bandwidth e.g. in band 254.

Proposal 8: Further discuss assumptions for baseband waveform filtering to be used in A-MPR evaluations.

Finally there is no formal conclusion on which ACLR requirement is set for NB-IoT. In previous meeting it was agreed that GSM ACLR will not be specified and that ACLR will not be relaxed compared to PC3. It would be useful to have continuity between PC3 and higher power classes in a sense that the principle behind the ACLR requirement is not fully changed. Therefore, it is suggested that UTRA ACLR is still specified to set a limit for wideband noise. Based on co-existence evaluations, the ACLR does not need to be more stringent than PC3 ACLR, and therefore it is proposed to take 37 dBc UTRA ACLR as starting point for the NB-IoT PC2 and PC1 ACLR.

Proposal 9: As a starting point, re-use 37 dB UTRA ACLR for NB-IoT PC2 and PC1, both for handheld and non-handheld UEs

## 3	Conclusion

In this NTN HPUE operation was discussed. Following observations and proposals were made:

Observation 1: Handheld battery powered satellite phones with much higher conducted output power than PC1 are available today.

Observation 2: Final requirements can be defined in a manner which facilitate easier feasibility.

Observation 3: Link budget improvements beyond PC2 are desirable.

Observation 4: Component performance and availability is likely to improve after standards enable using said higher power. Therefore, it is perfectly expected that all components are not commercially available today.

Proposal 1: PC1, including single Tx PC1, is considered feasible for NB-IoT handheld devices.

Proposal 2: RF requirements are defined for PC1 handheld devices.

Proposal 3: Specify PC2 ACLR of 31 dB for cat M1 UEs.

Proposal 4: For Cat M1, in simulations calibrate PA to just meet ACLR with 31 dBc ACLR and 1 dB MPR with 100 RB DFT-s-OFDM waveform.

Observation 5: RAN4 needs to agree evaluation assumptions for NB-IoT

Proposal 5: For NB-IoT PC2, as a starting point consider re-use of PC3 MPR.

Proposal 6: For NB-IoT PC2, for A-MPR evaluations assume 25 dBc IQ-image and LO-leakage and 60 dBc CIM3.

Proposal 7: For NB-IoT PC1, for MPR and A-MPR evaluations assume [30]dBc IQ-image and LO-leakage and 60 dBc CIM3.

Proposal 8: Further discuss assumptions for baseband waveform filtering to be used in A-MPR evaluations.

Proposal 9: As a starting point, re-use 37 dB UTRA ACLR for NB-IoT PC2 and PC1, both for handheld and non-handheld UEs.

## 4	References

[1]  Iridium 9555, https://www.iridium.com/products/iridium-9555/