3GPP TSG-RAN WG4 Meeting #114                                            R4-2502027

# Athens, Greece, 17th – 21st of February 2025

Title: 	Discussion on SAN RF requirements for NTN NB-IoT TDD

Agenda Item:	7.31.3 – SAN RF requirements

Release:	Rel-19

Source: 	THALES, Iridium Satellite LLC

Document for:	Discussion

# For information

As described in the approved WID RP-242415 for New WID on introduction of IoT-NTN TDD mode (Iridium, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024), the study and work objectives assume the following:

- LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763)

- Target the 1616-1626.5 MHz MSS allocated band

- Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT)

- Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band.

This work item includes the following objectives:

- Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4]

  - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2]

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

- Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4].

  - Specify band numbering

  - Specify SAN and UE RF characteristics

  - Specify DL and UL channelization.

  - Specify channel bandwidth as 200 kHz

  - Note1: No NTN-NTN coexistence study needed.

  - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

Current Work Plan from R4-2419663:

| RAN#106 (Dec. 2024) | RAN1#120 | 0.5 | - Based on the study outcome, discuss potential solutions - Identify potential specification impacts - Define the TDD pattern parameters |
| --- | --- | --- | --- |
|  | RAN2#129 | 0.5 | - Start discussion on TDD support for NB IoT NTN - Identify potential impacts on higher layers - Start the definition and configuration of signalling of the periodic pattern, and associated UE procedures. |
|  | RAN4#114 | 0.5+0.25 | - Decide on RF, discuss on RRM requirements that can be re-used from previously defined S/L band. Start preparing Draft CRs for the requirements that can be reused - Conclude on requirements stemming from introduction of TDD mode for NB-IoT NTN and potential impact of the periodic pattern: offset-subframes, DL-UL gap - Start discussion on demodulation performance and specific configuration related to the new TDD pattern |

Decisions from previous RAN4#113 IoT_NTN_TDD WF:

### Issue 0: Parameters of the baseline TDD pattern for analysis on the impact of requirements

Agreement:

- RAN4 to consider the following values for analysing the impact of requirements, as necessary:

  - N = 9, D = {8, 20, 30}, U = {8, 20, 30}

  - N = 8, D = 20, U = 20

- Among the above sets of values, the set with N = 9 and D = 8 is considered the baseline TDD pattern for analysis, as it represents the most challenging case in terms of requirement analysis.

Note 1: The above values, based on RAN1’s discussion in RAN1#118bis meeting, are recommended examples for assessing the impact of requirements on RF, RRM, and Demodulation aspects. The exact values are subject to RAN1’s decision.

Note 2: In terms of SNR, the above analysis focuses on the requirements for normal coverage.

Decisions from RAN plenary, December 2024, Madrid (WID update):

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with at least N=9 as baseline. No blind detection is assumed at the UE side. The value of N is fixed per band. Other parameters of the configuration of the periodic pattern (e.g. downlink to uplink guard period for TDD operation and the number of consecutive/contiguous DL and UL subframes) can be defined per cell.

- For this target band N=9 and minimum of D=8 and U=8 to be supported.

The RAN4#114 meeting agenda is:

7.31	Introduction of IoT-NTN TDD mode	[IoT_NTN_TDD]

7.31.1	General aspects	[IoT_NTN_TDD]

7.31.2	Band and system parameters (DL and UL channelization, channel bandwidth)	[IoT_NTN_TDD-Core]

7.31.3	SAN RF requirements	[IoT_NTN_TDD-Core]

7.31.4	UE RF requirements	[IoT_NTN_TDD-Core]

7.31.5	RRM core requirements	[IoT_NTN_TDD-Core]

7.31.6	Moderator summary and conclusions	[IoT_NTN_TDD-Core]

# General aspects for NTN NB-IoT TDD

Observation 1: First of all, RAN4 is kindly requested to take into account the WID update:

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with at least N=9 as baseline. No blind detection is assumed at the UE side. The value of N is fixed per band. Other parameters of the configuration of the periodic pattern (e.g. downlink to uplink guard period for TDD operation and the number of consecutive/contiguous DL and UL subframes) can be defined per cell.

- For this target band N=9 and minimum of D=8 and U=8 to be supported.

# SAN Requirements for IoT_NTN_TDD

First, please recall the proposals submitted in 7.31.2	- Band and system parameters (DL and UL channelization, channel bandwidth) [IoT_NTN_TDD-Core] at RAN4#114:

Observation 2:

Table 5.4A.2-1: E-UTRA channel numbers E-UTRA Operating Band; ΔFRaster (kHz); Downlink; Uplink. FDL_low (MHz); NOffs-DL; Range of NDL (First – <Step size> – Last); FUL_low (MHz); NOffs-UL; Range of NUL (First – <Step size> – Last). 256; 100; 2170; 229076; 229076 -<1>- 229375; 1980; 261844; 261844 -<1>- 262143. 255; 100; 1525; 228736; 228736 -<1>- 229075; 1626.5; 261504; 261504 -<1>- 261843. 254; 100; 2483.5; 228571; 228571 -<1>- 228735; 1610; 261339; 261339 -<1>- 261503. 253; 100; 1518; 228501; 228501 -<1>- 228570; 1668; 261269; 261269 -<1>- 261338. 249; 100; 1616; 196607; 196607-<1>-196711; 1616; 196607; 196607-<1>-196711. NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. NOTE 2: Band 249 is defined only for NB-IoT operation..

Observation 3: With respect to the definition of IoT-NTN TDD L-band, add the following information in 5.7.3 for the carrier frequency and EARFCN for DL and UL respectively:

“The carrier frequency of NB-IoT in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143 and the Offset of NB-IoT Channel Number to EARFCN in the range {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,-0.5,0,1,2,3,4,5,6,7,8,9} for FDD and TDD band 249.”

“The carrier frequency of NB-IoT in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 –262143, and the Offset of NB-IoT Channel Number to EARFCN in the range {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,0,1,2,3,4,5,6,7,8,9} for FDD and TDD band 249.”

Observation 4: Re-introduce specific requirements for TDD mode (e.g. transmit ON/OFF power Clauses 6.4 and 9.5 applicable at least to TS 36.108).

RAN4 decided to discuss/agree the following table and decided that transmitted signal quality has to be included in the specification:

Table 1: SAN RF Tx requirements for the new IoT-NTN TDD band (TS 36.108)

| NR SAN RF Tx requirement | Proposal for the new IoT-NTN TDD band (TS 36.108) |
| --- | --- |
| 6.2 Satellite Access Node output power | Not band specific --> No specification impact. |
| 6.3.2 RE power control dynamic range | Not band specific --> No specification impact. |
| 6.3.3 Total power dynamic range | Not band specific --> No specification impact. |
| 6.4 Transmit ON/OFF power | Add this specification to SAN starting from Rel-19 |
| 6.5 Transmitted signal quality | Not band specific --> No specification impact. |
| 6.6.2 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Not band specific --> No specification impact. |
| 6.6.4 Out-of-band emissions | Band specific --> Specification impact. Consider reuse of out-of-band emissions requirement for NTN IoT 251 similar as for NTN IoT 255 L-band Tx requirements (TS 36.108) |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | Not applicable to SAN. |

RAN4 decided to discuss/agree the following table and decided that the reference sensitivity could be reused from TS 36.108 following a similar process.

Table 2: SAN RF Rx requirements for the new IoT-NTN TDD band (TS 36.108)

| NR SAN RF Rx requirement | Proposal for the new IoT-NTN TDD band (TS 36.108) |
| --- | --- |
| 7.2 Reference sensitivity level | Not band specific --> No specification impact. Consider reuse of reference sensitivity level for NTN IoT 251 similar as for NTN IoT 254 L-band Rx requirements (TS 36.108) |
| 7.3 Dynamic range | Not band specific --> No specification impact. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Not band specific --> No specification impact. |
| 7.4.2 In-band blocking | Not applicable for SAN. |
| 7.5 Out-of-band blocking | Not band specific --> No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable for SAN. |
| 7.7 Receiver intermodulation | Not applicable for SAN --> No specification impact. |
| 7.8 In-channel selectivity | Not band specific --> No specification impact. |

Proposal 1: RAN4 to update the following Requirement set applicability from TS 36.108:

Table 4.6-1: Requirement set applicability

| Requirement | Requirement set |  |
| --- | --- | --- |
|  | SAN type 1-H | SAN type 1-O |
| SAN output power | 6.2 |  |
| Output power dynamics | 6.3 |  |
| Transmit ON/OFF power | NA 6.4 |  |
| Frequency error | 6.5.1 |  |
| Modulation quality | 6.5.2 |  |
| Time alignment error | NA |  |
| Occupied bandwidth | 6.6.2 |  |
| ACLR | 6.6.3 |  |
| Out-of-band emissions | 6.6.4 |  |
| Transmitter spurious emissions | 6.6.5 |  |
| Transmitter intermodulation | NA | NA |
| Reference sensitivity level | 7.2 |  |
| Dynamic range | 7.3 |  |
| ACS | 7.4.1 |  |
| In-band blocking | NA |  |
| Out-of-band blocking | 7.5 |  |
| Receiver spurious emissions | NA |  |
| Receiver intermodulation | NA |  |
| In-channel selectivity | 7.8 |  |
| Performance requirements | 8 |  |
| Radiated transmit power | 9.2 | 9.2 |
| OTA SAN output power |  | 9.3 |
| OTA output power dynamics |  | 9.4 |
| OTA transmit ON/OFF power |  | NA 9.5 |
| OTA frequency error |  | 9.6.1 |
| OTA modulation quality |  | 9.6.2 |
| OTA time alignment error |  | NA |
| OTA occupied bandwidth |  | 9.7.2 |
| OTA ACLR | NA | 9.7.3 |
| OTA out-of-band emission |  | 9.7.4 |
| OTA transmitter spurious emission |  | 9.7.5 |
| OTA transmitter intermodulation |  | NA |
| OTA sensitivity | 10.2 | 10.2 |
| OTA reference sensitivity level |  | 10.3 |
| OTA dynamic range |  | 10.4 |
| OTA ACS |  | 10.5.1 |
| OTA in-band blocking |  | NA |
| OTA out-of-band blocking | NA | 10.6 |
| OTA receiver spurious emission |  | NA |
| OTA receiver intermodulation |  | NA |
| OTA in-channel selectivity |  | 10.9 |
| Radiated performance requirements |  | 11 |

Proposal 2: Update the following Clause 6.4 in TS 36.108 with “The requirements in clause 6.4 are only applied for NB-IoT TDD SAN.”

## 6.4 Transmit ON/OFF power The requirement is not applicable in this version of the specification. ### The requirements in clause 6.4 are only applied for NB-IoT TDD SAN. ### 6.4.1 Transmitter OFF power Transmitter OFF power is defined as the mean power measured over 70 µs filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the SAN (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. #### 6.4.1.1 Minimum Requirement The transmitter OFF power spectral density shall be less than -85dBm/MHz. For SAN capable of multi-band operation, the requirement is only applicable during the transmitter OFF period in all supported operating bands. ### 6.4.2 Transmitter transient period The transmitter transient period is the time period during which the transmitter is changing from the OFF period to the ON period or vice versa. The transmitter transient period is illustrated in Figure 6.4.2-1. Figure 6.4.2-1 Illustration of the relations of transmitter ON period, transmitter OFF period and transmitter transient period. #### 6.4.2.1 Minimum requirements The transmitter transient period shall be shorter than the values listed in Table 6.4.2.1-1. Table 6.4.2.1-1 Minimum requirements for the transmitter transient period Transition; Transient period length [us]. OFF to ON; [17]. ON to OFF; [17].

Proposal 2bis: Update relevant clause “transmit ON/OFF power” for NB-IoT TDD SAN in TS 36.181 with TS 36.108 agreement.

Proposal 3: Update the following Clause 9.5 in TS 36.108 with “The requirements in clause 6.4 are only applied for NB-IoT TDD SAN.”

Update the following Clause 9.5 in TS 36.108 with “The requirements in clause 9.5 are only applied for NB-IoT TDD SAN.”

## 9.5 OTA transmit ON/OFF power The requirement is not applicable in this version of the specification. ### The requirements in clause 9.5 are only applied for NB-IoT TDD SAN. ### 9.5.2 OTA transmitter OFF power #### 9.5.2.1 General OTA transmitter OFF power is defined as the mean power measured over 70 us filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the SAN (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. For multi-band RIBs and single band RIBs supporting transmission in multiple bands, the requirement is only applicable during the transmitter OFF period in all supported operating bands. #### 9.5.2.2 Minimum requirement for SAN type 1-O The total power from all co-location reference antenna conducted output(s) shall be less than -106 dBm/MHz. ### 9.5.3 OTA transient period #### 9.5.3.1 General The OTA transmitter transient period is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1. This requirement shall be applied at each RIB supporting transmission in the operating band. #### 9.5.3.2 Minimum requirement for SAN type 1-O For SAN type 1-O, the OTA transmitter transient period shall be shorter than the values listed in the minimum requirement table 9.5.3.2-1. Table 9.5.3.2-1: Minimum requirement for the OTA transmitter transient period for SAN type 1-O Transition; Transient period length (µs). OFF to ON; [10]. ON to OFF; [10].

Proposal 3bis: Update clause “OTA transmit ON/OFF power” for NB-IoT TDD SAN in TS 36.181 with TS 36.108 agreement.

Proposal 4: For SAN TS 36.108, extra 0.5 dB for the REFSENS TDD compared to FDD due to the lack of duplexer.

Proposal 5: For SAN TS 36.108, adapt FRC configuration with new TDD frame structure (maximum 8 ms transmission time).

- Option 1: Reuse UE FRC (from TS 36.101) for SAN FRC.

Table A.3.2-2e Fixed Reference Channel for Receiver Requirements (TDD) – for CAT-NB1 and CAT-NB2

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 0.2 |
| Number of subcarriers |  | 12 |
| Uplink-Downlink Configuration (Note 7) |  | 1 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of HARQ Processes | Processes | 1 |
| Maximum number of HARQ transmissions |  | 1 |
| Transport block size | Bits | 88 |
| Number of Sub-Frames per transport block |  | 1 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits Per Sub-Frame | Bits | 320 |
| LTE CRS port |  | N/A |
| Number of NRS ports |  | 1 |
| Number of NPDSCH repetitions Number of NPUSCH repetitions |  | 1 |
| UE DL Category UE UL Category |  | NB1 or NB2 |
| Note 1: Category NB1 or NB2 in stand-alone mode has been considered here. Note 2: Reference signal, Synchronization signals and NPBCH allocated as per TS 36.211. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 4: Parameters related to NPDSCH scheduling are defined in Table A.3.2-1e to Table A.3.2-1g. Note 5: NPDCCH and information bit payload are not transmitted in the subframes used for transmission of SI messages. Note 6: SI messages transmission should be prioritized over NPDCCH transmission in case of collision. NPDCCH transmission is postponed until the next NB-IoT downlink subframe in case NPDCCH transmission occurs in a non NB-IoT downlink subframe, where an NB-IoT downlink subframe is a subframe that does not contain NPSS/NSSS/NPBCH/SIB1-NB transmission. Note 7: As per Table 4.2-2 in TS 36.211 [4] Note 8: Number of repetition NRep as defined in table 16.4.1.3-2 in TS 36.213 [6]. |  |  |

- Option 2: Adapt the existent SAN FRC.

  - For conducted specifications:

For SAN supporting NTN NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.1 with parameters specified in Table 7.2.2-3 and 7.2.2-4 for SAN type 1-H in all operating band. Table 7.2.2-3: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (GEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, PREFSENS (dBm). 200; 15; FRC A14-1 in Annex A.14; -124.9. 200; 3.75; FRC A14-2 in Annex A.14; -130.9. Table 7.2.2-4: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, PREFSENS (dBm). 200; 15; FRC A14-1 in Annex A.14; -128.0. 200; 3.75; FRC A14-2 in Annex A.14; -134.0. Table 7.2.2-5: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (GEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, PREFSENS (dBm). 200; 15; TBD in Annex A.14; TBD. 200; 3.75; TBD in Annex A.14; TBD. Table 7.2.2-6: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, PREFSENS (dBm). 200; 15; TBD in Annex A.14; TBD. 200; 3.75; TBD in Annex A.14; TBD.

  - For OTA specifications:

For SAN supporting standalone NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameter specified in table 10.3.2-3 and table 10.3.2-4 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA. Table 10.3.2-3: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (GEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, EISREFSENS (dBm). 200; 15; FRC A14-1 in Annex A.14; -124.9 - ΔOTAREFSENS. 200; 3.75; FRC A14-2 in Annex A.14; -130.9 - ΔOTAREFSENS. Table10.3.2-4: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, EISREFSENS (dBm). 200; 15; FRC A14-1 in Annex A.14; -128.0 - ΔOTAREFSENS. 200; 3.75; FRC A14-2 in Annex A.14; -134.0 - ΔOTAREFSENS. Table 10.3.2-3: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (GEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, EISREFSENS (dBm). 200; 15; TBD in Annex A.14; TBD- ΔOTAREFSENS. 200; 3.75; TBD in Annex A.14; TBD - ΔOTAREFSENS. Table10.3.2-4: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, EISREFSENS (dBm). 200; 15; TBD in Annex A.14; TBD - ΔOTAREFSENS. 200; 3.75; TBD in Annex A.14; TBD - ΔOTAREFSENS.

Proposal 6: Companies to further discuss FRC for reference sensitivity and in-channel selectivity in SAN 36.108 with respect to band 249 for better adaptation to 8ms TDD Tx time (corresponding to D/U=8).

Table A.3-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 | A14-2 |
| --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Diversity | No | No |
| Modulation | π/2 BPSK | π/2 BPSK |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) Note 1 | 4 | 16 |
| Number of NPUSCH repetition | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 |
| Payload size (bits) | 32 | 32 |
| Allocated resource unit | 2 | 2 |
| Code rate (target) | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 |
| Total symbols per resource unit | 96 | 96 |
| Tx time (ms) | 16 | 64 |
| Note: Channel estimation lengths are included in the table for information only. |  |  |

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 | A14-2 |
| --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Diversity | No | No |
| Modulation | π/2 BPSK | π/2 BPSK |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| Number of NPUSCH repetition | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 |
| Payload size (bits) | 32 | 32 |
| Allocated resource unit | 2 | 2 |
| Code rate (target) | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 |
| Total symbols per resource unit | 96 | 96 |
| Tx time (ms) | 16 | 64 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |

Proposal 7: RAN4 to clean up the spec TS 36.108 and remove redundant data e.g. A.3-1 FRC not cited at all in all the text.

Proposal 8: Companies to further discuss FRC for dynamic range in SAN 36.108 with respect to band 249 for better adaptation to 8ms TDD Tx time (corresponding to D/U=8).

Table A.4-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 | A15-2 |
| --- | --- | --- |
| Sub carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| Note: Channel estimation lengths are included in the table for information only. |  |  |

Table A.15-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 | A15-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |

Proposal 9: RAN4 to clean up the spec TS 36.108 and remove redundant data e.g. A.4-1 FRC not cited at all in all the text.

Proposal 10: Companies to further discuss FRC for NB-IoT NPUSCH format 1 in SAN 36.108 with respect to band 249 for better adaptation to 8 ms TDD Tx time (corresponding to D/U=8). Reference channel A7-2 could be used (A7-1 does not seem applicable).

Table A.7.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A7-1 | A7-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 |
| Number of allocated subcarriers | 1 | 12 |
| Diversity | No | No |
| Modulation | BPSK | QPSK |
| ITBS / IRU | 0 / 1 | 9 / 0 |
| Payload size (bits) | 32 | 136 |
| Allocated resource unit | 2 | 1 |
| Code rate (target) | 1/3 | 2/3 |
| Code rate (effective) | 0.29 | 0.56 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 288 |
| Total symbols per resource unit | 96 | 144 |
| Channel estimation length (ms) Note 1 | 16 | 2 (when repetition = 2) 4 (when repetition > 2) |

Proposal 11: Companies to discuss equations for driving requirements (e.g. REFSENS, in-channel selectivity, dynamic range), and SNR targets for specific configurations.

NOTE: TR 36.802 can be used as starting point for the equations, FRC (when applicable) and related SNR values.

Proposal 12: Companies may consider increasing the number of tones in order to decrease transmission time less than 8 ms (for compatibility with D/U=8 for the new TDD frame structure).

# SAN Specific TDD performance demodulation requirements for TS 36.108

Current TS 36.108 specification presents for NPUSCH:

## 8.5 Performance requirements for Narrowband IoT ### 8.5.1 Requirements for NPUSCH format 1 #### 8.5.1.1 Requirements The performance requirement of NPUSCH format 1 is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A.7. The performance requirements assume HARQ retransmissions. An NB-IoT Base Station supports 15 kHz subcarrier spacing requirements, or 3.75 kHz subcarrier spacing requirements, or both. For 15kHz subcarrier spacing multi-subcarrier, the demodulation requirements apply for the supported number of subcarriers. Table 8.5.1.1-1: Test parameters Parameter; Value. Maximum number of HARQ transmissions; 4. RV sequence; RV0, RV2. ##### 8.5.1.1.1 Minimum requirements The throughput shall be equal to or larger than the fraction of maximum throughput stated in table 8.5.1.1.1-1 for the single-subcarrier of 3.75KHz subcarrier spacing, and in table 8.5.1.1.1-2 for multi-subcarrier of 15KHz subcarrier spacing at the given SNR for 1Tx. Table 8.5.1.1.1-1: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 3.75KHz subcarrier spacing, 1Tx Number of TX antennas; Number of RX antennas; Subcarrier spacing; Number of allocated subcarriers; Propagation conditions and correlation matrix (Annex D)); FRC (Annex A); Frequency offset; TxDuration; Repetition number; Fraction of maximum throughput; SNR [dB]. 1; 1; 3.75KHz; 1; NTN TDLA100-1; A7-1; 128 Hz; 256; 4; 70%; -2.0. 1; 1; 3.75KHz; 1; NTN TDLC5-1; A7-1; 128 Hz; 256; 4; 70%; -3.6. 1; 2; 3.75KHz; 1; NTN TDLA100-1; A7-1; 128 Hz; 256; 4; 70%; -5.4. 1; 2; 3.75KHz; 1; NTN TDLC5-1; A7-1; 128 Hz; 256; 4; 70%; -6.5. Table 8.5.1.1.1-2: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx Number of TX antennas; Number of RX antennas; Subcarrier spacing; Number of allocated subcarriers; Propagation conditions and correlation matrix (Annex D); FRC (Annex A); Frequency offset; Tx-Duration; Repetition number; Fraction of maximum throughput; SNR [dB]. 1; 1; 15KHz; 12; NTN TDLA100-1; A7-2; 8 Hz; 16; 16; 70%; -3.2. 1; 1; 15KHz; 12; NTN TDLC5-1; A7-2; 8 Hz; 16; 16; 70%; -4.0. 1; 2; 15KHz; 12; NTN TDLA100-1; A7-2; 8 Hz; 16; 16; 70%; -7.1. 1; 2; 15KHz; 12; NTN TDLC5-1; A7-2; 8 Hz; 16; 16; 70%; -7.8. ### 8.5.2 Performance requirements for NPUSCH format 2 #### 8.5.2.1 DTX to ACK performance The DTX to ACK probability for NPUSCH format 2 case denotes the probability that ACK is detected when nothing is sent on the wanted signal and only the noise is present per NPUSCH format 2 transmission. An NB-IoT Base Station supports 15 KHz sub-carrier spacing requirements, or 3.75 KHz sub-carrier spacing requirements, or both. ##### 8.5.2.1.1 Minimum requirement The DTX to ACK probability, i.e. the probability that ACK is detected when nothing was sent, shall not exceed 1% per NPUSCH format 2 transmission. Where the performance measure definition is as follows: where: - #(false ACK bits) denotes the number of detected ACK bits. - #(ACK/NACK bits) denotes the number of HARQ-ACK information bit per NPUSCH format 2 transmission. - #( NPUSCH format 2 DTX) denotes the number of DTX occasions. #### 8.5.2.2 ACK missed detection requirements The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent per NPUSCH format 2 transmission. ##### 8.5.2.2.1 Minimum requirements The ACK missed detection probability shall not exceed 1% at the SNR given in table 8.5.2.2.1-1 and table 8.5.2.2.1-2 for 1Tx case. Table 8.5.2.2.1-1: Minimum requirements for NPUSCH format 2, 200KHz Channel Bandwidth, 3.75KHz subcarrier spacing, 1Tx Number of TX antennas; Number of RX antennas; Propagation conditions and correlation matrix (Annex D); Number of allocated subcarriers; Subcarrier spacing; Frequency offset; TxDuration; Repetition number; SNR [dB]. 1; 1; NTN TDLA100-1; 1; 3.75KHz; 64 Hz; 256; 4; 1.8. 1; 2; NTN TDLA100-1; 1; 3.75KHz; 64 Hz; 256; 4; -3.9. Table 8.5.2.2.1-2: Minimum requirements for NPUSCH format 2, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, 1Tx Number of TX antennas; Number of RX antennas; Propagation conditions and correlation matrix (Annex D); Number of allocated subcarriers; Subcarrier spacing; Frequency offset; TxDuration; Repetition number; SNR [dB]. 1; 1; NTN TDLA100-1; 1; 15KHz; 16 Hz; 32; 16]; 5.1. 1; 2; NTN TDLA100-1; 1; 15KHz; 16 Hz; 32; 16; -3.3.

Current TS 36.108 specification also says for NPRACH:

### 8.5.3 Performance requirements for NPRACH #### 8.5.3.1 NPRACH False alarm probability The false alarm requirement is valid for any number of receive antennas, for all repetition numbers and for any number of subcarriers. The false alarm probability is the conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise. ##### 8.5.3.1.1 Minimum requirement The false alarm probability shall be less than or equal to 0.1%. #### 8.5.3.2 NPRACH detection requirements The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. For NTN-TDLA100 and NTN-TDLC5, a timing estimation erros occurs if the estimation error of the timing of the largest delay path is larger than the time orror tolerance given in Table 8.5.3.2-2.

Table 8.5.3.2-1: Test preambles for NPRACH Parameter; Value. Narrowband physical layer cell identity; 0. Initial subcarrier index; 0. Table 8.5.3.2-2: Time error tolerance for AWGN, NTN-TDLA100-1 PRACH; PRACH SCS; Time error tolerance. preamble; (kHz); NTN-TDLA100-1. 0; 3.75KHz; 3.646us. 1; 3.75KHz; 3.646us. ##### 8.5.3.2.1 Minimum requirements The probability of detection shall be equal to or exceed 99% for the SNR levels listed in table 8.5.3.2.1-1. Table 8.5.3.2.1-1: NPRACH missed detection requirements for FDD Number of TX antennas; Number of RX antennas; TxDuration; Repetition number; Propagation conditions and correlation matrix (Annex D); Frequency offset; SNR [dB]. Preamble format 0; Preamble format 1. 1; 1; 8; 8; NTN-TDLA100-1; 200 Hz; 13.3; 13.7. 1; 1; 16; 16; NTN-TDLA100-1; 200 Hz; 9.8; 9.6. 1; 2; 8; 8; NTN-TDLA100-1; 200 Hz; 6.0; 6.1. 1; 2; 16; 16; NTN-TDLA100-1; 200 Hz; 1.6; 1.7.

Proposal 13: Update the NPRACH performance accordingly for TDD 249 band:

### 8.5.3	Performance requirements for NPRACH

#### 8.5.3.1	NPRACH False alarm probability

The false alarm requirement is valid for any number of receive antennas, for all repetition numbers and for any number of subcarriers.

The false alarm probability is the conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

##### 8.5.3.1.1	Minimum requirement

The false alarm probability shall be less than or equal to 0.1%.

#### 8.5.3.2	NPRACH detection requirements

The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. For NTN-TDLA100 and NTN-TDLC5, a timing estimation errors occurs if the estimation error of the timing of the largest delay path is larger than the time error tolerance given in Table 8.5.3.2-2.

The requirements for TDD are optional and only valid for SAN supporting TDD.

Table 8.5.3.2-1: Test preambles for NPRACH

| Parameter | Value |
| --- | --- |
| Narrowband physical layer cell identity | 0 |
| Initial subcarrier index | 0 |
| Uplink-downlink configuration for TDD | N=9 ; D/U=8 TBD (currently 1 in the TN) |
| Special subframe configuration for TDD | TBD (currently 7 in the TN) |

Table 8.5.3.2-2: Time error tolerance for AWGN, NTN-TDLA100-1

| PRACH | PRACH SCS | Time error tolerance |
| --- | --- | --- |
| preamble | (kHz) | NTN-TDLA100-1 |
| 0 | 3.75KHz | 3.646us |
| 1 | 3.75KHz | 3.646us |

Proposal 14: Add a new table for NPRACH missed detection requirements for TDD. Consider 2 preamble formats: Preamble format 0 and Preamble format 1.

Table 8.5.3.2.1-2: NPRACH missed detection requirements for TDD

| Number of TX antennas | Number of RX antennas | TxDuration | Repetition number | Propagation conditions and correlation matrix (Annex D) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Preamble format 0 | Preamble format 1 |
| 1 | 1 | TBD (currently 8) | TBD (currently 8) | NTN-TDLA100-1 | 200 Hz | TBD | TBD |
| 1 | 1 | TBD (currently 16) | TBD (currently 16) | NTN-TDLA100-1 | 200 Hz | TBD | TBD |
| 1 | 2 | TBD (currently 8) | TBD (currently 8) | NTN-TDLA100-1 | 200 Hz | TBD | TBD |
| 1 | 2 | TBD (currently 16) | TBD (currently 16) | NTN-TDLA100-1 | 200 Hz | TBD | TBD |

Proposal 15: Consider only A7-2 (and not A7-1) for demodulation performance requirements for band 249 TDD NB-IoT. In other words, Consider only NPUSCH format 1 with 15 kHz and 12 sub-carriers (and potentially also NPUSCH format 1) for demodulation performance requirements for band 249 TDD NB-IoT.

Table A.7.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A7-1 | A7-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 |
| Number of allocated subcarriers | 1 | 12 |
| Diversity | No | No |
| Modulation | BPSK | QPSK |
| ITBS / IRU | 0 / 1 | 9 / 0 |
| Payload size (bits) | 32 | 136 |
| Allocated resource unit | 2 | 1 |
| Code rate (target) | 1/3 | 2/3 |
| Code rate (effective) | 0.29 | 0.56 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 288 |
| Total symbols per resource unit | 96 | 144 |
| Channel estimation length (ms) Note 1 | 16 | 2 (when repetition = 2) 4 (when repetition > 2) |

Proposal 16: Revisit demodulation requirements by taking into account specific TDD system constraints with D/U=8:

Table 8.5.1.1.1-3: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx, for TDD

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 8 Hz | 16 TBD | 16 TBD | 70% | TBD |
| 1 | 1 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 8 Hz | 16 TBD | 16 TBD | 70% | TBD |
| 1 | 2 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 8 Hz | 16 TBD | 16 TBD | 70% | TBD |
| 1 | 2 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 8 Hz | 16 TBD | 16 TBD | 70% | TBD |

Proposal 17: Add for EVM clause

“For TDD the averaging in the time domain can be calculated from subframes of different frames and should have a minimum of 10 subframes averaging length. TDD special fields (DwPTS and GP) are not included in the averaging.”

# Conclusion

Observation 4: Re-introduce specific requirements for TDD mode (e.g. transmit ON/OFF power Clauses 6.4 and 9.5 applicable at least to TS 36.108).

Proposal 1: RAN4 to update the following Requirement set applicability from TS 36.108:

Table 4.6-1: Requirement set applicability

| Requirement | Requirement set |  |
| --- | --- | --- |
|  | SAN type 1-H | SAN type 1-O |
| SAN output power | 6.2 |  |
| … | … |  |
| Transmit ON/OFF power | NA 6.4 |  |
| … | … |  |
| Transmitter intermodulation | NA | NA |
| … | … |  |
| Performance requirements | 8 |  |
| Radiated transmit power | 9.2 | 9.2 |
| … |  | … |
| OTA transmit ON/OFF power |  | NA 9.5 |
| … |  | … |
| OTA ACLR | NA | 9.7.3 |
| … |  | … |

Proposal 2: Update the following Clause 6.4 in TS 36.108 with “The requirements in clause 6.4 are only applied for NB-IoT TDD SAN.”

## 6.4 Transmit ON/OFF power The requirement is not applicable in this version of the specification. ### The requirements in clause 6.4 are only applied for NB-IoT TDD SAN. ### 6.4.1 Transmitter OFF power Transmitter OFF power is defined as the mean power measured over 70 µs filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the SAN (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. #### 6.4.1.1 Minimum Requirement The transmitter OFF power spectral density shall be less than -85dBm/MHz. For SAN capable of multi-band operation, the requirement is only applicable during the transmitter OFF period in all supported operating bands. ### 6.4.2 Transmitter transient period The transmitter transient period is the time period during which the transmitter is changing from the OFF period to the ON period or vice versa. The transmitter transient period is illustrated in Figure 6.4.2-1. Figure 6.4.2-1 Illustration of the relations of transmitter ON period, transmitter OFF period and transmitter transient period. #### 6.4.2.1 Minimum requirements The transmitter transient period shall be shorter than the values listed in Table 6.4.2.1-1. Table 6.4.2.1-1 Minimum requirements for the transmitter transient period Transition; Transient period length [us]. OFF to ON; [17]. ON to OFF; [17].

Proposal 2bis: Update relevant clause “transmit ON/OFF power” for NB-IoT TDD SAN in TS 36.181 with TS 36.108 agreement.

Proposal 3: Update the following Clause 9.5 in TS 36.108 with “The requirements in clause 6.4 are only applied for NB-IoT TDD SAN.”

Update the following Clause 9.5 in TS 36.108 with “The requirements in clause 9.5 are only applied for NB-IoT TDD SAN.”

## 9.5 OTA transmit ON/OFF power The requirement is not applicable in this version of the specification. ### The requirements in clause 9.5 are only applied for NB-IoT TDD SAN. ### 9.5.2 OTA transmitter OFF power #### 9.5.2.1 General OTA transmitter OFF power is defined as the mean power measured over 70 us filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the SAN (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. For multi-band RIBs and single band RIBs supporting transmission in multiple bands, the requirement is only applicable during the transmitter OFF period in all supported operating bands. #### 9.5.2.2 Minimum requirement for SAN type 1-O The total power from all co-location reference antenna conducted output(s) shall be less than -106 dBm/MHz. ### 9.5.3 OTA transient period #### 9.5.3.1 General The OTA transmitter transient period is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1. This requirement shall be applied at each RIB supporting transmission in the operating band. #### 9.5.3.2 Minimum requirement for SAN type 1-O For SAN type 1-O, the OTA transmitter transient period shall be shorter than the values listed in the minimum requirement table 9.5.3.2-1. Table 9.5.3.2-1: Minimum requirement for the OTA transmitter transient period for SAN type 1-O Transition; Transient period length (µs). OFF to ON; [10]. ON to OFF; [10].

Proposal 3bis: Update clause “OTA transmit ON/OFF power” for NB-IoT TDD SAN in TS 36.181 with TS 36.108 agreement.

Proposal 4: For SAN TS 36.108, extra 0.5 dB for the REFSENS TDD compared to FDD due to the lack of duplexer.

Proposal 5: For SAN TS 36.108, adapt FRC configuration with new TDD frame structure (maximum 8 ms transmission time).

- Option 1: Reuse UE FRC (from TS 36.101) for SAN FRC.

Table A.3.2-2e Fixed Reference Channel for Receiver Requirements (TDD) – for CAT-NB1 and CAT-NB2

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 0.2 |
| Number of subcarriers |  | 12 |
| Uplink-Downlink Configuration (Note 7) |  | 1 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of HARQ Processes | Processes | 1 |
| Maximum number of HARQ transmissions |  | 1 |
| Transport block size | Bits | 88 |
| Number of Sub-Frames per transport block |  | 1 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits Per Sub-Frame | Bits | 320 |
| LTE CRS port |  | N/A |
| Number of NRS ports |  | 1 |
| Number of NPDSCH repetitions Number of NPUSCH repetitions |  | 1 |
| UE DL Category UE UL Category |  | NB1 or NB2 |
| Note 1: Category NB1 or NB2 in stand-alone mode has been considered here. Note 2: Reference signal, Synchronization signals and NPBCH allocated as per TS 36.211. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 4: Parameters related to NPDSCH scheduling are defined in Table A.3.2-1e to Table A.3.2-1g. Note 5: NPDCCH and information bit payload are not transmitted in the subframes used for transmission of SI messages. Note 6: SI messages transmission should be prioritized over NPDCCH transmission in case of collision. NPDCCH transmission is postponed until the next NB-IoT downlink subframe in case NPDCCH transmission occurs in a non NB-IoT downlink subframe, where an NB-IoT downlink subframe is a subframe that does not contain NPSS/NSSS/NPBCH/SIB1-NB transmission. Note 7: As per Table 4.2-2 in TS 36.211 [4] Note 8: Number of repetition NRep as defined in table 16.4.1.3-2 in TS 36.213 [6]. |  |  |

- Option 2: Adapt the existent SAN FRC.

  - For conducted specifications:

For SAN supporting NTN NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.1 with parameters specified in Table 7.2.2-3 and 7.2.2-4 for SAN type 1-H in all operating band. Table 7.2.2-3: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (GEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, PREFSENS (dBm). 200; 15; FRC A14-1 in Annex A.14; -124.9. 200; 3.75; FRC A14-2 in Annex A.14; -130.9. Table 7.2.2-4: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, PREFSENS (dBm). 200; 15; FRC A14-1 in Annex A.14; -128.0. 200; 3.75; FRC A14-2 in Annex A.14; -134.0. Table 7.2.2-5: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (GEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, PREFSENS (dBm). 200; 15; TBD in Annex A.14; TBD. 200; 3.75; TBD in Annex A.14; TBD. Table 7.2.2-6: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, PREFSENS (dBm). 200; 15; TBD in Annex A.14; TBD. 200; 3.75; TBD in Annex A.14; TBD.

  - For OTA specifications:

For SAN supporting standalone NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameter specified in table 10.3.2-3 and table 10.3.2-4 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA. Table 10.3.2-3: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (GEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, EISREFSENS (dBm). 200; 15; FRC A14-1 in Annex A.14; -124.9 - ΔOTAREFSENS. 200; 3.75; FRC A14-2 in Annex A.14; -130.9 - ΔOTAREFSENS. Table10.3.2-4: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, EISREFSENS (dBm). 200; 15; FRC A14-1 in Annex A.14; -128.0 - ΔOTAREFSENS. 200; 3.75; FRC A14-2 in Annex A.14; -134.0 - ΔOTAREFSENS. Table 10.3.2-3: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (GEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, EISREFSENS (dBm). 200; 15; TBD in Annex A.14; TBD- ΔOTAREFSENS. 200; 3.75; TBD in Annex A.14; TBD - ΔOTAREFSENS. Table10.3.2-4: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, EISREFSENS (dBm). 200; 15; TBD in Annex A.14; TBD - ΔOTAREFSENS. 200; 3.75; TBD in Annex A.14; TBD - ΔOTAREFSENS.

Proposal 6: Companies to further discuss FRC for reference sensitivity and in-channel selectivity in SAN 36.108 with respect to band 249 for better adaptation to 8ms TDD Tx time (corresponding to D/U=8).

Table A.3-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 | A14-2 |
| --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Diversity | No | No |
| Modulation | π/2 BPSK | π/2 BPSK |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) Note 1 | 4 | 16 |
| Number of NPUSCH repetition | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 |
| Payload size (bits) | 32 | 32 |
| Allocated resource unit | 2 | 2 |
| Code rate (target) | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 |
| Total symbols per resource unit | 96 | 96 |
| Tx time (ms) | 16 | 64 |
| Note: Channel estimation lengths are included in the table for information only. |  |  |

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 | A14-2 |
| --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Diversity | No | No |
| Modulation | π/2 BPSK | π/2 BPSK |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| Number of NPUSCH repetition | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 |
| Payload size (bits) | 32 | 32 |
| Allocated resource unit | 2 | 2 |
| Code rate (target) | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 |
| Total symbols per resource unit | 96 | 96 |
| Tx time (ms) | 16 | 64 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |

Proposal 7: RAN4 to clean up the spec TS 36.108 and remove redundant data e.g. A.3-1 FRC not cited at all in all the text.

Proposal 8: Companies to further discuss FRC for dynamic range in SAN 36.108 with respect to band 249 for better adaptation to 8ms TDD Tx time (corresponding to D/U=8).

Table A.4-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 | A15-2 |
| --- | --- | --- |
| Sub carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| Note: Channel estimation lengths are included in the table for information only. |  |  |

Table A.15-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 | A15-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |

Proposal 9: RAN4 to clean up the spec TS 36.108 and remove redundant data e.g. A.4-1 FRC not cited at all in all the text.

Proposal 10: Companies to further discuss FRC for NB-IoT NPUSCH format 1 in SAN 36.108 with respect to band 249 for better adaptation to 8 ms TDD Tx time (corresponding to D/U=8). Reference channel A7-2 could be used (A7-1 does not seem applicable).

Table A.7.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A7-1 | A7-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 |
| Number of allocated subcarriers | 1 | 12 |
| Diversity | No | No |
| Modulation | BPSK | QPSK |
| ITBS / IRU | 0 / 1 | 9 / 0 |
| Payload size (bits) | 32 | 136 |
| Allocated resource unit | 2 | 1 |
| Code rate (target) | 1/3 | 2/3 |
| Code rate (effective) | 0.29 | 0.56 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 288 |
| Total symbols per resource unit | 96 | 144 |
| Channel estimation length (ms) Note 1 | 16 | 2 (when repetition = 2) 4 (when repetition > 2) |

Proposal 11: Companies to discuss equations for driving requirements (e.g. REFSENS, in-channel selectivity, dynamic range), and SNR targets for specific configurations.

NOTE: TR 36.802 can be used as starting point for the equations, FRC (when applicable) and related SNR values.

Proposal 12: Companies may consider increasing the number of tones in order to decrease transmission time less than 8 ms (for compatibility with D/U=8 for the new TDD frame structure).

Proposal 13: Update the NPRACH performance accordingly for TDD 249 band:

### 8.5.3	Performance requirements for NPRACH

#### 8.5.3.1	NPRACH False alarm probability

The false alarm requirement is valid for any number of receive antennas, for all repetition numbers and for any number of subcarriers.

The false alarm probability is the conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

##### 8.5.3.1.1	Minimum requirement

The false alarm probability shall be less than or equal to 0.1%.

#### 8.5.3.2	NPRACH detection requirements

The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. For NTN-TDLA100 and NTN-TDLC5, a timing estimation errors occurs if the estimation error of the timing of the largest delay path is larger than the time error tolerance given in Table 8.5.3.2-2.

The requirements for TDD are optional and only valid for SAN supporting TDD.

Table 8.5.3.2-1: Test preambles for NPRACH

| Parameter | Value |
| --- | --- |
| Narrowband physical layer cell identity | 0 |
| Initial subcarrier index | 0 |
| Uplink-downlink configuration for TDD | N=9 ; D/U=8 TBD (currently 1 in the TN) |
| Special subframe configuration for TDD | TBD (currently 7 in the TN) |

Table 8.5.3.2-2: Time error tolerance for AWGN, NTN-TDLA100-1

| PRACH | PRACH SCS | Time error tolerance |
| --- | --- | --- |
| preamble | (kHz) | NTN-TDLA100-1 |
| 0 | 3.75KHz | 3.646us |
| 1 | 3.75KHz | 3.646us |

Proposal 14: Add a new table for NPRACH missed detection requirements for TDD. Consider 2 preamble formats: Preamble format 0 and Preamble format 1.

Table 8.5.3.2.1-2: NPRACH missed detection requirements for TDD

| Number of TX antennas | Number of RX antennas | TxDuration | Repetition number | Propagation conditions and correlation matrix (Annex D) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Preamble format 0 | Preamble format 1 |
| 1 | 1 | TBD (currently 8) | TBD (currently 8) | NTN-TDLA100-1 | 200 Hz | TBD | TBD |
| 1 | 1 | TBD (currently 16) | TBD (currently 16) | NTN-TDLA100-1 | 200 Hz | TBD | TBD |
| 1 | 2 | TBD (currently 8) | TBD (currently 8) | NTN-TDLA100-1 | 200 Hz | TBD | TBD |
| 1 | 2 | TBD (currently 16) | TBD (currently 16) | NTN-TDLA100-1 | 200 Hz | TBD | TBD |

Proposal 15: Consider only A7-2 (and not A7-1) for demodulation performance requirements for band 249 TDD NB-IoT. In other words, Consider only NPUSCH format 1 with 15 kHz and 12 sub-carriers (and potentially also NPUSCH format 1) for demodulation performance requirements for band 249 TDD NB-IoT.

Table A.7.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A7-1 | A7-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 |
| Number of allocated subcarriers | 1 | 12 |
| Diversity | No | No |
| Modulation | BPSK | QPSK |
| ITBS / IRU | 0 / 1 | 9 / 0 |
| Payload size (bits) | 32 | 136 |
| Allocated resource unit | 2 | 1 |
| Code rate (target) | 1/3 | 2/3 |
| Code rate (effective) | 0.29 | 0.56 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 288 |
| Total symbols per resource unit | 96 | 144 |
| Channel estimation length (ms) Note 1 | 16 | 2 (when repetition = 2) 4 (when repetition > 2) |

Proposal 16: Revisit demodulation requirements by taking into account specific TDD system constraints with D/U=8:

Table 8.5.1.1.1-3: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx, for TDD

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 8 Hz | 16 TBD | 16 TBD | 70% | TBD |
| 1 | 1 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 8 Hz | 16 TBD | 16 TBD | 70% | TBD |
| 1 | 2 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 8 Hz | 16 TBD | 16 TBD | 70% | TBD |
| 1 | 2 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 8 Hz | 16 TBD | 16 TBD | 70% | TBD |

Proposal 17: Add for EVM clause

“For TDD the averaging in the time domain can be calculated from subframes of different frames and should have a minimum of 10 subframes averaging length. TDD special fields (DwPTS and GP) are not included in the averaging.”

# Annex – previous meeting agreements

The WF covers the contributions submitted under the following AI:

- 7.28.1 General aspects and work plan	[IoT_NTN_TDD]

- 7.28.2 Study of impact due to periodic pattern	[IoT_NTN_TDD-Core]

### Issue 0: Parameters of the baseline TDD pattern for analysis on the impact of requirements

Agreement:

- RAN4 to consider the following values for analyzing the impact of requirements, as necessary:

  - N = 9, D = {8, 20, 30}, U = {8, 20, 30}

  - N = 8, D = 20, U = 20

- Among the above sets of values, the set with N = 9 and D = 8 is considered the baseline TDD pattern for analysis, as it represents the most challenging case in terms of requirement analysis.

Note 1: The above values, based on RAN1’s discussion in RAN1#118bis meeting, are recommended examples for assessing the impact of requirements on RF, RRM, and Demodulation aspects. The exact values are subject to RAN1’s decision.

Note 2: In terms of SNR, the above analysis focuses on the requirements for normal coverage.

### Issue 1-2: Impact on SAN RF requirements

Agreement:

- The assessment of the impact on SAN RF requirements (TS 36.108) shown in the table below will serve as a baseline for further discussion on the exact requirement definition. For the requirements to be impacted for [249] band, if any, NTN IoT 254 L-band Tx/Rx requirements in TS 36.108 will serve as a baseline. If identified, new requirements can be defined for SAN RF, e.g. transient period.

| 6.2 Satellite Access Node output power | Not band specific --> No specification impact. |
| --- | --- |
| 6.3.2 RE power control dynamic range | Not band specific --> No specification impact. |
| 6.3.3 Total power dynamic range | Not band specific --> No specification impact. |
| 6.4 Transmit ON/OFF power | Add this specification to SAN starting from Rel-19 |
| 6.5 Transmitted signal quality | Not band specific --> No specification impact. |
| 6.6.2 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Not band specific --> No specification impact. |
| 6.6.4 Out-of-band emissions | Band specific --> FFS: Specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | Not applicable to SAN. |
| 7.2 Reference sensitivity level | Not band specific --> No specification impact. |
| 7.3 Dynamic range | Not band specific --> No specification impact. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Not band specific --> No specification impact. |
| 7.4.2 In-band blocking | Not applicable for SAN. |
| 7.5 Out-of-band blocking | Not band specific --> No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable for SAN. |
| 7.7 Receiver intermodulation | Not applicable for SAN --> No specification impact. |
| 7.8 In-channel selectivity | Not band specific --> No specification impact. |

### Issue 1-3: Impact on UE RF requirements

Agreement:

- The assessment of the impact on UE RF requirements (TS 36.102) shown in the table below will serve as a baseline for further discussion on the exact requirement definition.

| 36.102 Clause: Requirements | Band Agnostic | Re-use for TDD band [249] | Note |
| --- | --- | --- | --- |
| 6.2B.1: UE maximum output power | Yes | Yes | Support UE power class 3 (23 dBm) Support UE power class 5 (20 dBm) |
| 6.2B.1: UE maximum output power tolerance | No |  |  |
| 6.2B.2: UE maximum output power reduction | Yes | Yes |  |
| 6.2B.3: UE additional maximum output power reduction | No | - | Note: Reuse which band A-MPR, B254 A-MPR is not defined yet and the band [249] UL frequency range is not fully aligned with B254. |
| 6.2.B.4: Configured transmit power | Yes | Yes |  |
| 6.3: Output power dynamics | Yes | Yes FFS | Note: For 36.102 6.3B.4, 36.102 refers to 36.101 6.3.5F, the testing period for aggerate power control may depend on different UL pattern. |
| 6.4: Transmit signal quality | Yes | Yes |  |
| 6.5B.3.2: Spectrum emission mask | Yes | Yes |  |
| 6.5B.3.4: Adjacent Channel Leakage Ratio | Yes | Yes |  |
| 6.5B.4.2: General spurious emissions | Yes | Yes |  |
| 6.5B.4.3: Spurious emissions for UE co-existence | No |  | Band [249] needs to be added in Table 6.5A.4.3-1 Spurious emission band UE co-existence. |
| 6.5B.4.4: Additional spurious emissions | No |  | Additional spurious emissions requirement for B254 can be reused for the new IoT-NTN TDD L-band. UL frequency range of B254 is superset of [B249]. |
| 6.6: Transmit intermodulation for category NB1 and NB2 | Yes | Yes |  |
| 7.2: Diversity characteristics | Yes | Yes |  |
| 7.3: Reference sensitivity power level | No | No | To be studied for IoT-NTN TDD mode. |
| 7.4: Maximum input level | Yes | Yes |  |
| 7.5: Adjacent Channel Selectivity | Yes | Yes |  |
| 7.6B.2: In-band blocking | No | Yes | Different band group may have different In-band blocking tables for Cat-M1 and NB1/NB2 separately. Currently, bands 255,254, and 253 apply the same IBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use IBB requirements from B255,254 and 253 |
| 7.6B.3: Out-of-band blocking | No |  | Different Out-of-band requirements for IoT NTN bands. Currently, bands 255,254, and 253 apply the same OBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use OBB requirements from B255, 254 and 253 |
| 7.6B.4: Narrow band blocking | No | Yes |  |
| 7.7: Spurious response | Yes | Yes |  |
| 7.8: Intermodulation characteristics | Yes | Yes |  |
| 7.9: RX spurious emission | No | Yes | Re-use spurious emission requirements from Table 7.9.1-1 of 36.101 |
| A.1 DL reference measurement channels |  |  | NPDSCH Reference Channel for TDD should be introduced. Fixed Reference Channel for TDD should be introduced. |
| A.2 OFDMA Channel Noise Generator (OCNG) |  |  | OCNG Patterns for TDD should be introduced. |
| A.3 Testing related to Satellite Access |  |  | No impact. |
| A.4 UL reference measurement channels |  |  | Reference measurement channels for TDD should be introduced. |

### Issue 2-1: Impact on RRM requirements

Agreement:

- From the RRM requirement point of view, no impact on the following requirements is expected due to the new periodic pattern(s):

  - UE uplink timing

  - PUR

Agreement:

- For the following RRM requirements, modifications are expected due to the new periodic pattern(s):

  - RLM (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - MSG3-based channel quality report (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - Cell re-selection (example modifications include extended cell detection/evaluation/measurement periods, side conditions, etc.)

  - RRC Re-establishment (example modifications include extended cell detection/measurement periods, side conditions, etc.)

  - RRC Connection Redirection to Non-anchor Carrier (example modifications include updates to the definition of the symbols, side conditions, etc.)

  - Cell measurements (example modifications include extended cell detection/measurement periods, side conditions, etc.)

Note 1: The exact requirement modifications are subject to further discussion in future RAN4 meetings based on RAN1 conclusion/progress on this feature.

Note 2: Different modifications can be made for different TDD patterns, if multiple, DRX configurations, SNR values, etc.

Note 3: Not all existing requirements for NB-IoT NTN may be applicable to the IoT_NTN_TDD feature, which is subject to further discussions in future RAN4 meetings.

Note 4: It is assumed that UE will be made aware of the periodic pattern of frequency layers to be measured.

Note 5: UE power consumption aspect may be considered when modifying the above requirements as needed.

Note 6: The requirement applicability rule defined in Rel-17/18 applies to the IoT_NTN_TDD feature as well. For instance, the above requirements may not be applicable for inter-satellite scenarios if the UE is not provided with the target inter-satellite’s information.

Demodulation aspect:

No observation in this meeting.

# Annex for information – previous meeting discussions

### Issue 1-1: Operating Bands

<Band number>

- RAN4 to consider the following proposals for the requirement definition during the normative work item phase in future RAN4 meetings.

  - Assign “[249]” E-UTRA operating band number for the new TDD [1616-1626.5] MHz NTN NB-IoT frequency band introduction.

E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 2532 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525 MHz | FDD |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| [249] | [1616] MHz | - | [1626.5] MHz | [1616] MHz | - | [1626.5] MHz | TDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. |  |  |  |  |  |  |  |

  - Use a channel raster of 100 kHz and a Step size of <1>

  - Both FDL_low and FUL_low equal 1616 MHz.

  - For the range of both NDL and NUL:

    - Option 1: Starts from 2^18-2^16-1=196607 (below FDD NTN NOffs-DL) or 2^18-2^14-1=245759 (in-between FDD NTN NOffs-DL and FDD NTN NOffs-UL)

    - Option 2: {198196 - 198300}