3GPP TSG RAN WG4 Meeting #114	R4-2502064

Athens, Greece, 17 Feb. 2025 – 21 Feb. 2025

Agenda Item:	7.27.2.1

Source: 	Ericsson

Title:	Discussion on measurements enhancements for purpose of supporting LTM

Document for:	Discussion

# Introduction

In this contribution we discuss Event triggered measurement requirements based L1 measurement requirements.

# Discussion

## Event triggered measurement requirements

### L1-RSRP requirements for event triggered reporting

Event triggered L1-RSRP reporting delay:

Following agreements are reached for defining the event triggered measurement reporting delay.

- Starting point in event triggered L1 reporting requirements is the time when a condition exists at the UE which will trigger the reporting. - Ending point in event triggered L1 report requirements is defined as the point when the UE starts to transmit the first UL transmission to report measurement result over the air interface. - Agree on the following high-level proposal: - The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period [on the cells corresponding to the event or on the candidate cells]. - FFS reuse L1-RSRP measurement period from the existing spec with Treport = 0. - Additional time TSSB_time_index for SSB index acquisition is needed, unless deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell.

According to the current L1 measurement period requirement, measurement occasion depends on the measurement report periodicity. This is because, even if the UE measures frequently, it can only report as often as the measurement report periodicity. However, we think the same doesn't apply to L1 event-triggered reporting, since the UE doesn't know when the event entering condition will be met. Hence, we think that the UE needs to measure at every measurement occasion for the event triggered measurement reporting and since the LTM L1-RSRP is assumed to based on the single shot measurement, the measurement period includes the time required for one sample. We believe this is similar to the aperiodic reporting discussed in the Rel-18 LTM. We copy the agreement from R4-2406502 below for reference.

Issue 2-2-4: Measurement period for aperiodic reporting <Agreement> If a LTM CSI report is configured as aperiodic reporting, measurement period for aperiodic reporting is same as the existing measurement period requirements specified in clause 9.14 and 9.15 with TReport = 0.

Another open issue in the last meeting WF was whether to specify the delay on the [on the cells corresponding to the event or on the candidate cells]. The main intention behind this text is, not all the events may be evaluated on the candidate cells. E.g., LTM2 is evaluated on the serving cell. We think similar issue exists in L3 event triggered reporting and, in the legacy, it was mentioned as the event that may trigger the measurement report. We suggest following the similar approach.

- Measurement period of event triggered measurement reporting shall be no larger than the LTM L1-RSRP measurement period with TReport= 0. Where L1-RSRP measurement period corresponds to the cell that triggers the event triggered reporting.

Impact of filtering on measurement period:

In Rel-18, LTM L1-RSRP measurement period is based on the single shot measurement (i.e., M is 1). However, event triggered measurement report main purpose is to reduce the signalling overhead in the UL. To avoid false event triggering reports due to fast fading, RAN1 is discussing filtering for the event triggered measurement reporting. If the filtering is agreed, we need to define new measurement delay with higher number of samples than single shot measurement. That means measurement period may contain single shot or multiple samples measurement.

- Measurement period of the L1-RSRP can be revisited based on RAN1 conclusion on filtering.

Impact of TTT on the measurement period:

TTT was not considered in L3 event triggered measurement reporting requirements. If the UE can report the measurement without TTT, within certain delay, we think UE should not have any difficulty in sending the same measurement report after the TTT. Moreover, in the RAN4 test, if the TTT needs to be tested, test procedure may become complicated. We do not see a big gain in considering the TTT for event triggered reporting delay requirements.

- RAN4 to not consider TTT (time to trigger) into account in the L1 event-triggered reporting delay requirements.

### Requirements for reporting criteria per measurement category

In last meeting there was a proposal to share the capability of L3 events with the LTM events or introduce a new capability.  Some of the proposals discussed was mentioned below.

- Option 1: Yes (CATT, MTK, ZTE, Apple, OPPO)

- Option 2: RAN4 to postpone the detailed discussion on ‘Requirements for reporting criteria per measurement category’ until the RRM requirements on the remaining issues are sorted out (QC, Ericsson)

- Option 3: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo)

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells |

We think L3 measurement is configured by CU while L1 measurement may be configured by DU. Hence, it’s difficult to coordinate between CU and DU on the Ecat capability for LTM L1 events. Moreover, based on our internal check with the product implementation team, we think the existing Ecat capability is fully utilized as different applications may need to be configured with different events.

- The existing Ecat capability is already fully utilized in product.

- RAN4 to agree that existing Ecat capability is not shared between L3 events and LTM L1 events.

We think RAN4 first needs to discuss and decide on the granularity of the LTM L1 events capability. Legacy Ecat was defined for frequency layer and different Ecat is defined for intra-frequency, inter-frequency and so on. Since, LTM CSI config may contain multiple cells consisting of both intra-frequency and inter-frequency measurements, we think sharing a combined Ecat between intra and inter-frequency allows the NW flexibility to configure different cells as per the channel or load conditions.

- RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells.

RAN2 agreed on introducing the following events for the purpose of event triggering measurement reporting.

| Event | Description |
| --- | --- |
| Event LTM2 | Beam of serving cell becomes worse than absolute threshold; |
| Event LTM3 | Beam of candidate cell becomes amount of offset better than beam of serving cell; |
| Event LTM4 | Beam of candidate cell becomes better than absolute threshold; |
| Event LTM5 | Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. |

The events are designed to support the event evaluation on the PCell/serving cells and candidate cells. We think RAN4 should consider this when discussing the Ecat capability. Since agreeing on actual number requires further analysis, we suggest the actual number to be FFS.

- RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS.

### Impact of event triggered to reporting to LTM procedures

Our understanding of the use case of the RAN2 introduced events is as follows:

- Event LTM2: As the name suggests this event is similar to event A2 for the event triggered L3 measurements. When the beam of the serving cell becomes worse than absolute threshold, NW may request an aperiodic report to schedule early TCI state activation or PDCCH order based RACH.

- Event LTM3: As the name suggests this event is similar to event A3 for the event triggered L3 measurements. When beam of candidate cell becomes amount of offset better than beam of serving cell, NW can trigger the cell switch based on the event triggered report from the UE.

- Event LTM4: As the name suggests this event is similar to event A4 for the event triggered L3 measurements. When beam of candidate cell becomes better than absolute threshold, based on the UE report, NW may activate the TCI states of the candidate cell or update the active TCI state list.

- Event LTM5: As the name suggests this event is similar to event A5 for the event triggered L3 measurements. When Beam of serving cell becomes worse than absolute threshold1 AND beam of candidate cell becomes better than another absolute threshold2, based on the UE report, NW may activate the TCI states of the candidate cell or update the active TCI state list or perform LTM cell switch.

Based on the above analysis we can observe that RAN2 agreed events are designed to request a candidate cell L1 measurement report to perform early DL/UL synchronization on candidate cells or trigger an LTM cell switch.

TCI state known condition

The network may configure the UE with both L3-based events and L1-based events simultaneously. For example, the network may trigger an LTM cell switch based on Event LTM3 and schedule early TCI state activation and PDCCH order-based RACH based on L3-based events. In this case, the current TCI state known conditions need to be updated if the early TCI state activation is based on an L3 measurement report and the cell switch is based on an event-triggered L1 report. This is because the existing known conditions (at least for TCI state activation) are only based on the periodic L1-RSRP report

Rel-18 candidate cell TCI state activation delay requirements are applicable to known TCI states and for unknown TCI states under certain conditions. Rel-18 TCI state known conditions and the conditions (clause 8.25 of TS 38.133) under which TCI state activation requirements are applicable (clause 6.3.1 of TS 38.133) are mentioned in the below table.

Known conditions of the TCI state: The candidate TCI state in the TCI state activation list is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state activation, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state - The MAC CE command indicating the activation of downlink TCI state is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. - The target downlink TCI state remains detectable during the downlink TCI state activation period - The SSB associated with the downlink TCI state remain detectable during the downlink TCI activation period - SNR of the downlink TCI state ≥ -3dB Otherwise, the candidate TCI state is unknown. TCI state activation requirements are applicable for: Known TCI state in FR1 and FR2 specifed in clause 8.25.2, and - Unknown TCI state in FR1 specifed in clause 8.25.2, provided the following conditions are met: - UE has reported L3 measurement result with the associated SSB index of the TCI state within 5 seconds before the LTM TCI state activation command. - SNR of the associated SSB is above -3dB.

We may need to consider a scenario where event triggered reporting is configured but the event is not triggered. In this scenario, though UE was measuring the RS, since the report was not sent, the TCI state is still considered as unknown based on the existing known condition definition. The cell switch delay requirements are not applicable in this case, as per the Release 18 agreements.

To avoid no requirement in this scenario, TCI state known condition defintion needs to be modified

- RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- Measurement period of event triggered measurement reporting shall be no larger than the LTM L1-RSRP measurement period with TReport= 0. Where L1-RSRP measurement period corresponds to the cell that triggers the event triggered reporting.

- Measurement period of the L1-RSRP can be revisited based on RAN1 conclusion on filtering.

- RAN4 to not consider TTT (time to trigger) into account in the L1 event-triggered reporting delay requirements.

- RAN4 to agree that existing Ecat capability is not shared between L3 events and LTM L1 events.

- RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells.

- RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS.

- RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting.

# References

[1] 	RP-241515, New WID: NR mobility enhancements Phase 4

[2] 	R4-2420101, WF on NR mobility enhancements Phase 4