3GPP TSG RAN WG4 Meeting #114	R4-2502065

Athens, Greece, 17 Feb. 2025 – 21 Feb. 2025

Agenda Item:	7.27.2.2

Source: 	Ericsson

Title:	Discussion on CSI-RS based L1 measurement requirements

Document for:	Discussion

# Introduction

In this contribution we discuss CSI-RS based L1 measurement requirements.

# Discussion

## CSI-RS based L1 measurement requirements

### Known cell conditions

For CSI-RS based L1-RSRP measurement, known cell conditions were discussed in last meeting and the following WF is agreed.

- Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition

There are a couple of open issues in the above agreement.

When the same SSB is configured for both L3 and L1 measurements, the detectability conditions are as follows:

- For L3 measurements, the SSB_RP must be at least -6dB.

- For L1 measurements, the SSB_RP must be at least -3dB.

Given that the SSB is used for timing and QCL relations, we believe it is reasonable to assume that the L3 detectability condition should suffice. This is because the coarse timing derived from the SSB can be considered adequate for CSI-RS reception. Moreover, general assumption in RAN4 is that SSB L1 measurements are needed for fine timing acquisition.

We believe "detectable" is used for RS that can be detected without any assistance information from the NW. Since CSI-RS requires SSB as a QCL relation, it cannot be detected on its own. However, there are instances in the specs where "measurable" is also used for SSB. Therefore, we could use either "measurable" or "detectable" and define what each term means. We consider CSI-RS to be measurable if the side condition for the SNR is satisfied, and the SNR side condition (-3dB) for serving cell CSI-RS measurements can be reused.

We think CSI-RS based L3 measurements needs SSB as a pre-requisite. Hence, we suggest considering SSB-based L3 measurement only.

- For CSI-RS based L1-RSRP measurements on neighbour cells, an LTM candidate cell is known if the following conditions are met

  - The UE has performed SSB based L3 measurement on the target cell during the last 5 seconds, and

  - The SSB QCL’ed with the CSI-RS remains detectable (Es/Io ≥ -6 dB) according to the cell identification requirements in clause 9.2.

  - The CSI-RS from the target cell configured for L1 measurement remains measurable.

    - CSI-RS is considered measurable if the CSI_RS Es/Io is ≥ -3dB.

### Intra-frequency and Inter-frequency definition

As per the RAN1 agreement, since the frequency location of the CSI-RS can be in any location, RAN4 should discuss how to categorize the measurements (e.g., intra-frequency and inter-frequency definitions). The following options were discussed in the last meeting:

- Agreement: - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of [active DL BWP], and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of [active DL BWP], and - It is applied for SCS = 60KHz - [the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).]

There are a couple of open issues in the above agreement.

Active DL BWP: we think SCS and CP of the active BWP can be used in the intra-frequency defintion.

Any CSI-RS resource of serving cell

The third bullet was motivated by the CSI-RS based L3 measurement intra-frequency definition. One disadvantage we see is that the NW always needs to configure serving cell CSI-RS based L1 measurements for the comparison of intra-frequency definition, even when the NW may not use CSI-RS for L1-RSRP in the serving cell (meaning the NW does not use fine beams in the serving cell but may plan to use them in candidate cells).

Some companies have concern that if the same CSI-RS resource is configured for both L3 and L1 measurements, there can be a scenario where they are classified as different types of measurements. Although CSI-RS based L3 measurements are not widely deployed, this might confuse spec readers. However, to avoid limiting the NW configuration to only use CSI-RS for L1-RSRP to define intra-frequency, we suggest using any CSI-RS resource (e.g., TRS or CSI-RS for L1-RSRP) configured for the UE in the serving cell.

- RAN4 to use ‘any of the CSI-RS resources of the serving cell instead of CSI-RS resources of the serving cell indicated for L1 measurement’ to define the CSI-RS based intra-frequency L1 measurement.

### Measurement procedure

In the RAN4#112bis meeting, two alternatives were discussed for the UE measurement behavior for CSI-RS based L1-RSRP measurement.

RAN4#112bis agreement Alternative 1: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions. FFS the details of the conditions. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1. Alternative 2: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

Further, in the RAN4#113 meeting, the two alternatives were consolidated into the following agreement:

Consolidated agreement after RAN4#113

- Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements:

  - Step 1: SSB based L3 measurements and reports at least before LTM configuration.

  - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

    - Further discuss whether Step 2 can be skipped under certain conditions or based on UE capability or based on network RRC configuration. FFS the details.

    - Note: Step 2 is removed for FR1.

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

    - Depending on whether step 2 is skipped, the requirements in step 3 may be different.

  - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

    - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

We can summarize the two alternatives discussed in RAN4#112bis as SSB based RX beam sweeping and CSI-RS based RX beam sweeping. SSB based RX beam sweeping is supported by UE from Rel-15, while CSI-RS based beam sweeping may need to be newly implemented by UE vendors. We discussed the advantages of CSI-RS based RX beam sweeping for CSI-RS based L1-RSRP measurement in the last meeting, and the details can be found in the annex for ease of reference. Considering the advantages of the CSI-RS based RX beam sweeping, we suggest defining Rx beam sweeping based on CSI-RS and SSB.

Based on the above analysis, we suggest the following proposals:

- RAN4 to define CSI-RS based RX beam sweeping as an optional UE capability for CSI-RS based L1-RSRP measurements.

- For a UE supporting CSI-RS based RX beam sweeping, RAN4 to agree on the following measurement procedure for CSI-RS based LTM candidate cell measurements:

  - Step 1: SSB based L3 measurements and reports at least before LTM configuration

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

- For a UE not supporting CSI-RS based RX beam sweeping, RAN4 to agree on the following measurement procedure for CSI-RS based LTM candidate cell measurements:

- Step 1: SSB based L3 measurements and reports at least before LTM configuration.

- Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

  - Note: Step 2 is removed for FR1.

- Step 3: CSI-RS based L1 measurements on the same candidate cell

- Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

- Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

### Defintion of the frequency layer

If two CSI-RS resources belong to the same frequency layer, the UE is expected to measure them at the same time using the same FFT. As per our understanding, the UE can measure the two CSI-RS resources simultaneously using the same FFT if their SCS, CP type, BW, and center frequency are the same, and they have overlapping periodicity (i.e., they are fully or partially overlapping in the time domain).

- Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and the CSI-RS occasions are fully/partially overlapping in time domain.

### Supported measurement types

When a measurement is within the active BWP, it can be assumed to be performed without a measurement gap. If the measurement is outside the active BWP, it can be assumed to be performed with a measurement gap. Since CSI-RS is used for L1-RSRP of candidate cells for mobility purposes, RTD > CP should be considered. In short, measurements inside BWP, outside BWP, RTD > CP, and RTD < CP should all be considered.

- CSI-RS based L1 measurements for candidate cells shall support

- Measurement within active BWP, Measurements outside active BWP

- RTD < CP, and RTD > CP

### Measurement gap

When only a single gap configuration is supported, the UE should share the measurement gap (MG) between the SSB and CSI-RS. Sharing the measurement gap with the SSB may further extend the measurement delay of each measurement objects on the RRM measurements. If the CSI-RS does not overlap with the SMTC, the UE may perform both the CSI-RS based and SSB based measurements in different gap occasions.

- RAN4 to define the gap-based CSI-RS L1-RSRP measurement which do not impact the legacy SSB based L1-/L3-RSRP measurement within gap.

In Rel-17, concurrent gaps were introduced which may be useful for CSI-RS based L1-RSRP measurement; however, it may bring additional workloads in RAN4. We think RAN4 can further discuss which type of measurement gap can be used for CSI-RS based L1-RSRP once all the essential issues for defining CSI-RS based L1-RSRP requirements are settled.

- RAN4 to define single gap-based CSI-RS L1-RSRP requirement first. RAN4 can further discuss concurrent gaps-based CSI-RS L1-RSRP requirement if time allows.

### Measurement on CSI-RS with repetition ON/OFF

Currently, RAN1 is discussing whether CSI-RS with repetition ON should be supported for L1-RSRP measurements on candidate cells or not. Since RAN1 is still discussing the implications of supporting CSI-RS with repetition ON for L1-RSRP measurements, we suggest RAN4 to wait for the outcome of these discussions.

- RAN4 to wait for RAN1 conclusion on support of CSI-RS with repetition ON.

### CSI-RS resource type

In the last meeting, the resource types for CSI-RS based L1-RSRP were discussed. Currently, RAN1 has agreed to support both periodic and semi-persistent reporting. RAN4 requirements are supported for periodic reporting anyway. There was a discussion in the last meeting on whether to support the requirements applicability for semi-persistent CSI-RS. Since the RAN4 core requirements should be accompanied with performance requirements to make sure the performance of the core requirements, RAN4 requirements are defined for the features that are widely used. This procedure is followed mainly to reduce the test cost for the UE.

We think the semi-persistent reporting for L1-RSRP can be widely used in the NWs as it brings the flexibility of enabling the reporting when certain events are met (This can be used in conjunction with other L3/L1 events to enable or disable the measurement reporting) and disabling it for saving the UL resource overhead. Hence, it should be supported by RAN4 requirements for guaranteeing the measurement performance.

- Semi-persistent CSI-RS should be considered for defining CSI-RS based L1-RSRP requirements

### Restriction on CSI-RS resource configuration

In the last meeting, restriction on CSI-RS resource configuration was discussed and following WF was agreed. We think option 1 needs main consideration as part of this section and the other options were considered as part of other issues discussion.

<Candidate solutions> - Option 1: It is proposed to introduce a CSI-RS based L1 measurement timing configuration. (Apple) - Option 2: RAN4 requirements apply only when all CSI-RS resources on candidate cells on the same layer are configured with the SCS and CP type. (Huawei, Qualcomm) - Option 3: Only the CSI-RS resources with the same center frequency, SCS and CP type can be measured within one measurement gap occasion. (Huawei) - Option 4: The CSI-RSs of the target cells are considered as the same frequency layer if the SSBs of these target cells are of the same SSB frequency, SCS and CP type. (vivo)

We copy the snippet of the CSI-RS based L3-RSRP requirements applicability to provide further analysis on the option 1.

Intra-frequency requirement applicability:

Inter- frequency requirement applicability:

RAN1 has agreed to use CSI-RS for BM/L1-RSRP as the CSI-RS type for CSI-RS based L1-RSRP measurements. In contrast, CSI-RS based L3-RSRP uses the CSI-RS type, CSI-RS-Resource-Mobility. The periodicity and offset of CSI-RS-Resource-Mobility are {4ms, 5ms, 10ms, 20ms, 40ms}, whereas for CSI-RS for L1-RSRP, they are {4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640} slots.

Since the CSI-RS for L1-RSRP is designed to be quite flexible for network configuration, unless RAN1/2 introduces some limitations on the periodicity, we believe RAN4 should not impose a general limitation on the periodicity and slot offset across all configurable options. Though CSI-RS for L1-RSRP/BM is configured as UE specific reference signal, NW may transmit the same CSI-RS resource for different UEs in the serving cell and the same CSI-RS resource may be configured as LTM candidate resource for measurements for the LTM cell switch. This essentially requires coordination among different schedulers of the different LTM candidate cells to be within a specified window. We think this scheduling coordination is difficult to achieve in practice and we suggest not introducing them.

- RAN4 not to define L1 measurement timing configuration for CSI-RS based L1-RSRP measurements for LTM.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- For CSI-RS based L1-RSRP measurements on neighbour cells, an LTM candidate cell is known if the following conditions are met

  - The UE has performed SSB based L3 measurement on the target cell during the last 5 seconds, and

  - The SSB QCL’ed with the CSI-RS remains detectable (Es/Io ≥ -6 dB) according to the cell identification requirements in clause 9.2.

  - The CSI-RS from the target cell configured for L1 measurement remains measurable.

    - CSI-RS is considered measurable if the CSI_RS Es/Io is ≥ -3dB.

- RAN4 to use ‘any of the CSI-RS resources of the serving cell instead of CSI-RS resources of the serving cell indicated for L1 measurement’ to define the CSI-RS based intra-frequency L1 measurement.

- RAN4 to define CSI-RS based RX beam sweeping as an optional UE capability for CSI-RS based L1-RSRP measurements.

- For a UE supporting CSI-RS based RX beam sweeping, RAN4 to agree on the following measurement procedure for CSI-RS based LTM candidate cell measurements:

  - Step 1: SSB based L3 measurements and reports at least before LTM configuration

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

- For a UE not supporting CSI-RS based RX beam sweeping, RAN4 to agree on the following measurement procedure for CSI-RS based LTM candidate cell measurements:

- Step 1: SSB based L3 measurements and reports at least before LTM configuration.

- Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

  - Note: Step 2 is removed for FR1.

- Step 3: CSI-RS based L1 measurements on the same candidate cell

- Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

- Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

- Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and the CSI-RS occasions are fully/partially overlapping in time domain.

- CSI-RS based L1 measurements for candidate cells shall support

- Measurement within active BWP, Measurements outside active BWP

- RTD < CP, and RTD > CP

- RAN4 to define the gap-based CSI-RS L1-RSRP measurement which do not impact the legacy SSB based L1-/L3-RSRP measurement within gap.

- RAN4 to define single gap-based CSI-RS L1-RSRP requirement first. RAN4 can further discuss concurrent gaps-based CSI-RS L1-RSRP requirement if time allows.

- RAN4 to wait for RAN1 conclusion on support of CSI-RS with repetition ON.

- Semi-persistent CSI-RS should be considered for defining CSI-RS based L1-RSRP requirements.

- RAN4 not to define L1 measurement timing configuration for CSI-RS based L1-RSRP measurements for LTM.

# References

[1] 	RP-241515, New WID: NR mobility enhancements Phase 4

[2] 	R4-2414060, WF on NR mobility enhancements Phase 4

# Annex

Figure 1: Measurement procedure using Alternative 1

Alternative 1:

Step 1: UE performs SSB based L3 measurements with RX beam sweeping in FR2 and determine the RX beam for L3 measurements.

Step 2: If the same SSB is configured for L1 measurements, UE performs RX beam sweeping to find the best Rx beam for the SSB based L1-RSRP measurements. In this step UE may find a finer RX beam for receiving the SSB. This step is useful for finding the best Tx and Rx beam combination for SSB based L1-RSRP measurement. Due to the Rx beam gain and able to receive the SSB in Tx beam peak direction, UE may get higher L1-RSRP compared to L3-RSRP. In this step UE determines the best RX beam for a given SSB and uses the same for the receiving CSI-RS which was QCLed with SSB. In the figure 1, red solid beam may be the best Rx beam.

Step 3: the Rx beam determined in step 2 may be used to receive the CSI-RS which was configured with same SSB as QCL source RS. In the figure 1, UE uses the red solid beam to receive all the three CSI-RS (as three CSI-RS has same SSB as QCL source).

Why Alternative 1 may not work well: Based on the Rx beam width selected by UE, it may happen that the RX beam may be good for receiving one CSI-RS but may not be good for receiving the other CSI-RS. In the figure 1, the red RX beam (best Rx beam for SSB based L1-RSRP) may be good for receiving the green CSI-RS beam but may be not so good for receiving other two CSI-RS. We think wide beam used for SSB based L3 measurement may work well for receiving three CSI-RS resources than the SSB based fine beam for L1 measurement.

Figure 2: Measurement procedure using alternative 2

Alternative 2:

In this approach, UE performs the RX beam sweeping using the periodic or semi-persistent CSI-RS. In step 1, UE performs SSB based L3 measurements and determine the wide beam to be used for SSB based L3 measurements. In step 2, instead of doing Rx beam sweeping on SSB to determine the fine beam, UE uses the periodic or semi-persistent CSI-RS for the Rx beam sweeping. We show the periodic CSI-RS based Rx beam sweeping in figure 3.

Figure 3: Rx beam sweeping using CSI-RS

From the figure 3, we can observe that UE can get the best Tx and Rx beam pair if the UE has performed the RX beam sweeping using CSI-RS. As shown in figure 3, orange CSI-RS beam can be best received with black Rx beam, green CSI-RS beam can be best received with green Rx beam, and blue CSI-RS beam can be received with red Rx beam.

If a UE supports early CSI acquisition and reporting, best Tx and Rx beam pair can be useful in determining the CQI more accurately. When NW schedules data in the candidate cell using fine beam UE will be able to support better throughput using alternative 2.