3GPP TSG-RAN4 Meeting #114	R4-2502130

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | 0692 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_RAIL_EU_900MHz-Core, NR_RAIL_EU_1900MHz_TDD-Core) NB-IoT operation modes clarification for FRMCS BS in multi-carrier operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RAIL_EU_900MHz-Core, NR_RAIL_EU_1900MHz_TDD-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It was observed, that additional BS output power requirements for FRMCS BS are referring to the “standalone NB-IoT” operation, which is not supported by NR BS specification TS 38.104. In this CR, we provide clarification for FRMCS multi-carrier operation, to avoid ambiguity on the supported NB-IoT operation modes for FRMCS BS operating in bands n100/n101. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of a clarification note for FRMCS multi-carrier operation, to avoid ambiguity on the supported NB-IoT operation modes for FRMCS BS operating in bands n100/n101. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification wording on the supported NB-IoT operation modes for FRMCS BS operating in bands n100/n101 would remain ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.141-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

### 6.2.4	Additional requirements (regional)

In certain regions, additional regional requirements may apply.

For Band n41 and n90 operation in Japan, the rated output power, Prated,c.sys for BS type 1-H or sum of Prated,c,AC over all antenna connectors for BS type 1-C declared by the manufacturer shall be equal to or less than 20 W per 10 MHz bandwidth.

For band n100 in CEPT countries subject to the ECC Decision (20)02 [21], Prated,c,AC of the WA BS shall not exceed 51.5 dBm/5MHz + (fDL-922.1) x 40/3 dB, with fDL being the centre frequency in MHz. This limit is derived from ECC Decision (20)02 [21] assuming a 17 dBi maximum antenna gain and 4 dB losses, and assuming one antenna connector. The above rated output power limit for band n100 applies to uncoordinated deployments and in case of coordinated deployments, higher output power values may be allowed.

For CEPT countries subject to the ECC Decision (20)02 [21], administrations wishing to allow multiple carriers, i.e., more than one wideband carrier (LTE, NR, NB-IoT standalone, or NR/LTE with NB-IoT in-band) in band n100, should consider the implementation of a coordination procedure or other mitigation measures.

NOTE 1:	For more details on the maximum level derivation, refer to TR 38.853 [23].

NOTE 2:	The additional maximum output power requirement for 3 MHz channel bandwidth will be updated in a later version, based on CEPT WG FM’s feedback.

NOTE 3:	Standalone NB-IoT operation is not covered by the present release of this specification.

For band n101 in CEPT countries subject to the ECC Decision (20)02 [21], Prated,c,AC shall not exceed 51 dBm/10MHz or 48 dBm/5MHz. This limit is derived from ECC Decision (20)02 [21] assuming a 18 dBi maximum antenna gain and 4 dB losses, and assuming one antenna connector. The above rated output power limit for band n101 applies to uncoordinated deployments and in case of coordinated deployments, higher output power values may be allowed.

For CEPT countries subject to the ECC Decision (20)02 [21], administrations wishing to allow multiple carriers, i.e., more than one wideband carrier (LTE, NR, NB-IoT standalone, or NR/LTE with NB-IoT in-band) in band n101, should consider the implementation of a coordination procedure or other mitigation measures.

NOTE 4: 	For more details on the maximum level derivation, refer to TR 38.852 [22].

NOTE 5:	Standalone NB-IoT operation is not covered by the present release of this specification.

------------------------------ End of modified section -------------------------