TSG-RAN Working Group 4 (Radio) meeting #114	R4-2502160

Athens, EU, 17th to 21st February 2025

Source:	Ericsson

Title:	Simulation of BS-to-BS isolation for edge-to-edge co-location isolation at 7 GHz

Agenda item:	7.13.3

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#113 in Orlando) simulation results for the coupling loss between two sideways mounted Urban Macro AAS BSs at 3.5 GHz were reported [1].

The BS-to-BS isolation is used as a background for BS RF requirements defined to guarantee co-location of two base station operating in different bands at the same site. In the current specification, two co-location requirements are defined: transmitter spurious emission requirement and receiver out-of-band blocking. In addition, TDD OFF power and Transmitter intermodulation is also relying on BS-to-BS isolation and corresponding test method used for co-location requirements. It is therefore essential, to evaluate relevance of the traditional assumptions for co-location requirements.

With the correct assumption on BS-to-BS coupling, the test concept can be evolved such that the need for a specific co-location reference antenna can be avoided as indicated in [2]. Considering the work to be started for transmitter spurious emissions and BS RF specification improvements [3] it would be reasonable to also look into the details regarding the technical background for co-location requirements. In this contribution we present preliminary simulation results relevant for 7 GHz and band n104.

# Discussion

For higher bands such as the band n104, a working assumption is that the physical antenna area shall be the same as in the 3.5 GHz case to maintain a decent link budget and coverage. Moreover, to maintain the spatial performance of the beams, the antenna element separations must also be kept the same in relation to the frequency (wavelength) used. This leads to much larger array sizes. Some preliminary observations, of colocation coupling loss in such a scenario, are presented.

## Simulation assumptions

The assumption to have a fixed physical size compared to the 3.5 GHz case results in a large numerical problem. In order to have a reasonable total simulation time and effort, the element model is simplified. Here, a pair of crossed dipoles over, but not connected to, a ground plane is used.

The simulation assumptions are provided in Table 2.1-1 and Table 2.1-2.

Table 2.1-1: Network assumptions

| Carrier Frequency | 6775 MHz |
| --- | --- |
| BS height | 25 m |
| UE height | 1.5 m |
| Minimum 2D distance BS to UE | 35 m |
| Number of UEs per sector | 1500 |
| ISD | 0.5 km |
| Network layout | Hexagon |

Table 2.1-2: BS antenna assumptions

| Parameter | Assumption |
| --- | --- |
| Sub-array configuration | 3x1 |
| Array configuration | 8x16 |
| Pre-set sub-array down-tilt | 3 degrees |
| Mechanical down-tilt | 3 degrees |
| Vertical element separation | 0.7λ |
| Horizontal element separation | 0.5λ |
| Design frequency | 7 GHz (λ=42.8275 mm) |
| Evaluation frequencies | 6425, 6775 and 7125 MHz |
| Relative bandwidth | 10% |

The placement of the victim and aggressor base stations and the individual base station array antenna geometry is visualised in Figure 2.2-2. Compared to previously presented result it should be noted that the relative bandwidth is 10% here, compared to about 22% in [1].

Moreover, the simulation methodology from [1] is reused, where the element to element scattering matrix is computed using a commercial full wave electromagnetic tool.

### Results

Two systems serving the same cell are considered and the AASs are mounted side by side in the same site, see Table 2.2-1 and below. The AAS array geometry and relative placement are shown in Fig. 2.2-1. The separation between the arrays is 10 cm, and hence the chassis of the AASs are neglected. Hence, for a real product the coupling loss can be higher.

Figure 2.2-1: Deployment of the two AASs at band n104 separated 10 cm edge to edge, see Table 2.1-1.

### Array to array coupling

The scenario of the array to array (A-to-A) coupling is depicted in Fig. 2.2.1-1. 1500 random users per system are used, and at each time instant a pair of randomly selected users are considered for the calculation of A-to-A coupling loss.

Figure 1.2.1-1: User positions for the A-to-A coupling loss case

The empirical CDF of the A-to-A coupling loss is depicted below. Compared to the 3.5 GHz results (55±20 dB coupling loss) shown in [1] the coupling loss between the beam ports increase. At 50% percentile the coupling loss is larger than 75 dB and the beamforming spread is around 50 dB, see Fig. 2.2.1-2.

Figure 2.2.1-2: A-to-A coupling loss

The A-to-A coupling loss at band n104 is significantly higher than the legacy 30 dB value.

The A-to-A coupling loss is significantly higher in band n104 compared to the 3.5 GHz case.

For each beam direction of the left (or right) array, the coupling loss depends on frequency. In Fig. 2.2.1-3 the minimum coupling loss over frequency is depicted versus beam pointing direction and for the two arrays.

Figure 2.2.1-3: Minimum A-to-A coupling loss versus beam direction

## Array to subarray coupling

In the Array to subarray (A-to-SA) coupling case, beams are directed to the users of the left system as depicted below. The coupling loss is then calculated from the corresponding beam port (including beamforming weights applied to all sub-arrays of the left array which have a fixed subarray down tilt) to all the subarray ports of the right system.

Figure 2.2.2-1: User positions of the left system.

Figure 2.2.2-2: A-to-SA coupling loss

Compared to the 3.5 GHz results shown in R4-2418398 (55±15 dB coupling loss) the coupling loss is around 70±20 dB, i.e., 15 dB higher. The antenna element used here is simplified compared to the 3.5 GHz case. The larger spread in A-to-SA comes from the spread of coupling loss over the subarray positions, see below.

The A-to-SA coupling loss is significantly higher in band n104 compared to the 3.5 GHz case.

A-to-A and A-to-SA coupling losses are different.

Minimum coupling loss for A-to-SA case is depicted in Fig. 2.2.2-3. For the beams of the left array pointing to the users depicted in Fig. 2.2.2-1, and for the subarrays of the right array. For the beam directions of the left array, the minimum coupling loss over frequency and subarrays of the right array, is used. For the subarrays of the right array, the minimum coupling loss over frequency and beam directions of the left array, are presented.

Figure 2.2.2-3: Minimum A-to-SA coupling loss

The non-uniform coupling loss with respect to subarray position in the Array-to-subarray will impact requirements.

Based on the discussion in the previous sections we propose the following:

Co-location of two systems at different bands should be studied.

Co-location of systems with different array size should be studied, e.g. Urban Macro and Urban micro.

Study the impact on requirements of non-uniform coupling loss with respect to subarray position in the array-to-subarray case.

# Conclusion

The information related to the coupling loss characteristics is needed when requirement levels for transmitter co-location spurious emission requirement levels and receiver co-location out-of-band blocking requirement levels are derived. The information presented in this contribution can be used as input for later re-evaluation of co-location requirement levels for bands in the upper region of FR1.

Co-location coupling loss calculations for a 10 cm side-by-side Urban macro scenario have been presented. The following observations have been made.

Observation 1	The A-to-A coupling loss at band n104 is significantly higher than the legacy 30 dB value.

Observation 2	The A-to-A coupling loss is significantly higher in band n104 compared to the 3.5 GHz case.

Observation 3	The A-to-SA coupling loss is significantly higher in band n104 compared to the 3.5 GHz case.

Observation 4	A-to-A and A-to-SA coupling losses are different.

Observation 5	The non-uniform coupling loss with respect to subarray position in the Array-to-subarray will impact requirements.

Based on these observations the following proposals are made:

Proposal 1	Co-location of two systems at different bands should be studied.

Proposal 2	Co-location of systems with different array size should be studied, e.g. Urban Macro and Urban micro.

Proposal 3	Study the impact on requirements of non-uniform coupling loss with respect to subarray position in the array-to-subarray case.

# References

[1]	R4-2418398, “Simulation of BS-to-BS isolation for edge-to-edge co-location scenario”, Ericsson

[2]	R4-2415971, “On the topic of aspects related to BS-to-BS co-location”, Ericsson

[3]	RP-243288, “Moderator's summary of discussion about R19-RAN4-BSRF”, Huawei