3GPP TSG-RAN4 Meeting #114	R4-2502169

# Athens Greece, 17th – 21nd February, 2025

Source: 	Qualcomm Incorporated

Title: 	CSI Compression: Testability and Interoperability

Agenda item:	7.20.3

Document for:	Discussion

# Introduction

In this paper, we present two types of simulation results:

- Results with Track 1 based dataset where one company’s encoder is tried with another company’s decoder

- Results with track 2 based mixed dataset where same company’s encoder and decoder are trained together.

We also provide our views regarding fully and partially specified decoder options.

# Past Agreements

## Model of Encoder-Decoder Pair

Following agreements occurred during the email discussion after RAN4 111 [1].

Based on the two rounds of discussions, the following parameters are proposed: Fig 1. Detailed model diagrams of encoder. Fig 2. Detailed model diagrams of decoder. Table 1. CNN model parameters for encoder. Model architecture parameters for encoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 47M (vs. 1.6M for eTypeII). Model size; 448Kbyte. Num of Resnet blocks; 6. Layer after resnet blocks; 2 layers (one FC layer, one Conv layer). Layer before resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,32). Size of message (feature map) passed across layers; (13,32). Table 2. CNN model parameters for decoder. Model architecture parameters for decoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 740M. Model size; 7.2Mbyte. Num of Resnet blocks; 6. Layer before resnet blocks; 2 layers (one FC layer, one Conv layer). Layer after resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,128). Size of message (feature map) passed across layers; (13,32). Other model related parameters: Activation function; ReLU. Latent message size; 32 float values. Payload size, bits; 64 per layer.. Quantization; 2 bits. System parameters: Frequency range; 2GHz (4GHz optional). MIMO Layers; 1 layer (2 layers optional, companies to. Sub-bands; 13. Channel estimation; Ideal. Metrics; SGCS of AI CSI feedback and eTypeII CSI feedback, both relative to ideal CSI.. Companies are also invited to provide the training parameters used.

Following agreements happened a couple of meeting ago [2].

Parameter/Configurations; Configuration for alignment. Channel/data-set related. UE antenna element assumptions; Ideal omni-directional antenna elements following RAN1 assumptions. Sub-band channel averaging; - Average of covariance channel matrices per PRBs in the sub-band - 1 sample per PRB is selected. Any other pre-processing; - Apply phase normalization: - Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - i.e. the phase of the first value of the eigenvector is 0. - L2 norm of the eigenvector is equal to 1.. Data collection assumptions; - Strive to ensure that the samples are decorrelated, especially when collected from the same UEs.. Training dataset size; - Aim to use the dataset around 600K. - At least 100K sample to be used.. Dataset quantization; - Float32. Other data-set characteristics/statistics; - Companies to propose. - Companies are encouraged to report some statistics about the dataset. - Statistics are FFS - Companies are encouraged to report the CDF of the SGCS across sub-bands and input samples. - If possible, also report CDF for eTypeII.. Model-related. Quantization of encoder output; - Report results without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report - Companies still need to report quantization results: - Companies to report whether/how quantization aware training is implemented (e.g., how quantization is treated at back propagation) - Companies can try quantization aware training with bypassing of the quantization in back propagation - Quantization options, companies to provide the details: - Fixed unform quantization with equal ranges - Trainable quantization. Normalization before quantization in the encoder; - For the results with quantization, to use a sigmoid function in the encoder for normalization - Sigmoid in the decoder is optional.. Use and configuration of drop-out; - Not to use drop-out layers. Training loss function, use of regularization; - Use (1-SGCS) as loss function as reference. Optimizer; - Use Adam. Learning rate. - Fixed learning rate of 10^-4. Batch size. - 256. (Max) Number of epochs. - 150 epochs. Stopping criteria. - No improvement over 25 epochs.. Weight initialization. - Default. Kernel Parameters: - Dilation - Padding Type The value of groups (VG); - Default. Additional data to be submitted: - CDF of SGCS per subband - CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format)

RAN4 agreed to the following during the last meeting [3]:

Issue 4-1: Simulation results discussion for Option 3 … Additional data to be submitted: - CDF of SGCS per subband - CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format)

## System Parameters

The system parameter assumptions of 38.843 were considered to be baseline for RAN4 performance evaluation.

Table 6.2.1-1: Baseline System Level Simulation assumptions for AI/ML based CSI feedback enhancement evaluations

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g., UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, 2GHz as baseline, optional for 4GHz (if R16 as baseline) FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz (if R17 as baseline) |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz (if R16 as baseline) 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz (if R17 as baseline) |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU. |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g., 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | At least, FTP model 1 with packet size 0.5 Mbytes is assumed. Other options are not precluded |
| Traffic load (Resource utilization) |  | 20/50/70%. Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) CSI prediction: 100% outdoor (10, 20, 30, 60, 120 km/h) including outdoor-to-indoor car penetration loss per TR 38.901 if the simulation assumes UEs inside vehicles. No explicit trajectory modeling considered for evaluations. |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline. Up to companies to choose the error modelling method for realistic channel estimation. Ideal DL channel estimation is optionally taken into the baseline of evaluation methodology for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Up to companies to report whether/how ideal channel is used in the dataset construction and performance evaluation/inference. Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation. |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. The CSI feedback overhead is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE. - For AI/ML based solutions: The above-mentioned "CSI feedback overhead" is calculated as max allowed bits at the given rank. - For legacy Type II CB: Option 2b is mandatorily reported by companies, while Option 2a can be optionally reported up to companies if partial NZC report is assumed for the legacy Type II CB - Option 2a: The above-mentioned "CSI feedback overhead" is calculated as each CSI reported payload with a given rank - Option 2b: The above-mentioned "CSI feedback overhead" is calculated as max allowed bits at the given rank Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | For CSI compression: Companies need to report which option is used between: - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. Additional assumptions from R17 TypeII EVM: Same consideration with respect to utilizing angle-delay reciprocity should be considered for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline. Optionally, Type I Codebook (if it outperforms Type II Codebook) can be considered for comparing AI/ML schemes. For CSI-prediction: Both of the following are taken as baseline: - The nearest historical CSI without prediction - Non-AI/ML or AI/ML with collaboration Level x based CSI prediction for which corresponding details would need to be reported Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example. For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on CSI without compression (e.g., eigenvector from measured channel), which is taken as an upper bound for performance comparison. |
| Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions. The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations. |  |  |

## Different Options for Fully and Partially Specified Encoeder-Decoder Pair

RAN4 agreed to the following recently [4].

Issue 4-1: Option 3 (fully specified decoder) next steps - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused

Issue 1-2: Option 3 next step - decoder(s) selection choose the decoders with Low, medium, high SGCS Agreement: among the decoders submitted by companies Continue the feasibility study on the two tracks below: Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset) - The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s). - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

RAN4 made the following agreement regarding reference encoder decoder pair a couple of meeting ago [4]

Issue 4-4: Options for Option 4 Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

Issue 1-3: Option 4a(Dataset based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based option 3 - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1 - Or mixed dataset from track 2 - Step 5: Label selected dataset(s) (mapping of encoder input to latent message) [using encoder] corresponding to the selected test decoder(s) from option 3 - Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s) - Performance alignment to be checked/discussed - Step 7: Conclude on overall feasibility of Option 4a - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders(using different structures) Selected test decoder and matching encoder pair from Option 3 can be used to generate the common dataset (that includes latent messages) based on aggregated/mixed dataset

Issue 1-4: Option 4b (encoder based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more encoder(s) for further analysis based on option 3 - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1 - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2 - Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s) - Performance alignment to be checked/discussed - Step 6: Conclude on overall feasibility of Option 4b - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures)

## Mixed Dataset Generation Approach for Track 2

Seven companies’ uploaded datasets were used to generate the mixed dataset of above three files in the following way.

- First 100K samples from each company’s dataset were collected.

- Those 100K samples of each company were randomly reshuffled.

- Those seven sets of 100K samples were aggregated.

- The 700K samples were split into above three files.

Last 70K samples of the aggregated dataset would be used for testing of the mixed dataset. Both options are allowed regarding the split between training and testing.

- Approach 1: Split the remaining 630K samples into 560K for training and 70K for validation (i.e., 1st 560K for training, next 70K for validation and the last 70K for testing).

- Approach 2: Companies split the first 630K samples into training and validation (i.e., first 630K for training + validation and the last 70K for testing)

| File name [8] | Index within uploaded file | Source dataset | Purpose |
| --- | --- | --- | --- |
| MIXEDR4_113dsei00.npy | 0-79,999 | CATT | Training |
| MIXEDR4_113dsei00.npy | 80,000-159,999 | Ericsson | Training |
| MIXEDR4_113dsei00.npy | 160,000-239,999 | Mediatek | Training |
| MIXEDR4_113dsei00.npy | 240,000-319,999 | Nokia | Training |

| File name | Index within uploaded file | Source dataset | Purpose |
| --- | --- | --- | --- |
| MIXEDR4_113dsei01.npy | 0-79,999 | Oppo | Training |
| MIXEDR4_113dsei01.npy | 80,000-159,999 | Vivo | Training |
| MIXEDR4_113dsei01.npy | 160,000-239,999 | Qualcomm | Training |

| File name | Index within uploaded file | Source dataset | Purpose |  |
| --- | --- | --- | --- | --- |
| MIXEDR4_113dsei02.npy | 0-9,999 | CATT | Validation |  |
| MIXEDR4_113dsei02.npy | 10,000-19,999 | Ericsson | Validation |  |
| MIXEDR4_113dsei02.npy | 20,000-29,999 | Mediatek | Validation |  |
| MIXEDR4_113dsei02.npy | 30,000-39,999 | Nokia | Validation |  |
| MIXEDR4_113dsei02.npy | 40,000-49,999 | Oppo | Validation |  |
| MIXEDR4_113dsei02.npy | 50,000-59,999 | Vivo | Validation |  |
| MIXEDR4_113dsei02.npy | 60,000-69,999 | Qualcomm | Validation |  |
| MIXEDR4_113dsei02.npy | 70,000-79,999 | CATT | Testing | 0.6570 |
| MIXEDR4_113dsei02.npy | 80,000-89,999 | Ericsson | Testing | 0.6663 |
| MIXEDR4_113dsei02.npy | 90,000-99,999 | Mediatek | Testing | 0.7066 |
| MIXEDR4_113dsei02.npy | 100,000-109,999 | Nokia | Testing | 0.6905 |
| MIXEDR4_113dsei02.npy | 110,000-119,999 | Oppo | Testing | 0.7078 |
| MIXEDR4_113dsei02.npy | 120,000-129,999 | Vivo | Testing | 0.6586 |
| MIXEDR4_113dsei02.npy | 130,000-139,999 | Qualcomm | Testing | 0.7823 |

# Discussion

## Option 3 (Fully Specified Decoder)

Table 1: Results with Track 1 and joint encoder-decoder training

| Company | Tdoc Number | SGCS of CNN without quantization (based on R4-2414447) | SGCS of CNN with quantization (based on R4-2414447) | Rel-16 eTypeII CSF |
| --- | --- | --- | --- | --- |
| Oppo | R4-2417618 | 0.771 | 0.717 |  |
| CATT | R4-2415174 | 0.78 |  |  |
| Apple | R4-2418523 | 0.7604 | 0.698 | 0.685 |
| Rohde and Schwarz | R4-2416395 |  | 0.7728 |  |
| Samsung | R4-2418095 | 0.6872 | 0.6459 |  |
| Mediatek | R4-2417803 | 0.753 | 0.697 | 0.719 |
| CMCC | R4-2415251 | 0.7902 |  | 0.7 |
| Nokia | R4-2419178 | 0.755 | 0.705 | 0.691 |
| Vivo | R4-2418748 | 0.784 | 0.732 | 0.72 |
| Intel | R4-2418898 | 0.740 | 0.691 | 0.690 |
| Ericsson | R4-2416314 | 0.7584 | 0.6977 | 0.7 |
| Huawei, HiSilicon | R4-2418969 |  | 0.7 | 0.66 |
| ZTE, Sanechips | R4-2419171 | 0.7876 | 0.716 | 0.6803 |
| Qualcomm | R4-2419749 | 0.74 | 0.683 | 0.67 |
| Xiaomi | R4-2415072 |  | 0.68 | 0.676 |

Table 1 displays the results with track 1 and joint encoder-decoder training that companies reported during the last meeting.

Table 2: Results with Track 1 and QC Encoder Training

| Enc-model | Dec-model (frozen) | Train dataset | SGCS with Different Test datasets |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QC | CATT | Mixed |
| QC-Enc | Huawei-Dec with SQ | QC | 0.4962 |  |  |
| QC-Enc | Huawei-Dec with SQ | CATT |  | 0.4666 |  |
| QC-Enc | Huawei-Dec with SQ | mixed |  |  | 0.5532 |
| QC-Enc | Samsung-Dec | QC | 0.5903 |  |  |
| QC-Enc | CATT-Dec | CATT |  | 0.7769 |  |
| QC-Enc | CATT-Dec with SQ | QC | 0.37 |  |  |
| QC-Enc | CATT-Dec with SQ | mixed |  |  | 0.4647 |
| QC-Enc | QC-Dec with SQ | CATT |  | 0.3744 |  |

Note: SQ refers to scalar quantization

Table 2 shows our results with track 1 and Qualcomm encoder training. The 2nd column of Table 2 shows the three decoder models of other companies that got selected for this study. We trained our encoder with the frozen decoder with different training datasets and then tested the encoder-decoder pair with different testing datasets. The frozen decoders of different companies, mentioned in column 2 of Table 1, were trained with their individual training datasets.

Table 2 shows that track 1 with separate encoder decoder training did not perform well in most scenarios. Only, the combination of Qualcomm encoder, CATT decoder and CATT training dataset performed well. The obtained SGCS of this combination is very similar to the SGCS that CATT reported with its joint encoder-decoder training based on track 1. This happens because CATT trained their Track 1 encoder-decoder jointly with their own dataset. Later, when we train our encoder with CATT’s frozen decoder and CATT’s dataset, our encoder becomes similar to that of CATT’s encoder in track 1. In all other scenarios of Table 2, Qualcomm encoder and the frozen decoder are trained with different training datasets, and the resultant encoder-decoder pair doesn’t generate good SGCS.

Observation 1: Track 1 has not generated good SGCS performance in the scenarios where encoder and decoder are trained separately with different datasets.

- Note 1: When Qualcomm encoder was trained with CATT’s frozen decoder and CATT’s training dataset, obtained SGCS was approximately same as the SGCS obtained via track 1 and CATT’s joint encoder decoder training.

- Note 2: Further investigation is needed to see why track 1 has not produced good results when encoder and decoder get separately trained different datasets even though the datasets are generated based on the same assumptions.

Proposal 1: RAN4 requests companies whose decoders/encoders get selected for different steps of option 3 and 4 to upload their datasets, as well.

- Note 1: Other companies can use these datasets and the encoders/decoders of the selected companies for sanity check,

- Note 2: Three companies’ decoders got selected for study in Track 1. Only one among these three companies uploaded its encoder input dataset.

Table 3: Results with Track 2 and joint encoder-decoder training

| Enc-model | Dec-model | Train dataset | SGCS with Different Test datasets |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | QC | CATT | mixed | E/// | Nokia | MTK | Oppo | Vivo |
| QC-Enc | QC-Dec w SQ | joint on mixed | 0.6065 | 0.6264 | 0.6956 | 0.6613 | 0.6881 | 0.6936 | 0.7053 | 0.6967 |
| QC-Enc | QC-Dec | joint on mixed |  |  | 0.7476 |  |  |  |  |  |

Table 2 shows the SGCS results with Qualcomm’s joint encoder decoder training, mixed training datasets and different testing datasets. Please note that the test dataset of individual companies of Table 2 is not a subset of the mixed dataset. The test dataset is generated from the last 15% encoder input samples that were uploaded by individual companies.

Observation 2: Encoder-decoder pair jointly trained via track 2 approach has good generalization performance, i.e., encoder-decoder jointly trained via mixed dataset performs well with different individual companies’ test dataset.

- Above observation holds even when the test datasets of individual companies are not subsets of the mixed dataset.

Proposal 2: RAN4 selects three decoders among different companies’ uploaded ones based on lowest, median and highest SGCS with joint-encoder-decoder pair and mixed dataset in Track 2 for the next step.

## Option 4 (Partially Specified Approach)

Issue 4-4: Options for Option 4 Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

In option 4b, TE first trains its decoder based on reference encoder. UE is later expected to develop a nominal decoder that works well with the reference encoder and then train its encoder based on the nominal decoder. Later, UE’s developed encoder gets tested with TE’s developed decoder.

Observations from Track 1 results of Table 2 suggest that performance will be poor if TE and UE develop their decoder and encoder respectively with different training datasets. Use of same training dataset during encoder and decoder will guarantee good SGCS performance. Hence, training dataset (i.e., channel information) and reference encoder both need to be specified in option 4b.

If channel information and reference encoder are both specified in option 4b, then option 4b becomes a superset of option 4a where (channel information, encoder output) is specified. Hence, RAN4 doesn’t need to evaluate two approaches in option 4. RAN4 can just evaluate one approach in this option where both training dataset (channel information) and reference encoder are specified.

Eventually, a reference decoder may also need to be specified to ensure good performance in this approach. Specifying a reference decoder will allow UE to train its encoder in one step instead of the current two step process where it first develops a nominal decoder and then trains its own developed encoder based on the nominal decoder.

Observation 3: In option 4b,

- TE is expected to train its decoder based on reference encoder.

- UE is expected to develop a nominal decoder that works well with the reference encoder and then train its encoder based on the nominal decoder.

- There will be mismatch between TE and UE if the training dataset, i.e., channel information, is not specified.

Observation 4: Training dataset, i.e., channel information, and reference encoder both need to be specified in option 4b.

Observation 5: If channel information and reference encoder are specified in option 4b, then option 4b becomes a superset of option 4a where (channel information, encoder output) is specified.

Proposal 2: RAN4 specifies both training dataset (channel information) and reference encoder in option 4 and focuses only on this option for the partially specified approach.

- FFS: Specification of reference decoder in the partially specified approach.

## File Sharing for Feasibility Study

### Alignment of Uploaded Models for Feasibility Study

Several aspects of uploaded decoders from different companies were not aligned with one another. Those issues are mentioned below:

- Quantization codebook

- Use of Sigmoid Layer

- Output size

- Number of inputs

- Batch size

- Naming convention

- OPSET version

We list those issues, in details, below.

RAN4 previously agreed to use uniform scalar quantization of encoder outputs, different companies assumed different codebooks. Majority of companies assumed [1/8, 3/8, 5/8, 7/8]. However, some companies assumed a 2 bit quantization codebook that splits the range [-1, 1] uniformly. Since Sigmoid layer was agreed to be applied to encoder input, [1/8 3/8 5/8 7/8] should be used as the quantization codebook.

RAN4 agreed to use Sigmoid layer in the encoder and mentioned that Sigmoid in the decoder is optional. Companies should have a uniform understanding about whether to use Sigmoid in the decoder or not. We suggested not using Sigmoid layer in the decoder.

Companies should also unify whether quantization and dequantization operations are included in uploaded encoders and decoders. We implement quantization and dequantization operation in the wrapper as ONNX does not include. Hence, input to our decoder should be de-quantizer.

The output size of uploaded decoder of majority of companies is batch size X 2 X 13 X 32. But, output size of uploaded decoder of one company was batch size X (2 * 13 * 32). This should be unified across companies.

Majority of uploaded decoders had only one input. However, one company’s uploaded decoder had four inputs. Number of inputs to decoder should be unified across companies.

In [2], RAN4 agreed to assume a batch size of 256 for alignment. Some companies assumed a fixed batch size and some other companies assumed a flexible batch size in their encoder/decoder models. The companies that focused on a fixed batch size assumed different values of batch sizes. A company should use a batch size of 256 if it is using a fixed batch size for its encoder/decoder model.

Proposal 3: RAN4 confirms the following to align uploaded encoder/decoder models.

  - Encoder output goes through a Sigmoid function and quantized via [1/8 3/8 5/8 7/8] codebook.

  - Sigmoid is not used in the decoder.

  - Input to the decoder is de-quantizd.

  - Quantization and dequantization operations are implemented in the wrapper as ONNX does not include these.

  - Output size of decoder should be (batch size X 2 X 13 X 32). Each decoder should have only one input.

  - Companies should use batch size of 256 if they are using fixed batch sizes in their uploaded encoder/decoder models.

### Naming Convention, OPSET Version and PyTorch

RAN4 already agreed that companies can upload both ONNX and PyTorch based encoder and decoder models. Interested companies should consider uploading both ONNX and PyTorch based encoder and decoder models.

We use onnx2Torch function to convert the ONNX model of uploaded decoder to PyTorch to train with our encoder.. However, it does not yet work for OPSET version 20-23.

Observation 6: OPSET version > 19 may not be compatible with some packages (e.g., onnx2torch) that converts ONNX to PyTorch.

Proposal 4: RAN4 encourages interested companies to upload both ONNX and PyTorch formats of AI-ML models.

The naming convention of different layers of encoder/decoder of each company should be consistent among its all of uploaded models. For example, layers within each model should be ordered by index.

Observation 7: An example of consistent naming convention across layers of a decoder:

- dec.0.weight

- dec.0.bias

- dec.2.weight

- dec.2.bias

- dec.3.weight

- dec.3.bias

- dec.3.running_mean

- dec.3.running_var

- dec.3.num_batches_tracked

- dec.5.conv1.weight

- dec.5.conv1.bias

- dec.5.bn1.weight

- dec.5.bn1.bias

- dec.5.bn1.running_mean

- dec.5.bn1.running_var

- dec.5.bn1.num_batches_tracked

- dec.5.conv2.weight

- dec.5.conv2.bias

- dec.5.bn2.weight

- dec.5.bn2.bias

- dec.5.bn2.running_mean

- dec.5.bn2.running_var

- dec.5.bn2.num_batches_tracked

- dec.6.conv1.weight

- dec.6.conv1.bias

- dec.6.bn1.weight

- dec.6.bn1.bias

- dec.6.bn1.running_mean

- dec.6.bn1.running_var

- dec.6.bn1.num_batches_tracked

- dec.6.conv2.weight

- dec.6.conv2.bias

- dec.6.bn2.weight

- dec.6.bn2.bias

- dec.6.bn2.running_mean

- dec.6.bn2.running_var

- dec.6.bn2.num_batches_tracked

Proposal 5: Companies are encouraged to use informative and consistent naming convention in all their encoder/decoder models.

- Note1: Layers should be ordered by index.

- Note 2: An example of consistent naming convention for different layers of decoder is shown in observation 7.

# Conclusions

Observation 1: Track 1 has not generated good SGCS performance in the scenarios where encoder and decoder are trained separately with different datasets.

- Note 1: When Qualcomm encoder was trained with CATT’s frozen decoder and CATT’s training dataset, obtained SGCS was approximately same as the SGCS obtained via track 1 and CATT’s joint encoder decoder training.

- Note 2: Further investigation is needed to see why track 1 has not produced good results when encoder and decoder get separately trained different datasets.

Observation 2: Encoder-decoder pair jointly trained via track 2 approach has good generalization performance, i.e., encoder-decoder jointly trained via mixed dataset performs well with different individual companies’ test dataset.

- Above observation holds even when the test datasets of individual companies are not subsets of the mixed dataset.

Observation 3: In option 4b,

- TE is expected to train its decoder based on reference encoder.

- UE is expected to develop a nominal decoder that works well with the reference encoder and then train its encoder based on the nominal decoder.

- There will be mismatch between TE and UE if the training dataset, i.e., channel information, is not specified.

Observation 4: Training dataset, i.e., channel information, and reference encoder both need to be specified in option 4b.

Observation 5: If channel information and reference encoder are specified in option 4b, then option 4b becomes a superset of option 4a where (channel information, encoder output) is specified.

Observation 6: OPSET version > 19 may not be compatible with some packages (e.g., onnx2torch) that converts ONNX to PyTorch.

Observation 7: An example of consistent naming convention across layers of a decoder:

- dec.0.weight

- dec.0.bias

- dec.2.weight

- dec.2.bias

- dec.3.weight

- dec.3.bias

- dec.3.running_mean

- dec.3.running_var

- dec.3.num_batches_tracked

- dec.5.conv1.weight

- dec.5.conv1.bias

- dec.5.bn1.weight

- dec.5.bn1.bias

- dec.5.bn1.running_mean

- dec.5.bn1.running_var

- dec.5.bn1.num_batches_tracked

- dec.5.conv2.weight

- dec.5.conv2.bias

- dec.5.bn2.weight

- dec.5.bn2.bias

- dec.5.bn2.running_mean

- dec.5.bn2.running_var

- dec.5.bn2.num_batches_tracked

- dec.6.conv1.weight

- dec.6.conv1.bias

- dec.6.bn1.weight

- dec.6.bn1.bias

- dec.6.bn1.running_mean

- dec.6.bn1.running_var

- dec.6.bn1.num_batches_tracked

- dec.6.conv2.weight

- dec.6.conv2.bias

- dec.6.bn2.weight

- dec.6.bn2.bias

- dec.6.bn2.running_mean

- dec.6.bn2.running_var

- dec.6.bn2.num_batches_tracked

Proposal 1: RAN4 selects three decoders among different companies’ uploaded ones based on lowest, median and highest SGCS with joint-encoder-decoder pair and mixed dataset in Track 2 for the next step.

- Note: As previously agreed, in next step, companies are expected to submit results by training their own encoder with their individual training dataset and uploaded frozen decoder.

Proposal 2: RAN4 specifies both training dataset (channel information) and reference encoder in option 4 and focuses only on this option for the partially specified approach.

- FFS: Specification of reference decoder in the partially specified approach.

Proposal 3: RAN4 confirms the following to align uploaded encoder/decoder models.

  - Encoder output goes through a Sigmoid function and quantized via [1/8 3/8 5/8 7/8] codebook.

  - Sigmoid is not used in the decoder.

  - Input to the decoder is de-quantizd.

  - Quantization and dequantization operations are implemented in the wrapper as ONNX does not include these.

  - Output size of decoder should be (batch size X 2 X 13 X 32). Each decoder should have only one input.

  - Companies should use batch size of 256 if they are using fixed batch sizes in their uploaded encoder/decoder models.

Proposal 4: RAN4 encourages interested companies to upload both ONNX and PyTorch formats of AI-ML models.

Proposal 5: Companies are encouraged to use informative and consistent naming convention in all their encoder/decoder models.

- Note1: Layers should be ordered by index.

- Note 2: An example of consistent naming convention for different layers of decoder is shown in observation 7.

# References

[1] R4-[111][133] NR_AIML_air-email discussions after RAN4#111 meeting, https://list.etsi.org/scripts/wa.exe?A2=3GPP_TSG_RAN_WG4;aa5369d9.2406C&S=

[2] R4-2414447, “Alignment of model training and data set parameters for AI/ML based CSI compression”, Nokia,  3GPP TSG-RAN4 Meeting #112.

[3] R4-2413169, “CSI Compression: Testability and Interoperability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112.

[4] R4-2414323, “WF on AI/ML”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112

[5] R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #112bis.

[6] R4-2415889, “CSI Compression: Testability and Interoperability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112-bis

[7]https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_113/Inbox/Drafts/%5B113%5D%5B129%5D%20NR_AIML_air_part1/CSI_Compression_Results_Summary/%5B113%5D%5B129%5DAIML_Summary_of_CSI_Compression_Sim_Results_v3_Nokia2_QC2.xlsx

[8]https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/CSI_compression/Datasets/R4_113