3GPP TSG-RAN4 Meeting #114	R4-2502174

# Athens, Greece, 17th - 21st February, 2025

Agenda item:	7.20.2

Source:	Keysight Technologies UK Ltd

Title:	On Test Systems and Testability Aspects for FR2 AI/ML Beam Management

Document for:	Approval

# Introduction

This contribution addresses test system and testability considerations for AI/ML beam management use cases.

# Discussion

Not much progress has been made in terms of channel models and test systems in the last few meetings for beam management use cases [2], [3], [4]. Regarding the test setup requirements, the following agreements and actions were noted in the last WF.

- Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A

This contribution intends to provide additional views on test setup requirements and is taking into account discussions from [5], [6], [7]. These contributions also provide an extensive overview of FR2 OTA systems, CDL vs TDL channel models, etc.

# Test Setup Requirements: CDL Models

Based on our experience with developing beam management test cases, we believe a relatively simple and straightforward extension of an FR2 MIMO OTA system to 8 probes with a sample system visualized in Figure 1, is suitable to meet the following assumptions for a single TRP or two TRPs with close angular proximity:

Emulation of Tx-beam powers and RX-PAS for 4 Tx-beams (dual polarization) [6] or 8 Tx-beams (single polarization) for CDL-A, B, C, D and E UMi/UMa/InO scenarios.

  - The emulation of Tx-beams with accurate RX-PAS AoA profiles to take into account the strongest beams at the center of the test zone.

- Additionally, to emulate set B Tx beams beyond 4 [up to 16] while exercising the AI/ML algorithms, power offsets can be applied to the Tx-beams with the closest match of the 4 available RX-PAS profiles and exact power offset based on the 3D beam and CDL model characteristics

- More importantly, to emulate set A Tx beams [up to 128], power offsets can be applied to the Tx-beams with the closest match of the 4 available RX-PAS profiles and exact power offset based on the 3D beam and CDL model characteristics

- The RX-PAS AoA profile for beams emulated with power offsets cannot be guaranteed

- Support AoDs/ZoDs with a full range (0° to 360°)

- Support AoAs/ZoAs with a limited range of 110° in azimuth and 25° in elevation. for a single UE orientation

- For accurate performance assessment of all or top-K Set A beams, testing will not require exercising the AI/ML algorithms on the UE-side and different UE orientations and/or sequential testing would be required when

  - More than 4 beams need to be assessed

  - N beams (with N≤4) have beam peaks with relative AoA/ZoA differences larger than the AoA/ZoA ranges supported by the system

Figure 1: Sample FR2 test system supporting AI/ML BM testing. Probes in blue correspond to the FR2 MIMO OTA probes.

Observation 1: A relatively simple and straightforward extension of an FR2 MIMO OTA system to 8 probes is suitable to emulate 4 dual-polarized/8 single-polarized Tx-beam RX-PAS profiles, set B Tx beams up to [16] with power offsets, and set A Tx beams up to [192] with power offsets.

Proposal 1: UE/chipset/infra vendors to provide feedback on whether modelling 4 dual-polarized/8 single-polarized RX-PAS accurately with the remaining beams modelled with power offsets is sufficient

Observation 2: The extension of an FR2 MIMO OTA system to 8 probes is suitable to support unlimited range of AoDs/ZoDs and a range of AoAs/ZoAs of 110°/25°.

The minimum spacing (granularity) between typical mm-wave probes was determined to be ~3-4° [10] and the minimum probe spacing for the FR2 MIMO MPAC system [11], [12] is 10°.

Some of these testing aspects are already supported by Keysight’s S8708A Advanced Performance Test (APT) toolset [6].

Observation 3: Various beam management performance testing can readily be supported with a commercially available test system, i.e., complexity and time-to-market concerns should not be an issue.

Proposal 2: Consider an 8-probe OTA system with 2 additional probes when compared to the FR2 MIMO OTA system with a range of AoAs/ZoAs of 110°/25° as the starting point for AI/ML Beam management testing for 1 TRP and/or 2 TRPs with close angular proximity.

# On Single-Probe Systems for AI/ML OTA Testing

Recently, an alternative to multi-probe systems, e.g., Enhanced IFF and MPAC, was introduced in [13], [14], [15] to assess the beam management use cases in an alternate fashion. The suggested approaches have in common that a single-probe test system is used with the UBF applied, i.e., the UE antenna patterns are completely locked during the testing and the dynamic nature of the antenna system is turned off during the test but modelled/emulated during the test instead. Here, actual or assumed UE antenna patterns are introduced in the channel emulator and convolved with the channel model. This test approach, for UEs with static antenna systems, was previously introduced for LTE in [16] and referred to as Radiated Two-Stage (RTS) or Advanced Data Throughput Framework (ADTF) methodology and is further explained and illustrated next.

The first stage of the RTS methodology is the assessment of the UE antenna and will be covered later on in this section and the initial focus is on the second stage of the RTS methodology which focuses on the accurate channel modelling aspect this methodology is capable of.

In Figure 2, a typical single-probe IFF setup for the RTS methodology when applied to FR2 is visualized. This setup consists of the gNB (the remote radio heads/upconverters are skipped for simplicity), a channel emulator (CE) with advanced capabilities, e.g., to convolve UE antenna patterns with the channel model and the application of additional static channel matrices. Last but not least, this setup uses single-probe OTA system, which for FR2 is based on the Indirect Far Field (IFF) approach as de-facto industry standard.

Figure 2: Single-Probe Setup for RTS Methodology

The various sequences of the RTS approach are outlined next. In the very first step of the second stage of RTS, shown in Figure 3, a call between the gNB and the DUT inside the chamber is established and subsequently, the UE Beamlock Function (UBF) is applied to lock the beams selected by the UE. For these measurements, the CE is set to pass-through, i.e., no channel models and/or channel matrices other than the identity matrix are applied. This allows the transmitted signals to be received independently at each DUT receiver after the antenna system.

Figure 3: RTS Methodology: Call Established and UBF Applied

Subsequently, as illustrated in Figure 4, using a set of standardized magnitude and phase measurements, i.e., SS-RSRPB and SS-RSARP [17], the transmission matrix of the OTA environment, H, is determined based on UE reports which describes the coupling between each of the CE outputs/chamber probe’s polarization and the respective DUT’s receivers.

Figure 4: RTS Methodology: Transmission Matrix H determined from SS-RSRPB and SS-RSARP measurements

The knowledge of this transmission matrix now allows the radiated test environment including UE antennas to be compensated by applying the inverse transmission matrix, H-1, in the CE as illustrated in Figure 5. This compensation of the OTA channel by applying the inverse matrix in the CE, i.e.,

H* H-1=1001,

thus corresponds to a quasi-conducted test, in FR2 commonly referred to as wireless-cable mode [18], as the outputs of the CE almost directly maps into the corresponding receivers, just like a conducted test, with very little cross-talk which is shown schematically in the bottom of Figure 5.

Figure 5: RTS Methodology: Applying the Inverse Matrix in the CE invokes the Virtual Cable Mode

Once the wireless-cable mode is established and kept intact for the duration of the remaining testing, the knowledge of the DUT’s complex antenna patterns PDUT, i.e., magnitude and phase, can be used to apply any complex channel model to the DUT and accurately assess the device performance under faded channel conditions. As illustrated Figure 6, the convolution of the channel model and the corresponding DUT antenna patterns thus emulates the desired channel model conditions generally more accurately than a 3D MPAC OTA system which is typically used for FR1 and FR2 MIMO OTA measurements. The convolution of CM and antenna pattern is effectively emulating an MPAC system with infinite number of probes and thus no loss in accuracy that a very sparse MPAC system could introduce. Extensive correlation and harmonization exercises between the MPAC OTA and the RTS methodologies for 2x2 LTE MIMO have shown equivalence between these approaches which were eventually considered harmonized [16].

Figure 6: RTS Methodology: Equivalence with MPAC

Clearly, this brief overview of the RTS methodology shows that a single-probe OTA system paired with a CE with advanced capabilities, together with the knowledge of the complex antenna patterns of the UE is, in principle, able to emulate complex, 3D channel models.

The current applicability of the RTS methodology, however, is limited to static UE antenna systems as outlined in [16], [19], e.g., from [19]

Use of the TDD RTS test method requires the device supports the ATF defined in Annex R, uses only two Rx antennas for the tested band and the antenna pattern shall not change in response to the radio environment.

The complex antenna pattern, used inside the CE in stage 2 as explained above, is assessed in the first stage of RTS where the complex pattern of the DUT is recorded with both magnitude and phase using a dedicated antenna test function for LTE [16], [19] or using SS-RSRPB/RSARP measurement capabilities for NR [17]. The applicability of RTS could potentially include dynamic UE antenna systems, however, this would require the following aspects to be followed/known

- Every antenna pattern from each RX antenna/antenna module (including the beams from each codebook) would have to be fully known and assessed in 3D for magnitude and phase

- Prior and exact knowledge which antenna pattern(s) will be active for any UE orientation and radio environment

Aspect #1 could potentially be achievable, i.e., the assessment of every pattern for each module and codebook. However, the test time to collect all 3D antenna patterns would be prohibitive given the large number of different patterns in the tens or rather hundreds, i.e., beams for each codebook and each module, the need for rather fine measurement grids due to the highly directive nature of FR2 beams, and the need for various test functions to switch between different receivers and to cycle between different code books.

Aspect #2 seems to be the most challenging as it is highly unlikely that OEMs can provide accurate mapping of which antenna(s) will be activated for each test condition/coordinate and especially radio/channel condition.

Without the knowledge of both #1 and #2, the RTS methodology is simply not applicable for active antenna systems which AI/ML Beam Management test cases are based on.

The following ideas were captured in [13], [14], [15] with respect to the UE patterns

From [13]: “It is preferred that the antenna beam steering patterns are also standardized. TBD if we need to measure antenna beam patterns of each DUT independently.” From [14]: “Regarding the UE Rx Beam/Antenna pattern, there are 3 alternatives: - RAN4 defines a Reference UE Rx Beam/Antenna pattern which is used for AI/ML BM requirement definition and testing. - UE vendor provides the UE Rx Beam/Antenna pattern to the TE during testing. - TE measures the UE Rx Beam/Antenna pattern before starting the test (the probably well known Radiated Two Stage Method RTS)” From [15] “If to use synthetic channels defined in 38.901, BS/UE antenna configuration & radiation pattern defined in 38.901 can be used to determine TE transmission power for each Tx beam in set B (as shown in Fig.4). RAN4 can further discuss which antenna configuration to use during training and inference.”

The key concerns with the proposed approaches are as follows:

- Standardized antenna patterns: the AI algorithm developed by OEMs/chipset vendors could be tailored to vendor specific patterns/antenna system architectures, i.e., the UE could fail the test with standardized pattern but would otherwise pass if assessed with the actual antenna patterns or vice versa

- Measured antenna patterns: the actual empirical assessment of the antenna patterns would be very time intensive as outlined above.

- Vendor provided antenna patterns: the antenna patterns could be synthesized to make the conformance test case pass even though it would fail with the actual pattern

Observation 4: The RTS methodology, applied to AI/ML Beam Management use cases, would require either standardized, measured, or vendor provided antenna patterns.

Observation 5: The measurement of all possible antenna patterns would be extremely time consuming

Observation 6: The standardized or vendor provided antenna patterns could lead to incorrect assessments/verdicts of conformance test cases

With antenna patterns other than the actual, measured patterns, i.e., standardized or vendor provided, the AI/ML use case are merely assessing the chipset performance and/or AI/ML algorithm rather than the full end-to-end performance. Regardless of whether the antenna patterns are standardized, measured, or provided by vendors, the mapping, #2 above, i.e., which pattern the UE would select for any given test condition/coordinate and radio/channel condition, has not been addressed and is likely vendor proprietary and cannot/should not be standardized as it cannot be disclosed by OEMs/chipset vendors.

Observation 7: The RTS methodology, applied to AI/ML Beam Management use cases, would require mapping of antenna patterns to any given test condition/coordinate and radio/channel condition which is likely vendor proprietary.

It is therefore proposed not to consider the single-probe OTA system approach any further for AI/ML Beam Management use cases.

Proposal 3: Do not consider the single-probe OTA system approach any further for AI/ML Beam Management use cases

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: A relatively simple and straightforward extension of an FR2 MIMO OTA system to 8 probes is suitable to emulate 4 dual-polarized/8 single-polarized Tx-beam RX-PAS profiles, set B Tx beams up to [16] with power offsets, and set A Tx beams up to [192] with power offsets.

Observation 2: The extension of an FR2 MIMO OTA system to 8 probes is suitable to support unlimited range of AoDs/ZoDs and a range of AoAs/ZoAs of 110°/25°.

Observation 3: Various beam management performance testing can readily be supported with a commercially available test system, i.e., complexity and time-to-market concerns should not be an issue.

Observation 4: The RTS methodology, applied to AI/ML Beam Management use cases, would require either standardized, measured, or vendor provided antenna patterns.

Observation 5: The measurement of all possible antenna patterns would be extremely time consuming

Observation 6: The standardized or vendor provided antenna patterns could lead to incorrect assessments/verdicts of conformance test cases

Observation 7: The RTS methodology, applied to AI/ML Beam Management use cases, would require mapping of antenna patterns to any given test condition/coordinate and radio/channel condition which is likely vendor proprietary.

Proposal 1: UE/chipset/infra vendors to provide feedback on whether modelling 4 dual-polarized/8 single-polarized RX-PAS accurately with the remaining beams modelled with power offsets is sufficient

Proposal 2: Consider an 8-probe OTA system with 2 additional probes when compared to the FR2 MIMO OTA system with a range of AoAs/ZoAs of 110°/25° as the starting point for AI/ML Beam management testing for 1 TRP and/or 2 TRPs with close angular proximity.

Proposal 3: Do not consider the single-probe OTA system approach any further for AI/ML Beam Management use cases

# References

- RP-240774, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN Meeting #103, March 2024

- R4-2410570, WF on AI/ML, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #111, May 2024

- R4-2414323, WF on AI/ML, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #112, August 2024

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #112bis, October 2024

- R4-2409770, OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases, Keysight Technologies, 3GPP TSG-RAN4 Meeting #111, May 2024

- R4-2411254, OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases, Keysight Technologies, 3GPP TSG-RAN4 Meeting #112, August 2024

- R4-2416477, On Testability Aspects for FR2 AI/ML Beam Management, Keysight Technologies, 3GPP TSG-RAN4 Meeting #112bis, October 2024

- R4-2411587, Discussion on test setup for AI/ML based beam management, Rohde & Schwarz, 3GPP TSG-RAN4 Meeting #112, August 2024

- R4-2411279, Study on CDL channel model for AI/ML beam management in FR2, Anritsu Corporation, 3GPP TSG-RAN4 #112, August 2024

- R4-1904586, MPAC setup -Feasibility study, MVG Industries, 3GPP TSG-RAN WG4 #90-bis, April 2019

- TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)

- TS 38.151, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements

- R4-2417673, FR2 test setup for AI/ML beam management evaluation, Anritsu Corporation, 3GPP TSG-RAN4 #113, November 2024

- R4-2418941, Discussion on test setup for AI/ML based beam management, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #112bis, November 2024

- R4-2417894, Discussion on testability issues of AI/ML RAN4 BM use case, MediaTek Inc., 3GPP TSG-RAN WG4 Meeting # 113, November 2024

- TR 37.977, Verification of radiated multi-antenna reception performance of User Equipment (UE)

- TS 38.215, Physical layer measurements

- TS 38.521-4: User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance requirements

- TS 37.544, Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) Over The Air (OTA) performance; Conformance testing