3GPP TSG-RAN WG4 Meeting #114									                        R4-2502190

Athens, Greece, February 17 – 21, 2025

Agenda item:			7.1.1.3.3

Source:	Ericsson

Title:	On DeltaT_RxSRS requirements for 3T6R SRS-AS

Document for:	Approval

# Introduction

In RAN#104, a revised WID has been approved for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” WI [1]. One of the three main parts of the WI is to specify 6Rx UE RF requirements for both handheld and FWA UEs, with the following objectives:

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability

    - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability.

      - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this contribution, we focus on the second-to-last bullet point, which is to specify the requirements to support SRS antenna switching. In previous meetings, the requirements for ‘t1r6’, ‘t2r6’ and ‘t1r6-t2r6’ SRS antenna switching (AS) capabilities have been agreed. Even though ‘t3r6’ AS capability is also included in the WID, it is not currently specified in TS38.306, so only in the revised WID it was agreed for “RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability”. The work in RAN1 has started in RAN1#118bis and finished in RAN1#119 meeting (Nov. 2024), so in this contribution we will analyze ‘t3r6’, ‘t1r6-t3r6’, ‘t2r6-t3r6’, and ‘t1r6-t2r6-t3r6’ SRS AS capabilities and propose the corresponding ΔTRxSRS requirements.

# Discussion

In the approved WF from RAN4#113 [2], the following was captured on the SRS AS requirements topic:

Issue 1-1-1: Whether to consider a subset of available patterns for 3T6R AS-SRS

Way Forward: RAN4 to further discuss the following options.

-	Option 1: Consider a subset of available patterns for 3T6R AS-SRS. FFS on the subset details.

-	Option 2: Consider all available patterns for 3T6R AS-SRS.

Up to Rel-19, for all 4 Rx and 8 Rx AS-SRS patterns, there have been no exceptions in TS38.101-1 [3] when it comes to the specification of ΔTRxSRS requirements. For 3T6R related AS-SRS, there are four different patterns: ‘t3r6’, ‘t1r6-t3r6’, ‘t2r6-t3r6’ and ‘t1r6-t2r6-t3r6’. Option 1 basically proposes to specify ΔTRxSRS requirement for only a subset of those patterns.

In our understanding, the only motivation for this proposal is the simplification of specifications, which are indeed complex for SRS Antenna Switching. However, there is no technical reason which would justify it, as there is nothing related to 3T6R SRS-AS patterns which is significantly different to 4 Rx and 8 Rx patterns, e.g. 4T8R.

The consequence of omitting some SRS-AS pattern, e.g. ‘t3r6’, would be the incomplete ΔTRxSRS requirements. One could interpret it as if there were no minimum requirement defined for ΔTRxSRS for ‘t3r6’ SRS-AS, meaning that it could be arbitrarily large, e.g., 15 dB. That could very negatively impact the performance of reciprocity-based CSI estimation, which is certainly not desirable given many challenges in this area.

Conformance testing of ΔTRxSRS requirements is a different subject, where currently there is no such testing specified. In our contribution [4] (as a part of TEI-17), we have proposed RMCs for SRS transmissions that should be introduced to TS38.101-1, where the rest could be handled by RAN5.

Proposal 1: Consider all available patterns for 3T6R AS-SRS.

Issue 1-2-1: Proposed ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6

Way Forward: RAN4 to check whether the ∆TRxSRS values in the table below could be agreed. The average values from company proposals, as well as the existing values for other AS-SRS configurations, were considered as the starting point and were rounded to align with 0.5dB increments to keep consistency with the existing specification approach.

Table: ΔTRxSRS values

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.0 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6.0 | 5.5 | 7.0 |
| Band n104 | 5.0 | 7.0 | 6.0 | 8.0 |

As a reminder, ΔTRxSRS is a maximum allowed relaxation applied to the lower bound of the maximum configured transmitted power PCMAX,f,c due to the additional insertion loss of a given diversity branch relative to the main branch. The main “contributors” to the insertion loss are the antenna switches, filters, as well as PCB trace/RF cable routing located between the PA which sounds the given SRS port and the antenna port/connector.

ΔTRxSRS is measured at the antenna connector, and it is important not to have too loose margins in order to limit degradation of DL-MIMO performance, i.e. to have an adequate reciprocal CSI-RS estimation, especially since the gNB measures the resulting radiated SRS power per SRS port(s) which include different antenna element gains (in a typical implementation, antenna elements on diversity paths have a poorer quality compared with the antenna elements on the main path).

In our contribution for the previous meeting [5], we have provided a detailed analysis on ‘t3r6’, ‘t1r6-t3r6’, ‘t2r6-t3r6’, and ‘t1r6-t2r6-t3r6’ AS-SRS configurations. We will not repeat the same analysis here; instead, we will only recall the proposed ΔTRxSRS requirements:

| Operating Bands | t3r6 (dB) | t1r6-t3r6 (dB) | t2r6-t3r6 (dB) | t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.0 | 4.0 | 4.0 | 5.0 |
| Band n79 | 4.5 | 5.5 | 5.5 | 6.5 |
| Band n104 | 5.0 | 6.0 | 6.0 | 7.0 |

Many other companies have also made their proposals on ΔTRxSRS requirement values, and the table found in the Way Forward [2] is the result of averaging of those values (rounded to align with 0.5dB increments), taking into account the existing values for other AS-SRS configurations.

Compared with our proposal, the average values from the WF are up to 1.0 dB relaxed, but for the sake of progress, we will be ready to compromise to those values.

Proposal 2: Agree with the values from the Way Forward, i.e. specify the following ΔTRxSRS requirements for 3T6R AS-SRS:

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.0 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6.0 | 5.5 | 7.0 |
| Band n104 | 5.0 | 7.0 | 6.0 | 8.0 |

# Conclusion

Based on the discussion in the previous sections we have made the following proposals:

Proposal 1: Consider all available patterns for 3T6R AS-SRS.

Proposal 2: Agree with the values from the Way Forward, i.e. specify the following ΔTRxSRS requirements for 3T6R AS-SRS:

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.0 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6.0 | 5.5 | 7.0 |
| Band n104 | 5.0 | 7.0 | 6.0 | 8.0 |

# References

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN#104.

- R4-2420376, “WF on 6Rx requirements”, AT&T, RAN4#113.

- 3GPP TS38.101-1 V19.0.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

- R4-2501566, “(NR_RF_TxD-Core) Reference measurement channels for SRS transmissions”, Ericsson, RAN4#114.

- R4-2419428, “On DeltaT_RxSRS requirements for 3Tx and 6Rx UE RF”, Ericsson, RAN4#113.