3GPP TSG-RAN WG4 Meeting #114									                        R4-2502192

Athens, Greece, February 17 – 21, 2025

Agenda item:			7.22.2

Source:	Ericsson

Title:	On UE RF requirements for NR MIMO Phase 5

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for the RAN1-led WI on “NR MIMO Phase 5” WI [1], where the objectives of the WI with the identified potential RAN4 UE RF impacts are the following:

RAN1:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

Acronyms:

- sTRP: single TRP (transmit-receive point)

- mTRP: multiple TRP (transmit-receive point)

- CJT: coherent joint transmission

- DU: distributed unit

- CRI: CSI-RS resource indicator

RAN2:

Specify higher layer support of the enhancements listed above.

RAN4:

Specify necessary core requirements for the enhancements listed above.

In this contribution, we continue the discussion on the UE RF requirements for “Support for 3-antenna-port codebook-based transmissions” (“3Tx” topic) and “Asymmetric DL sTRP/UL mTRP” working areas.

# Discussion

## 3Tx

In RAN4#113, a WF has been approved [2]. In the following, we discuss the remaining open issues for the “3Tx” topic.

### Power class

On the support for different power classes, the following has been agreed in the previous meeting:

## <Sub-topic 1-1> Power class

<Agreement>: PC for 3Tx requirement

- 3Tx PC1.5 cannot be supported for full power mode 0 in Rel-19

<Way forward>: FFS on PC1.5 for power scaling mode (3-layer) with its applicable UE type

According to the WID [1], the UL full power transmission modes 1 and 2 are not in the scope, while for the full power mode 0 it was agreed in the previous meeting that it cannot be supported for PC1.5, as it can be seen above. Hence, the uplink full power transmission (ul-FullPowerTransmission in PUSCH-Config), introduced in Rel-16, is not supported for PC1.5 in Rel-19.

The default “Rel-15 behavior” (where the uplink full power transmission is not supported) for a PUSCH transmission scheduled by a DCI format other than DCI format 0_0, or configured by ConfiguredGrantConfig or semiPersistentOnPUSCH, if txConfig in PUSCH-Config is set to 'codebook' and if each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, is [3]:

- “…the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of SRS ports supported by the UE in one SRS resource. The UE splits the power equally across the antenna ports on which the UE transmits the PUSCH with non-zero power.”

Since 3 Tx operation is set up by configuring the UE with a 4 port SRS resource with port 1003 disabled, the text above may be confusing. Following Rel-15 principles, the UE should scale down by 3 (‘the maximum number of SRS ports supported by the UE in one SRS resource’ should be equal to 3) rather than 4 for 3 Tx, even though a 4 port SRS resource is configured to the UE. While RAN1 have not agreed how to update the specification text above, RAN1 have agreed that the scaling down should be by 3 for PUSCH (and also for SRS) so that Rel-15 principles are followed:

Agreement (RAN1#116bis)

For 3-port codebook-based PUSCH transmission for a 3TX UE, scale factor s should be the ratio of the number of antenna ports with a non-zero PUSCH transmission power to 3 (except for full-power Mode 0).

- FFS: Whether specification needs to be updated to reflect the above

Agreement (RAN1#116bis)

For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, UE splits a linear SRS power equally across the 3 unmuted antenna ports of the 4-port SRS resource.

In particular, for a UE equipped with 3 Tx chains, the Rel-15 behavior for 3-layer PUSCH transmission implies that the scaling factor is equal to 1 (since both ‘the number of antenna ports with a non-zero PUSCH transmission power’ and ‘the maximum number of SRS ports supported by the UE in one SRS resource’ are equal to 3), and that the power is equally split across three antenna ports (three layers). The power of 29dBm (PC1.5) could be achieved by a UE equipped with 3 PAs of 26dBm each.

Furthermore, Rel-15 behavior is supported for 2Tx and 4Tx UEs, with no particular limitation on PC1.5 for an operating band supporting it.

Proposal 1: PC1.5 should be supported by 3Tx UEs with no uplink full power transmission modes defined, i.e. with the “Rel-15 behavior”. It should be feasible for both FWA and handheld UEs.

### MPR

On the MPR requirements topic, the following was agreed in the approved WF [2]:

## <Sub-topic 1-3> MPR

<Agreement>: PC3 for 3Tx

- Use the same MPR as MPR for PC3 single carrier.

<Agreement>: PC2 for 3Tx

- Use the same MPR as MPR for PC2 single carrier.

  - FFS on whether PC2 MPR with 1Tx or 2Tx will be applied

<Way forward>: FFS on PC1.5 MPR to use the same MPR as PC1.5 MPR with 2Tx. Subject to  Tx Diversity support

According to some evaluations submitted in the previous meetings, e.g. [4], for both PC2 and PC1.5 it was concluded that the MPR required is slightly smaller than the current corresponding 2Tx UL MIMO MPR requirement. Thus, it was proposed to reuse corresponding 2Tx MPR requirements for 3Tx as well, which is reasonable in our view.

For 2Tx MPR requirements for PC1.5, there are currently two tables in TS38.101-1: the default table and the table targeted to large FWA form factor with 20 dB or above antenna isolation. One approach would be to reuse both tables for 3Tx, and the other approach would be to only adopt the table targeted to large FWA form factor with 20 dB or above antenna isolation (since the required MPR requirements for 3Tx are smaller than the MPR for 2Tx).

Proposal 2: For both PC2 and PC1.5, reuse the legacy 2Tx MPR requirements to specify 3Tx MPR requirements. For PC1.5, either adopt only the 2Tx MPR table ‘targeted to large FWA form factor with 20 dB or above antenna isolation’ or adopt both 2Tx MPR tables specified in TS38.101-1.

## Asymmetric DL sTRP/UL mTRP

In the approved WF [2], the following was captured on this topic:

<Sub-topic 2-1> UE RF impact

<Way forward>: FFS on study of UE RF requirement for different DL/UL TCI states in the WI or TEI

- R4-2417632 and R1-2410870 are for information.

On this topic, the following question has been sent to RAN1 in RAN4#112-bis meeting in the approved LS [5]:

Question 1: RAN4 would like to inquire if the UL TRP(s) can transmit SSBs per the scope of the WID.

In RAN1#119, RAN1 has replied with the following [6]:

Reply for Question 1: From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

Based on the reply from RAN1, both scenarios where the UL mTRP transmits or does not transmit SSB are included in this WI. RAN4 should specify the UE RF requirements for both scenarios for the work to be completed in Rel-19 (if the impact is identified).

In [7], it has been recognized that there is going to be an impact on the beam correspondence requirements by introducing this feature, only for the case where the UL mTRP node transmits SSB. In our view, the requirements from clause 6.6 could be reused, but limited only to the case of SSB based beam correspondence (not CSI-RS based). Also, it should be clearly captured that the DL measurements are not being performed from the TRP node configured for PDSCH transmissions.

Observation 1: According to the RAN1 reply, a scenario where the UL TRP(s) can transmit SSBs is not precluded in this WI, and at least the beam correspondence requirements should be specified for that scenario for the work to be completed.

# Conclusion

Based on the discussion in the previous sections we have made the following observation:

Observation 1: According to the RAN1 reply, a scenario where the UL TRP(s) can transmit SSBs is not precluded in this WI, and at least the beam correspondence requirements should be specified for that scenario for the work to be completed.

We have also made the following proposals:

Proposal 1: PC1.5 should be supported by 3Tx UEs with no uplink full power transmission modes defined, i.e. with the “Rel-15 behavior”. It should be feasible for both FWA and handheld UEs.

Proposal 2: For both PC2 and PC1.5, reuse the legacy 2Tx MPR requirements to specify 3Tx MPR requirements. For PC1.5, either adopt only the 2Tx MPR table ‘targeted to large FWA form factor with 20 dB or above antenna isolation’ or adopt both 2Tx MPR tables specified in TS38.101-1.

# References

- RP-242394, “WID revision: NR MIMO Phase 5”, RAN#105.

- R4-2420400, “WF on UE RF requirements for MIMO enhancement”, Samsung, RAN4#113.

- 3GPP TS38.213 V18.5.0, “NR; Physical layer procedures for control”.

- R4-2414983, “R19 MPR for 3Tx”, OPPO, RAN4#112-bis.

- R4-2417117, “LS on UE RF issues for Rel-19 MIMO enhancement”, Huawei, RAN4#112-bis.

- R1-2410870, “Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement”, Qualcomm Incorporated, OPPO, RAN1#119.

- R4-2417632, “dCR on RF requirements for a UE with different TCI states for UL and DL”, Qualcomm Incorporated, RAN4#113.