3GPP TSG-RAN WG4 Meeting #114									                        R4-2502195

Athens, Greece, February 17 – 21, 2025

Agenda item:			7.10.2.3.1

Source:	Ericsson, Sony

Title:	On UE RF Tx requirements for IoT-NTN HPUE

Document for:	Approval

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for IoT-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the SAR related issue for IoT-NTN HPUE, captured in the approved WF in RAN4#113 [2].

# Discussion

In the approved WF [2], the following was captured:

Issue 2-1-2: If introduced, whether and how to enhancement current duty cycle method in TN.

- Proposals

  - Proposal 1:

    - To consider the serving time of each beam, or the beam service time ratio of the UE, as a potential upper bound of the UL ’duty-cycle’ for the NTN HPUE. (Samsung)

    - To further discuss the duty-cycle solution, and to check if it can be altered for NTN scenario to allow UE to stay in its declared power class as much as possible, which helps NTN HPUE to close the link in emergencies and compensate the rain/cloud fading during natural disasters. (Samsung)

  - Proposal 2:

    - Besides of maxUplinkdutycycle capability reporting, UE working status like near the head mode (SAR is required) or other modes like handheld mode (SAR is not required) also need to be indicated. (oppo)

    - BSR (buffer status report) can be reused to report the UE working status of following transmission time. (oppo)

  - Proposal 3:

    - Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value. (Ericsson, Sony)

    - If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details: (Ericsson, Sony)

      - If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance:

        - DPC = ‘00’: the criteria to report DPC is not met

        - DPC = ‘01’: ΔPPowerClass = 0 dB

        - DPC = ‘10’: ΔPPowerClass = 3 dB

        - DPC = ‘11’: ΔPPowerClass = 6 dB

      - If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c):

        - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications

        - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB

        - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB

        - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB

  - Proposal 4:

    - It is not reasonable for network to send the control message to control SAR of each UE. (Nokia)

    - In principle, applying P-MPR cannot invalidate the purpose of letting IoT-NTN UEs have high Tx power as PC1/PC1.5/PC2 devices do.(Nokia)

    - The combination of using duty cycle and small P-MPR can be considered for high power UE to meet SAR requirements.(Nokia)

- Agreement:

  - Further discuss the proposed schemes related to duty cycle related SAR scheme.

  - Other schemes or no duty cycle related scheme are not precluded.

For Specific Absorption Rate restriction (SAR) mitigation, in NR TN specifications (TS38.101-1 [3]) an optional method based on the “UL duty cycle” is specified, where the UE can report maxUplinkDutyCycle-PC2-FR1 or maxUplinkDutyCycle-PC1dot5-MPE-FR1 capabilities (otherwise a default duty cycle of 50% or 25% applies, respectively). If the percentage of transmitted uplink symbols at the UE exceeds the corresponding threshold, the UE is supposed to do a power-class fallback.

Such method is not well designed for FDD bands, as it is not specified how exactly the UE implements such a solution (i.e. what is the averaging period for the duty cycle at the UE), since it is only stated that “the percentage of uplink symbols transmitted in a certain evaluation period is larger than X% (The exact evaluation period is no less than one radio frame);” This period could be anything from one radio frame to many radio frames, and the network is not aware of how the UE makes that choice. Moreover, allegedly, no UEs in the field provide these capabilities. UEs use P-MPR method instead for managing excessive duty cycles.

However, the P-MPR method is blind for the network as the network is not aware if a P-MPR is applied due to a proximity sensor or in case of “simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications” (the original intentions), or for reducing the average power to facilitate SAR compliance.

Observation 1: “Duty-cycle” based method is not well designed for FDD bands and is allegedly not used in the field. P-MPR method is used instead, which is on another hand “blind” for the network.

In Rel-18, as part of “Enhancement of increasing UE power high limit for CA and DC” part of “Further NR coverage enhancements” WI, reporting of ΔPPowerClass (DPC) was introduced in the PHR (Power Headroom Report) for improving network scheduling by providing the network with the actual power-capability state of a UE. The ΔPPowerClass (DPC) in the PCMAX,f,c for a serving cell c is set by a UE depending on the actual scheduled UL duty cycle in accordance with its reported UL duty cycle capabilities. The background to the reporting is that RAN4 was informed by RAN1 that [4]:

RAN1 made the following agreement concerning enhancements to realize increasing UE power high limit for CA and DC. Agreements For enhancements to realize increasing UE power high limit for CA and DC, RAN1 can study based on RAN4’s input - Whether RAN1 enhancements to information exchange between UE and gNB are needed to improve scheduling and network performance when using higher power CA/DC. FFS how to realize such information exchange, e.g., signalling enhancement, and what is the spec impact.

The original scope was only CA and DC, but since the similar issues were recognized for non-CA case as well, it was also included to the RAN4 discussion.

In RAN4, it was agreed that such reporting is triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (the introduced UE capability deltaPowerClassReporting-r18). This would indeed circumvent some of the difficulties associated with duty-cycle reporting, as the network is told when a power class is modified for SAR mitigation without limiting UE implementation to this end. Thus, a “duty cycle” based method together with similar reporting could be considered as a solution for SAR compliance for NTN HPUEs as well.

However, as stated above, the problem is that no UEs in the field provide the UL duty-cycle capabilities and UEs use P-MPR method instead (as that method is a better fit for FDD bands). So, for NTN HPUEs the optional DPC-reporting feature should also be available to UEs using P-MPR for managing excessive UL duty cycles, which would ensure that the feature can be used for improving scheduling in the field based on the actual power capability for supporting UEs.

Observation 2: “Duty-cycle” based and the P-MPR based methods in combination with reporting of the actual power-capability state of a UE could be considered for NTN HPUEs, similar to Rel-18 enhancements introduced for TN.

The “updated” Rel-18 “Single Entry PHR MAC CE” has the following structure [5]:

Currently, the DPC field is defined as follows, irrespective of P field, which for FR1 only indicates that the power backoff is applied due to the power management (as allowed by P-MPRc):

DPC = ‘00’: the criteria to report DPC is not met

DPC = ‘01’: ΔPPowerClass = 0 dB

DPC = ‘10’: ΔPPowerClass = 3 dB

DPC = ‘10’: ΔPPowerClass = 6 dB

The PHR reporting that could be applied to NTN UEs could reuse exactly the same structure as for the “Single Entry PHR MAC CE”, but with an improvement where the DPC field could indicate either the actual ΔPPowerClass applied in PCMAX,f,c or the actual P-MPR applied (with granularity of 3dB):

- If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance:

  - DPC = ‘00’: the criteria to report DPC is not met

  - DPC = ‘01’: ΔPPowerClass = 0 dB

  - DPC = ‘10’: ΔPPowerClass = 3 dB

  - DPC = ‘11’: ΔPPowerClass = 6 dB

- If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c):

  - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications

  - DPC = ‘01’: 0 dB when P-MPRc = 0dB

  - DPC = ‘10’: 3 dB when 0 < P-MPRc ≤ 3 dB

  - DPC = ‘11’: 6 dB when 3 < P-MPRc ≤ 6 dB

Proposal 1: Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value.

Proposal 2: If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details:

- If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance:

  - DPC = ‘00’: the criteria to report DPC is not met

  - DPC = ‘01’: ΔPPowerClass = 0 dB

  - DPC = ‘10’: ΔPPowerClass = 3 dB

  - DPC = ‘11’: ΔPPowerClass = 6 dB

- If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c):

  - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications

  - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB

  - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB

  - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB

# Conclusion

Based on the discussion in the previous sections we have made the following observations:

Observation 1: “Duty-cycle” based method is not well designed for FDD bands and is allegedly not used in the field. P-MPR method is used instead, which is on another hand “blind” for the network.

Observation 2: “Duty-cycle” based and the P-MPR based methods in combination with reporting of the actual power-capability state of a UE could be considered for NTN HPUEs, similar to Rel-18 enhancements introduced for TN.

Based on the discussion in the previous sections we have made the following proposals:

Proposal 1: Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value.

Proposal 2: If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details:

- If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance:

  - DPC = ‘00’: the criteria to report DPC is not met

  - DPC = ‘01’: ΔPPowerClass = 0 dB

  - DPC = ‘10’: ΔPPowerClass = 3 dB

  - DPC = ‘11’: ΔPPowerClass = 6 dB

- If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c):

  - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications

  - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB

  - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB

  - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

- R4-2420404, “WF on HPUE for IoT NTN”, vivo, RAN4#113.

- 3GPP TS38.101-1 V19.0.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

- R1-2210739, LS on enhancements to realize increasing UE power high limit for CA and DC, RAN1.

- 3GPP TS38.321 V18.4.0, “NR; Medium Access Control (MAC) protocol specification”.