3GPP TSG-RAN WG4 Meeting #114							     	 	   		R4-2502210

Athens, GR, 17th – 21st February 2025

Title: 	Supporting full power mode 0 for 3Tx

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.22.2	UE RF requirements	[NR_MIMO_Ph5-Core]

Document for:	Discussion

# 1	Background

In R4#113, we discussed potential UE architectures for 3Tx in [1], and several agreements were captured in way forward [2]. In this contribution, we further discuss the support of power mode 0 for 3Tx.

# Discussion

## 2.1 Way forward input

The way forward [2] captured an agreement on full power mode 0:

<Agreement>: PC for 3Tx requirement

- 3Tx PC1.5 cannot be supported for full power mode 0 in Rel-19

<Way forward>: FFS on PC1.5 for power scaling mode (3-layer) with its applicable UE type.

If we agree that this is true for full power mode 0 using the 3 paths, it is unclear to us why the UE could not fall back to 2Tx to support full power mode 0.

In is our understanding that the PC1.5 3Tx architecture uses three PC2 PAs in order to achieve the power class level of 29dBm with three antenna UL MIMO. As such, with two paths, it is capable of also achieving the 29dBm power class level. Thus, to support full power mode 0 in Rel-19, it would be sufficient to use the current 2Tx TxD capability by falling back to only two paths for power mode 0.

Proposal: To support power mode 0 for 3Tx PC1.5 the UE is allowed to fall back to two paths (2Tx) and use the current 2Tx TxD capability to support full power mode 0.

# Conclusion

In this contribution, we observe that a 3Tx PC1.5 UE implementation can achieve full power mode 0 in 2Tx mode with the current TxD capability (2Tx).

Proposal: To support power mode 0 for 3Tx PC1.5 the UE is allowed to fall back to two paths (2Tx) and use the current 2Tx TxD capability to support full power mode 0.

# References

[1] R4-2419411 Architecture and assumptions for 3Tx MPR, Skyworks Solutions Inc., R4#113

[2] R4-2420329 WF on UE RF requirements for MIMO enhancement, Samsung, R4#113