3GPP TSG-RAN WG4 Meeting #114	R4-2502230

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.23.3.2

Source: 	Qualcomm Incorporated

Title: 		RRM requirements for SBFD operation

Document for:	Discussion

# Introduction

For release 19 work item on Evolution of NR duplex operation: Sub-band full duplex (SBFD), the main objective is to specify the sub-band full duplex operation. RAN4 has already started discussions on the RRM requirements and reached a few high-level agreements [2,3,4] in the last meetings. In this paper, we further discuss various issues related to CSI-RS based measurements with SBFD operation.

# CSI-RS based L1 measurements

- For CSI-RS puncturing in frequency domain in SBFD symbols,

  - For CSI-RS on SBFD symbols, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs.

  - For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (for the DU configuration) is no less than the existing threshold.

    - Note: in existing requirements, CSI-RS based RLM/BFD/CBD requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 24 RBs. CSI-RS based L1-RSRP/L1-SINR requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 48 RBs.

    - FFS for the DUD configuration.

  - For CSI-RS on SBFD symbols, if the CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, FFS whether existing accuracy requirements for CSI-RS based measurements can apply and related additional side conditions.

  - For CSI-RS on SBFD symbols, no accuracy requirements apply if the usable number of CSI-RS PRBs after applying the puncturing operation is below the existing threshold.

  - FFS the measurement behavior for DUD configuration.

  - Above bullets apply to L1-RSRP/SINR measurement and also RLM/BFD/CBD measurement.

RAN4 made several agreements related to CSI-RS based L1 measurements in the last meeting where one of the main issue discussed was related to processing of punctured CSI-RS in frequency domain in SBFD symbols.

The main FFS issues in the above agreements are related to DUD configuration when the minimum number of CSI-RS PRBs are spread across the two DL sub-bands. Basically, how to specify the measurement accuracy and measurement delay requirements in such cases.

We think, like DU configuration, even DUD configuration should ensure that atleast the minimum number of CSI-RS RBs are contiguous. In other words, to reuse the existing measurement delay and accuracy requirements, at-least one of the DL sub-band should have consecutive CSI-RS PRBs which is not less than the existing threshold, else the UE cannot meet the existing measurement delay and accuracy requirements.

Proposal 1: In DUD configuration, existing delay and accuracy requirements for CSI-RS based measurements apply only if the number of consecutive CSI-RS PRBs in atleast one of the DL subband is no less than the existing thresholds, i.e., 24 RBs for RLM/BFD/CBD and 48 RBs for L1-RSRP/L1-SINR.

# Conclusion

Proposal 1: In DUD configuration, existing delay and accuracy requirements for CSI-RS based measurements apply only if the number of consecutive CSI-RS PRBs in atleast one of the DL subband is no less than the existing thresholds, i.e., 24RBs for RLM/BFD/CBD and 48RBs for L1-RSRP/L1-SINR.

# References

[1] 	RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#104

[2] R4-2413900, “WF on SBFD RRM”, Huawei, RAN4#112

[3] R4-2416858, “WF on SBFD RRM”, Huawei, RAN4#112bis

[4] R4-2420097, “WF on RRM requirements for NR_duplex_evo”, Huawei, RAN4#113