# 3GPP TSG-RAN WG4 Meeting #114	R4-2502234

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.1.1.1.2

Source: 	Huawei, HiSilicon

Title:	Discussion on the Intra-band CA with PC1.5

Document for:	Approval

# 1	Introduction

Some progress were made on the RF requirements for PC1.5 intra-band CA as captured in the WF [1]. In the sequal, we’ll discuss the remaining open issues.

## 1.2 PSD assumption to derive MPR/A-MPR requirements for intra-band NC ULCA (with dualPA)

Agreement:

- Equal PSD as starting point

  - Companies are encouraged to check the MPR difference with 6dB PSD imbalance

- Inform RAN5 about the PSD condition for MPR derivation in RAN4

- The MPR difference compared to PC2 is supposed to be less than 3dB for PC1.5 in order to have performance gain over PC2

- Companies to clarify the PA configuration assumptions for MPR evaluation for PC3, PC2 and PC1.5 intra-band NC UL CA, for example, (PC3+PC3) for PC3

## 1.3 MPR applicability for intra-band contiguous and non-contiguous ULCA

Agreement:

- FFS for NC CA in next meeting

- Stop the discussion for contiguous CA because equal PSD has already been agreed as assumption to derive MPR requirements

## 1.4 PCMAX,c and PCMAX for intra-band NCCA (with dual PA)

Agreement:

- PCMAX,C is upper bounded by 26dBm

  - How to reflect it in spec is FFS

- For PCMAX, FFS in next meeting

  - Option 1: No change of current spec

  - Option 2: PPowerClass,CA = min{29,26+10*log(1+4*min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2))} dBm assuming 6dB PSD difference ,

or, =26+10*log(1+min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2)) assuming equal PSD,

can be used as a starting point.

## 1.5 Relative power tolerance

Agreement:

- FFS in next meeting

# 2	Discussion

In the current spec, MPRc = MPR is specified for both contiguous and non-contiguous intra-band ULCA. Use non-contiguous CA as an example, which is shown below:

It can be seen that Pcmax,c on the component carrier c shall follow the same requirement defined for single-carrier operation in subclause 6.2.4.

In previous discussions, it was often mistakenly proposed to set PCMAX,c = 26dBm for PC1.5 UL NC CA. Actually, it is the MOP of the component carrier that should be set to 26dBm, i.e. PPowerClass,c = 26dBm. And PCMAX,c is not a fixed value and can be set within the specified bounds.

Proposal 1: It’s proposed to set PPowerClass,c = 26dBm (not PCMAX,c = 26dBm) for the component carriers of UL NC CA for PC1.5 with dualPA-architecture.

Given that MPRc = MPR and P-MPRc = P-MPR, the lower and upper bounds per CC will be the same, i.e.

PCMAX_L,c1 = PCMAX_L,c2,   PCMAX_H,c1 = PCMAX_H,c2

For PC1.5 ULCA, the following can be derived based on the above conditions:

Table 1: The configured max output power for CA and component carriers

|  | PPwerClass,CA (dBm) | PPowerClass,c (dBm) | PCMAC_L,CA (dBm) | PCMAX_L,c1 (dBm) | PCMAX_L,c2 (dBm) | PCMAX_L,c1+PCMAX_L,c2 (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| 2Tx | 29 | 29 | 29-MPR | 29-MPR | 29-MPR | 32-MPR |
| DualPA | 29 | 26 | 29-MPR | 26-MPR | 26-MPR | 29-MPR |

It can be seen that:

Observation 1: With MPRc = MPR:

- For 2Tx, if both CCs are required to be transmitted at the lower bound PCMAX_L,c, the total power could exceed 29 dBm (if MPR<3), which is obviously not feasible;

- For DualPA, both CCs have to be transmitted at >= the lower bound PCMAX_L,c, in order for the total power to meet the lower bound of the total power PCMAX_L,CA, which means equal power per CC regardless of RB numbers.

It’s worth noting that when deriving the MPR under equal PSD condition, the actual MPRc per CC is not equal if different numbers of RBs are assigned per CC. For example, assume CC1 has 10 RBs and CC2 has 1 RB. At the point when the CA MPRCA is determined, the measured/simulated back-off per CC would differ by 10dB, i.e. MPRc2 – MPRc1 = 10dB.

Observation 2: When deriving the CA MPR under the equal PSD condition by measurement or simulations, the actual back-off per CC (i.e. MPRc) is not equal if different numbers of RBs are assigned per CC.

In summary, it’s not feasible for UE to meet the configured maximum power per CC PCMAX,c during ULCA operations.

Proposal 2: Under the condition of MPRc = MPR, a UE is not required to meet the configured maximum power per CC PCMAX,c during intra-band ULCA operations, and PCMAX,c is calculated only for PHR purpose.

As per the WF [1], RAN4 has agreed to inform RAN5 about the PSD condition for MPR derivation:

Inform RAN5 about the PSD condition for MPR derivation in RAN4

Logically, it’s reasonable to request that the test condition specified by RAN5 matches the assumption used in RAN4.

The equal PSD condition as described in the Relative power tolerance requirement could be used as the initial condition for the MPR test. Starting from this condition, same TPC “up” commands could then be sent to both component carriers to verify the configured total Tx power PCMAX for CA.

#### 6.3A.4.2.2	Relative power tolerance

##### 6.3A.4.2.2.1	Minimum requirements

For intra-band non-contiguous carrier aggregation, the requirements apply when the power of the target and reference sub-frames on each component carrier exceed the minimum output power as defined in subclause 6.3A.1 and the total power is limited by PUMAX as defined in subclause 6.2A.4. The UE shall meet the following requirements for transmission on both assigned component carriers when the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame:

a)	for all possible combinations of PUSCH and PUCCH transitions per component carrier, the corresponding requirements given in Table 6.3.4.3-1;

b)	for SRS transitions on each component carrier, the requirements for combinations of PUSCH/PUCCH and SRS transitions given in Table 6.3.4.3-1 with simultaneous SRS of constant SRS bandwidth allocated in the target and reference subrames;

c)	for RACH on the primary component carrier, the requirements given in Table 6.3.4.3-1for PRACH.

For a) and b) above, the power step P between the reference and target subframes shall be set by a TPC command and/or an uplink scheduling grant transmitted by means of an appropriate DCI Format.

Proposal 3: Inform RAN5 about the PSD condition for MPR derivation in RAN4 as well as to set the average power per PRB aligned across both assigned carriers as the initial test condition.

# 3	Conclusion

In this paper, we have shared our views on the configured output power requirements for PC1.5 intra-band ULCA.

The following observations and proposals are made:

Proposal 1: It’s proposed to set PPowerClass,c = 26dBm (not PCMAX,c = 26dBm) for the component carriers of UL NC CA for PC1.5 with dualPA-architecture.

Observation 1: With MPRc = MPR:

- For 2Tx, if both CCs are required to be transmitted at the lower bound PCMAX_L,c, the total power could exceed 29 dBm (if MPR<3), which is obviously not feasible;

- For DualPA, both CCs have to be transmitted at >= the lower bound PCMAX_L,c, in order for the total power to meet the lower bound of the total power PCMAX_L,CA, which means equal power per CC regardless of RB numbers.

Observation 2: When deriving the CA MPR under the equal PSD condition by measurement or simulations, the actual back-off per CC (i.e. MPRc) is not equal if different numbers of RBs are assigned per CC.

Proposal 2: Under the condition of MPRc = MPR, a UE is not required to meet the configured maximum power per CC PCMAX,c during intra-band ULCA operations, and PCMAX,c is calculated only for PHR purpose.

Proposal 3: Inform RAN5 about the PSD condition for MPR derivation in RAN4 as well as to set the average power per PRB aligned across both assigned carriers as the initial test condition.

# References

- R4-2420329 WF on HPUE RF requirements, Samsung, RAN4#113

- R4-2417098 WF on HPUE RF requirements, Samsung, Skyworks, Nokia, RAN4#112bis