3GPP TSG-RAN WG4 Meeting #114	R4-2502237

Athens, Greece, February 17 – 21, 2025

Source:	Nokia

Title:	Discussion on missing Case 2 requirements

Agenda item:	5.12.1

Document for:	Discussion

Introduction

In this paper we present Nokia’s views on missing Case 2 requirements for the NR_MG_enh2 WI [1]. As part of this discussion, we provide corresponding observations and proposals.

Discussion

Deactivated SCell measurements

At RAN4 #111, the discussion on measuring deactivated SCells for UE configured with concurrent measurement gaps with NCSG was continued. The following was agreed and captured in the way forward [2].

- Sub-topic 3-2: Rel-18 UE behavior for deactivated SCell measurements with NCSG Issue 3-2-1: [Case 2] When the UE is configured with Concurrent gaps with NCSG, what is the potential changes to UE behaviour for NCSG upon SCell activation (in Rel-18) Agreement from online session: - For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association]. - Further details on the processing delay between NCSG and Type 1/2 MG can be further discussed. - For UE configured with 2 NCSG, deactivated SCells are measured with NCSG - If the association is provided, deactivated SCells are measured with NCSG according to gap association. - If the association is not provided, UE is not expected to cause interruption outside the VIL due to measurement on any of the deactivated SCells, and the existing measurement delay requirement does not apply to this case.

For Rel-17, agreement was achieved for UE supporting NCSG, that deactivated SCell measurements will be performed within NCSG provided that NCSG is configured by the network.

For Rel-17, for UE supporting NCSG, deactivated SCell measurements are performed within NCSG provided that NCSG is configured by the network.

Hence for Rel-18 Case 2 requirements, the same UE behavior should apply: deactivated SCells are measured within NCSG, if UE supports concurrent gaps with NCSG.

If such UE is configured with one Type 1/2 MG and one NCSG, deactivated SCells are measured with NCSG, regardless of UE capabilities and gap association.

If such UE is configured with two NCSGs, deactivated SCells are measured with NCSG, according to gap association provided for each NCSG, i.e. according to the configured MOs. Else, if the gap association is not provided, UE is not expected to cause interruptions outside the VILs, as stated above.

The above should be reflected in normative text in section 9.1.13.2 related to Case 2 requirements. The following proposal is made.

The following text is added to clause 9.1.13.2 related to measuring deactivated SCells.

- “When UE supports concurrentMeasGapsNCSG-r18 capability and is configured with one NCSG pattern and at least one measurement gap pattern, the UE shall measure all de-activated SCCs within the NCSG, if fully or partially overlapped, regardless of the reported UE capabilities and gap association.

- When UE supports concurrentMeasGapsNCSG-r18 capability and is configured with two NCSG patterns, the UE shall measure all de-activated SCCs within the NCSGs, if fully or partially overlapped.

- -	if the gap association is provided, de-activated SCCs shall be measured with NCSG according to gap association.

- -	if the gap association is not provided, UE is not expected to cause interruption outside the VIL due to measurement on any of the de-activated SCCs, and the existing measurement delay requirement does not apply in this case.”

The proposed change is included in the accompanying CR [3].

Measurements upon SCell activation

There was also a CR submitted to RAN4 #113, adding the UE behavior when SCell is activated [4]. It proposed to take into account the reported UE capabilities per band and hence convert NCSG to MG for the configured measurement object if the UE reported “gap” in the NeedForGaps info for that band.

Firstly, this discussion is decoupled from UE behavior for measuring a deactivated SCell. Secondly, the network should have full control on the usage of MG or NCSG, based on UE capability report per band, and it may require to perform an RRC reconfiguration of the UE once the SCell is activated.

Once SCell is activated, the measurement object (MO) measured with NCSG for deactivated SCell may be conveyed to the existing MG (e.g. Type-2 MG), but some MOs may also need to be added and some removed due to mobility. Hence a new RRC reconfiguration command is needed in most cases. To assume there is no change in MOs when SCell(s) is/are activated, looks like a corner case to us.

Thus, there is no reason to specify a shift of MO from NCSG to the existing MG (Type-2 MG) as proposed in [4]. Instead, the network should have full control whether to add/remove MOs to/from the existing measurement gap or configure another concurrent measurement gap with a different MG pattern for these MOs previously assigned to deactivated SCell(s) or to keep the NCSG pattern with these MOs previously assigned to deactivated SCell(s). This adjustment is done via the RRC reconfiguration command, as in legacy case for an activated SCell. Thus, no further addition is needed in section 9.1.13.2.

After SCell activation, the network should have full control whether to add/remove MOs to/from the existing measurement gap or configure another concurrent measurement gap with a different MG pattern for these MOs previously assigned to deactivated SCell(s) or to keep the NCSG pattern with these MOs previously assigned to deactivated SCell(s), by applying the RRC reconfiguration command, as in legacy case for activated SCell.

No further text additions are needed in clause 9.13.2 related to SCell activation, as legacy UE behaviour is assumed.

Conclusion

In this paper we have presented Nokia’s views with regard to open issues related to Rel-18 WI on NR_MG_enh2 [1] for Case 2 requirements, in particular on measuring deactivated SCells for UE configured with concurrent measurement gaps with NCSG.

As part of this discussion, the following observations and proposals were made:

- For Rel-17, for UE supporting NCSG, deactivated SCell measurements are performed within NCSG provided that NCSG is configured by the network.

- The following text is added to clause 9.1.13.2 related to measuring deactivated SCells.

“When UE supports concurrentMeasGapsNCSG-r18 capability and is configured with one NCSG pattern and at least one measurement gap pattern, the UE shall measure all de-activated SCCs within the NCSG, if fully or partially overlapped, regardless of the reported UE capabilities and gap association.

When UE supports concurrentMeasGapsNCSG-r18 capability and is configured with two NCSG patterns, the UE shall measure all de-activated SCCs within the NCSGs, if fully or partially overlapped.

-	if the gap association is provided, de-activated SCCs shall be measured with NCSG according to gap association.

-	if the gap association is not provided, UE is not expected to cause interruption outside the VIL due to measurement on any of the de-activated.”

After SCell activation, the network should have full control whether to add/remove MOs to/from the existing measurement gap or configure another concurrent measurement gap with a different MG pattern for these MOs previously assigned to deactivated SCell(s) or to keep the NCSG pattern with these MOs previously assigned to deactivated SCell(s), by applying the RRC reconfiguration command, as in legacy case for activated SCell.

No further text additions are needed in clause 9.1.13.2 related to SCell activation, as legacy UE behaviour is assumed.

The proposed change in proposal 1 is included in the accompanying CR [3].

- References

- RP-232440, “Revised WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, Intel, MediaTek

- R4-2410376, “WF on NR_MG_enh2_part1”, MediaTek, RAN4 #111

- R4-2502239, “CR38.133 Corrections to Case 2 requirements”, Nokia, RAN4 #114

- R4-2419001, “(NR_MG_enh2-Core) CR on 38.133 MG enh2 on NCSGs“, Ericsson, RAN4#113