3GPP TSG-RAN4 Meeting #114	R4-2502256

# Athens, Greece,  16th – 21st February, 2025

Title:	CW phase noise requirement

Source:	Qualcomm Incorporated

Agenda item:	7.24.2.3

Release:	Rel-19

WI: 	Ambient_IoT_Solutions-Core

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

CW provided by the external source puts additional burden on the reader since CW cancellation can not use the same CW thus phase noise is not correlated with the original CW source. In this paper we discuss and propose to define a requirement for the CW phase noise to ensure it does not degrade system performance.

# 2. 	Discussion

CW phase noise contribution was discussed in the study items and for example some aspects were added to the TR [3]  in to at least in sections 6.7.3.3.3 and 6.7.5.2.2.4. In WF [3] phase noise requirement for the CW was agreed to TBD. In the discussion, the phase noise was seen impacting system performance if same phase noise performance as a modern LTE or NR handheld UE  phase noise performance was assumed.  While dedicated CW node is likely to have a better phase noise performance it should be specified as a minimum standard requirement to ensure the system is performing at an expected level.

Proposal: Define phase noise requirement for the CW at least when it is coming from outside of the topology.

Appropriate level for the phase noise requirement can be derived from the analysis in the TR. E.g. Figure 6.7.5.2.2.4-1 provides a good starting point for the values and the impact to SINR. However, RAN4 needs to wait for RAN1 to conclude the frequency domain definition for the D2R signal so that the appropriate offset and observation bandwidth for the noise for the requirement can be determined.

Observation: To define the numerical values, offset from CW and observation BW for the noise requirement, RAN4 needs to wait for frequency domain definitions to be concluded by RAN1

# Conclusion

We discussed the phase noise requirements and made one proposal and one observation:

Proposal: Define phase noise requirement for the CW at least when it is coming from outside of the topology.

Observation: To define the numerical values, offset from CW and observation BW for the noise requirement, RAN4 needs to wait for frequency domain definitions to be concluded by RAN1

# Reference

[1]	RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	R4-2420470 “WF on UE RF requirements for A-IoT”, Huawei, 3GPP TSG-RAN WG4 Meeting # 113, Orlando, United States,18 – 22 Nov., 2024

[3]	TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP