3GPP TSG-RAN WG4 Meeting # 114	R4-2502261

# Athens, GR, 17th – 21th February, 2025

Source:	LG Electronics

Title:	Discussion on RF requirement for device 1

Agenda item:	7.24.2.2

Document for:	Discussion

Introduction

In 3GPP RAN4 #113, WF[1] was approved. In this paper, we provide our views on RF requirement for device 1.

- Issue 3-3: TX(D2R) Agreement: - Whether requirements potentially needed or not is summarized in the following table: A-IoT device TX (D2R) RF Requirement; Potentially needed or not. Device 1; Device 2a; Device 2b. Transmit output power; Maximum output power /output power; NO for maximum output power YES for backscattering power or power backscattering loss; NO for maximum output power YES for backscattering power or power backscattering loss; YES. Output power dynamic; Transmit OFF power; No; NO; YES. Transmit ON/OFF time mask; NO; NO; NO. Minimum output power; N/A; N/A; YES. Power control; NO; NO; YES. Transmit signal quality; Frequency error; NO; TBD in WI phase; YES. Modulation quality requirements; TBD in WI phase; TBD in WI phase; TBD in WI phase. In band emissions; NO; TBD in WI phase; TBD. in WI phase. Carrier leakage; NO; TBD in WI phase; YES. Output RF spectrum emissions; Occupied bandwidth; YES; YES; YES. SEM; TBD in WI phase; TBD in WI phase; TBD in WI phase. ACLR; NO; NO; TBD in WI phase. Spurious emissions; TBD in WI phase; TBD in WI phase; TBD in WI phase. Unwanted emissions; TBD in WI phase; TBD in WI phase; TBD in WI phase. Transmit intermodulation; NO; NO; TBD in WI phase. - Issue 3-4: RX(R2D) Agreement: - Whether requirements potentially needed or not is summarized in the following table: A-IoT device- RX Requirement; Potentially needed or not. Device 1; Device 2a; Device 2b. Reference sensitivity; YES; YES; YES. Maximum input power; TBD in WI phase; TBD in WI phase; TBD in WI phase. ACS; NO; NO; Depend on co-existence study. ACSC; NO; NO; NO. In-band blocking; NO; NO; TBD in WI phase. Out-of-band blocking; NO; NO; TBD in WI phase. Receiver intermodulation; NO; NO; TBD in WI phase. Rx spurious emission; NO; NO; TBD in WI phase. Spurious response; NO; NO; NO. Interference rejection; TBD in WI phase; TBD in WI phase; TBD in WI phase.

Discussion

Backscattering power

Figure1. Device1 backscattering power

Transmitted output power (C) can be defined as follows.

CBackscattering power=ACW EIRP-Ploss air+M-Ploss iot device+Antenna Gain

| [Device 1 Structure] |  |
| --- | --- |

In the above equation, A(CW EIRP) means CW power from CW node and Ploss air means path loss on air. The CW signal passes through the path loss in the air and enters the A-IoT device. At this time, the loss that occurs when experiencing the inside of the A-IoT device is expressed as Ploss_iot_device , and M is defined as implementation margin. Ploss_iot_device  may be defined by referring to the above structure.

Backscaterring power= CW Received power-Ploss_iot_device +Antenna Gain

Ploss_iot_device =2*PBackscattering modulator

The CW received power means the power transmitted from the A-IoT device experiences the path loss in the air and the A-IoT device receives it, and the output power of the A-IoT device can be calculated as the value of adding Ploss_iot_device  (reflection loss) to the CW received power and adding the gain. Reflection loss can be expressed as 2*PBackscattering.

Proposal 1: Backscaterring power of Device 1 can be calculated: 
Backscaterring power= CW Received power-Ploss_iot_device +Antenna Gain

Ploss_iot_device =2*PBackscattering modulator

- 2. Conclusion

In this contribution, we want to share some views on Ambient IoT device1 for RF requirements and proposals are made as following:

Proposal 1: Backscaterring power of Device 1 can be calculated: 
TBackscaterring power= CW Received power-Ploss_iot_device +Antenna Gain

Ploss_iot_device =2*PBackscattering modulator

- 3. Reference

[1] R4-2420470, “WF on RF requirements for A-IoT”

[2] RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.