3GPP TSG-RAN WG4 Meeting #114	R4-2502281

Athens, Greece, February 17 – 21, 2025

Title: 	Way Forward for [113][310] NR_NTN_Ph3_UE_RF

Agenda Item:	7.29.5

Source: 	Qualcomm Inc.

Document for:	Approval

## Issue 1-1: PC3 output power for (e)RedCap

Agreement:

  - Rel-18 power boosting is applicable for NR NTN including (e)RedCap

    - No reference sensitivity specification impact (e.g. RSD) from power boosting.

  - No changes to the PC3 tolerance for HD-FDD and FD-FDD compared non-RedCap UE NR NTN

  - FFS on solutions on additional new output power enhancement

    - supported modulation orders

    - potential downselection between HD-FDD and FD-FDD

    - simultaneous support with rel-18 power boosting

    - Background: output power enhancement (e.g., FE IL improvement)

    - Other solutions not precluded

Note: No change for (e)RedCap MOP tolerance to enhance the output power does not preclude any solution in other Rel-19 WI.

## Issue 1-2: HD-FDD refsens for 2 Rx

Agreement:

- Reference sensitivity for 2Rx HD-FDD for n255 and n254 is

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
|  | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n254 | 15 | -100.0 | -96.8 | -95.0 |  |
|  | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

Note: This agreement does not preclude improved reference sensitivity for other bands

## Issue 1-3: Frequency error

Agreement:

The NR NTN non-RedCap UE frequency error requirements are reused for NTN (e)RedCap UE

## Issue 1-4: Release independence

Agreement:

As a starting point in case no new RAN1/RAN2 impact is identified:

- FD-FDD RedCap can be release independent from Rel-17

- FD-FDD eRedCap can be release independent from Rel-18

- HD-FDD (e)RedCap can be release independent from Rel-19