3GPP TSG-RAN WG4 Meeting #114	R4-2502353

# Athens, Greece, 17th February – 21st February 2025                      (revision of R4-2502090)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.108 | CR | 0030 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TEI18) CR to 36.108 NB-IoT In-band operation with NTN NR [NTNNBIoT_inbandNTNNR] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | THALES, EchoStar, Inmarsat, Viasat |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NB-IoT NTN only supports stand-alone operation but is lack of In-band NTN NR operation. There is strong NTN industry demand to enable NB-IoT NTN In-band operation with NTN NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of NB-IoT SAN support for in-band operation in NTN NR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-IoT SAN RF requirements will not be suitable to support NB-IoT In-band operation in NTN NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 3.1, 5.3B, 5.4B.2, 6.1, 6.3.1, 6.5.4.2, 7.1, 7.2.2, 9.1, 9.4.1, 9.6.4.1, 10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.181 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 1	Scope

The present document establishes the minimum RF characteristics and minimum performance requirements of Satellite Access Node (SAN) supporting standalone NB-IoT operation or E-UTRA or NB-IoT operation in NTN NR in-band.

NOTE: Minimum requirements for NB-IoT operation in NTN NR guardband are not specified and guardband operation is SAN implementation specific.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[3]	3GPP TS 36.181: "Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node conformance testing; Satellite Node conformance testing".

[4]	ITU-R Recommendation M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[5]	ITU-R Recommendation SM.328: "Spectra and bandwidth of emissions".

[6]	ITU-R Recommendation SM.1541-6: "Unwanted emissions in the out-of-band domain".

[7]	3GPP TS 38.108: “Satellite Access Node radio transmission and reception”.

[8]	void

[9]	3GPP TS 36.211: Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”

[10]	3GPP TR 38 811: "Study on New Radio (NR) to support non-terrestrial networks".

[11]	3GPP TS 36 102: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access".

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<< Unchanged text skipped>>

measurement bandwidth: RF bandwidth in which an emission level is specified.

minSENS: the lowest declared EIS value for the OSDD's declared for OTA sensitivity requirement.

minSENS RoAoA: The reference RoAoA associated with the OSDD with the lowest declared EIS.

minimum elevation angle: Minimum angle under which the satellite can be seen by a UE.

NB-IoT operation in NTN NR in-band: NB-IoT is operating in-band when it utilizes the resource block(s) within a normal NTN NR carrier.

NB-IoT operation in NTN NR guard band: NB-IoT is operating in guard band when it utilizes the unused resource block(s) within a NTN NR carrier's guard-band.

NB-IoT standalone operation: NTN NB-IoT is operating standalone when it utilizes its own spectrum.

necessary bandwidth: The width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions.

<< Unchanged text skipped>>

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

## 5.3B	Channel bandwidth for category NB1 and NB2

For category NB1 and NB2, requirements in present document are specified for the channel bandwidth listed in Table 5.3B-1.

Table 5.3B-1: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB1 and NB2 channel bandwidth

| NB-IoT | Standalone | NB-IoT operation in NTN NR in-band |
| --- | --- | --- |
| Channel bandwidth BWChannel (kHz) | 200 | NR channel bandwidth in Table 5.3.2-1 according to TS 38.108 |
| Transmission bandwidth configuration NRB | 1 | 1 |
| Transmission bandwidth configuration Ntone 15kHz | 12 | 12 |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 | 48 |

Figure 5.3B-1 shows the relation between the category NB1/NB2 SAN channel bandwidth and the Category NB1 /NB2 transmission bandwidth configuration (Ntone 15kHz, or Ntone 3.75kHz). The channel edges are defined as the lowest and highest frequencies of the carrier separated by the channel bandwidth, i.e. at FC +/- BWChannel /2.

Figure 5.3B-1 Definition of channel bandwidth and transmission bandwidth configuration

For NB-IoT standalone operation, NB-IoT requirements for receiver and transmitter shall apply with a frequency offset Foffset as defined in Table 5.3B-1.

Table 5.3B-1: Foffset for NB-IoT standalone operation

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Standalone NB-IoT | [200 kHz] |

For NB-IoT in-band operation in NTN NR, figure 5.3B-2 shows the relation between the channel bandwidth (BWChannel) and the transmission bandwidth configuration (NRB, Ntone 15kHz and Ntone 3.75kHz). The channel edges are defined as the lowest and highest frequencies of the carrier separated by the channel bandwidth, i.e. at FC +/- BWChannel /2.

Figure 5.3B-2 Definition of Channel Bandwidth and Transmission Bandwidth Configuration for NB-IoT operation in NTN NR in-band

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 5.4B.2	Channel raster, carrier frequency and EARFCN

The channel raster of UE category NB1/NB2 shall be as defined in clause 5.4A.2, and the channel raster per-frequency band shall be as defined in table 5.4A.2-1.

The carrier frequency of UE category NB1/NB2 in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, -0.5, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.4A.2-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL)

The carrier frequency of UE category NB1/NB2 in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.4A.2-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

NOTE 1:	MDL and MUL for NTN NB-IoT operation in NTN NR in-band and the equation showing the relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for DL and UL area specified in TS 36.102 [11], Clause 5.4B.2.3.

NOTE 2:	For the carrier including NPSS/NSSS for stand-alone operation, MDL = 0.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

## 6.1	General

Unless otherwise stated, the conducted transmitter characteristics are specified at the TAB connector for SAN type 1-H, with a full complement of transceiver units for the configuration in normal operating conditions.

The requirements for SAN output power, frequency error, EVM and DL RS power are identical for NB-IoT standalone mode and NB-IoT operation in NTN NR in-band, for other requirements for NB-IoT operation in NTN NR in-band are defined in TS 38.108.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 6.3.1	General

The requirements in clause 6.3 apply during the transmitter ON period. Transmit signal quality (as specified in clause 6.5) shall be maintained for the output power dynamics requirements of this Clause.

Power control is used to limit the interference level.

The requirements in clause 6.3 does not apply for NB-IoT standalone mode and NB-IoT operation in NTN NR in-band mode.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.5.4.2	Minimum requirements

For E-UTRA, DL RS power of each E-UTRA carrier shall be within  2.1 dB of the DL RS power indicated on the DL-SCH.

For NB-IoT, DL NRS power of each NB-IoT carrier shall be within  2.1 dB of the DL NRS power indicated on the DL-SCH.

The DL NRS power accuracy requirement in clause 6.5.4.2 is applicable for both NB-IoT standalone mode and NB-IoT operation in NTN NR in-band mode.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

# 7	Conducted receiver characteristics

## 7.1	General

Conducted receiver characteristics are specified at the TAB connector for SAN type 1-H, with full complement of transceivers for the configuration in normal operating condition.

Unless otherwise stated, the following arrangements apply for conducted receiver characteristics requirements in clause 7:

-	Requirements shall be met for any transmitter setting.

-	The requirements shall be met with the transmitter unit(s) ON.

-	Throughput requirements do not assume HARQ retransmissions.

-	When SAN is configured to receive multiple carriers, all the throughput requirements are applicable for each received carrier.

-	For ACS and blocking characteristics, the negative offsets of the interfering signal apply relative to the lower SAN RF Bandwidth edge or sub-block edge inside a sub-block gap, and the positive offsets of the interfering signal apply relative to the upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap.

NOTE 1:	In normal operating condition the SAN is configured to transmit and receive at the same time.

NOTE 2: The requirement for reference sensitivity level is identical for NB-IoT standalone mode and NB-IoT operation in NTN NR in-band, for other requirements for NB-IoT operation in NTN NR in-band are defined in TS 38.108.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

## 7.2	Reference sensitivity level

### 7.2.1	General

The reference sensitivity power level PREFSENS is the minimum mean power received at the TAB connector for SAN type 1-H at which a throughput requirement shall be met for a specified reference measurement channel.

### 7.2.2	Minimum requirements for SAN type 1-H

For SAN supporting E-UTRA, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.2-1 and 7.2.2-2 for SAN type 1-H in all operating band.

Table 7.2.2-1: Reference sensitivity levels of SAN supporting E-UTRA (GEO class payload)

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (NOTE) | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 1.4 | 15 | FRC A1-1 in Annex A.1 | -104.4 |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. |  |  |  |

Table 7.2.2-2: Reference sensitivity levels of SAN supporting E-UTRA (LEO class payload)

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (NOTE) | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 1.4 | 15 | FRC A1-1 in Annex A.1 | -107.5 |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. |  |  |  |

For SAN supporting standalone NB-IoT operation or NB-IoT operation in NTN NR in-band, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.1 with parameters specified in Table 7.2.2-3 and 7.2.2-4 for SAN type 1-H in all operating band.

Table 7.2.2-3: Reference sensitivity levels of SAN supporting standalone NB-IoT operation and NB-IoT operation in NTN NR in-band (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 |

Table 7.2.2-4: Reference sensitivity levels of SAN supporting standalone NB-IoT operation and NB-IoT operation in NTN NR in-band (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

# 9	Radiated transmitter characteristics

## 9.1	General

Radiated transmitter characteristics requirements apply on the SAN type 1-H or SAN type 1-O including all its functional components active and for all foreseen modes of operation of the SAN unless otherwise stated.

The requirements for SAN output power, frequency error, EVM and DL RS power are identical for NB-IoT standalone mode and NB-IoT operation in NTN NR in-band, for other requirements for NB-IoT operation in NTN NR in-band are defined in TS 38.108.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 9.4.1	General

Transmit signal quality (as specified in clause 9.6) shall be maintained for the output power dynamics requirements.

The OTA output power requirements are directional requirements and apply to the beam peak directions over the OTA peak directions set.

The requirements in clause 9.4 does not apply for NB-IoT standalone mode and NB-IoT operation in NTN NR in-band mode.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 9.6.4.1	Minimum requirements for SAN type 1-O

For SAN type 1-O, the OTA absolute accuracy for DL RS power shall be the same as clause 6.5.4.1.

The DL NRS power accuracy requirement in clause 6.5.4.2 is applicable for both NB-IoT standalone mode and NB-IoT operation in NTN NR in-band mode.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 10.3.2	Minimum requirement for SAN type 1-O

For SAN supporting E-UTRA, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameter specified in table 10.3.2-1 and table 10.3.2-2 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

Table 10.3.2-1: Reference sensitivity levels of SAN supporting E-UTRA (GEO class payload)

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (NOTE) | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 1.4 | 15 | FRC A1-1 in Annex A.1 | -104.4 - ΔOTAREFSENS |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. |  |  |  |

Table 10.3.2-2: Reference sensitivity levels of SAN supporting E-UTRA (LEO class payload)

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (NOTE) | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 1.4 | 15 | FRC A1-1 in Annex A.1 | -107.5 - ΔOTAREFSENS |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. |  |  |  |

For SAN supporting standalone NB-IoT operation and NB-IoT operation in NTN NR in-band, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameter specified in table 10.3.2-3 and table 10.3.2-4 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

Table 10.3.2-3: Reference sensitivity levels of SAN supporting standalone NB-IoT operation and NB-IoT operation in NTN NR in-band (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 - ΔOTAREFSENS |

Table10.3.2-4: Reference sensitivity levels of SAN supporting standalone NB-IoT operation and NB-IoT operation in NTN NR in-band (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 - ΔOTAREFSENS |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------