3GPP TSG-RAN WG4 Meeting #114	R4-2502360

# Athens, GR, 17 - 21 Feb, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0741 | rev | 1 | Current version: | 15.24.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Perf) CR on UE CSI requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. For FR1 aperiodic PMI reporting requirements, there is conflict between the Note 2 and the CQI/RI/PMI delay. 2. For FR2 aperiodic CSI reporting requirements with DDSU TDD pattern, there is conflict between the TS 38.214 and the CQI/RI/PMI delay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify CQI/RI/PMI delay to satisfy Note 2 and TS 38.214. See R4-2418944 for details. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsist between specification and RAN4 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <START OF THE CHANGE 1>

## 6.3	Reporting of Precoding Matrix Indicator (PMI)

The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reported PMI compared to the case when the transmitter is using random precoding, respectively. When the transmitter uses random precoding, for each PDSCH allocation a precoder is randomly generated with equal propability of each applicable i1 and i2 combination and applied to the PDSCH. A fixed transport format (FRC) is configured for all requirements.

The requirements for transmission mode 1 with higher layer parameter codebookType set to 'typeI-SinglePanel' are specified in terms of the ratio:

In the definition of γ, for 4TX and 8TX PMI requirements,  is 90 % of the maximum throughput obtained at using the precoders configured according to the UE reports, and is the throughput measured at with random precoding.

### 6.3.1	1RX requirements

(Void)

### 6.3.2	2RX requirements

#### 6.3.2.1	FDD

##### 6.3.2.1.1	Single PMI with 4TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.2.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.1-2.

Table 6.3.2.1.1-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 2 (N1,N2) = (2,1) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (9) |
|  | CSI-RS periodicity and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 4, (0) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (13) |
|  | CSI-RS periodicity and offset |  | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 4 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 7 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.1 FDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity. Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-3), this reported PMI cannot be applied at the gNB downlink before slot#(n+3). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |

Table 6.3.2.1.1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.3 |

##### 6.3.2.1.2	Single PMI with 8TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.2.1.2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.2-2.

T Table 6.3.2.1.2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 2 (N1,N2) = (4,1) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (9) |
|  | CSI-RS periodicity and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (5) |
|  | CSI-RS periodicity and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 9 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.2 |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity. Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |

Table 6.3.2.1.2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.5 |

#### 6.3.2.2	TDD

##### 6.3.2.2.1	Single PMI with 4TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.2.2.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.1-2.

Table 6.3.2.2.1-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configuration |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 2 (N1,N2) = (2,1) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (9 ) |
|  | CSI-RS periodicity and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 4, (0) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (13) |
|  | CSI-RS periodicity and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.1 TDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity. Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot #n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |

Table 6.3.2.2.1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.3 |

##### 6.3.2.2.2	Single PMI with 8TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.2.2.2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.2-2.

Table 6.3.2.2.2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 2 (N1,N2) = (4,1) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (9) |
|  | CSI-RS periodicity and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (5) |
|  | CSI-RS periodicity and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 7 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.2 TDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity. Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |

Table 6.3.2.2.2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.5 |

### 6.3.3	4RX requirements

#### 6.3.3.1	FDD

##### 6.3.3.1.1	Single PMI with 4TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.3.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.1-2.

Table 6.3.3.1.1-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 4 (N1,N2) = (2,1) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (9) |
|  | CSI-RS periodicity and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 4, (0) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (13) |
|  | CSI-RS periodicity and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 4 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 7 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.1 FDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity. Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-3), this reported PMI cannot be applied at the gNB downlink before slot#(n+3). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |

Table 6.3.3.1.1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.3 |

##### 6.3.3.1.2	Single PMI with 8TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.3.1.2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.2-2.

Table 6.3.3.1.2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 4 (N1,N2) = (4,1) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (9) |
|  | CSI-RS periodicity and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (5) |
|  | CSI-RS periodicity and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 9 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.2 FDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity. Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |

Table 6.3.3.1.2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.5 |

#### 6.3.3.2	TDD

##### 6.3.3.2.1	Single PMI with 4TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.3.2.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.2.1-2.

Table 6.3.3.2.1-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configuration |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 4 (N1,N2) = (2,1) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (9 ) |
|  | CSI-RS periodicity and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 4, (0) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (13) |
|  | CSI-RS periodicity and offset |  | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.1 TDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity. Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |

Table 6.3.3.2.1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.3 |

##### 6.3.3.2.2	Single PMI with 8TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.3.2.2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.2.2-2.

Table 6.3.3.2.2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 4 (N1,N2) = (4,1) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (9) |
|  | CSI-RS periodicity and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (5) |
|  | CSI-RS periodicity and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannnelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 7 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.2 TDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity. Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |

Table 6.3.3.2.2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.5 |

# <END OF THE CHANGE 1>