3GPP TSG-RAN WG4 Meeting #114												 		    R4-2502581

Athens, Greece, Feb 17th – 21st, 2025

Source:	MediaTek

Title:	WF on RRM requirements for NR_MG_enh2

Document for:	Approval

# Topic #1: Remaining maintenance issues

### Sub-topic 1-1: Rel-18 UE behavior for deactivated SCell measurements with NCSG

Issue 1-1-1: When the UE is configured with Concurrent gaps with NCSG, whether all deactivated SCells are measured within NCSG, regardless the gap association?

- Proposal:

  - Option 1: [ZTE, CMCC, HW, E///, MTK, Nokia, OPPO]

    - For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities and gap association.

- Recommended WF

  - Update existing agreement: Remove the brackets from the existing agreement:

    - For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association].

Issue 1-1-2: When the measurement of the measurement object is related to a deactivated SCell, and after this SCell is activated, and when UE is configured with the concurrent measurement gap and NCSG, what is the expected UE behaviour?

Agreement:

- When the measurement of the measurement object is related to a deactivated SCell, and after this deactivated Scell is activated, and when UE is configured with the concurrent measurement gap and NCSG, the UE shall follow the gap association rule (i.e., association between measurement object and certain measurement gap) to perform measurement for that SCell.

Issue 1-1-3: When the following text should be added to clause 9.1.13.2 related to measuring deactivated SCells?

Agreement:

Wording is agreed in CRs drafting and this issue is closed.

### Sub-topic 1-2: DRX in NFG operation

Issue 1-2-1: Misalignment between DRX-on duration and SMTC for NFG measurements?

- Background:

  - Agreement from RAN4#112 meeting:

    - Interruptions are not allowed during DRX ON duration

      - Exclude the time extended due to drx-inactivityTimer

        - if there is no SMTC occasion within a time period starting [4ms] before the starting point of the DRX ON duration and ending [4ms] after the ending point of the DRX ON duration.

Agreement:

- Revise the value of the time period before and after the DRX-ON cycle for which the presence of SMTC allows interruptions during the DRX-ON period to 2 ms.

------- End of Document -------