3GPP TSG-RAN WG4 Meeting #113	R4-2502595

Athens, GR, Feb.17 – Feb.21, 2025

Title: 	WF on RRM requirements for NR_MIMO_Ph5_Part1

Agenda Item:	7.19.4

Source: 	Samsung

Document for:	Approval

<Topic 1: RRM Requirement of Enhancement for UE-initiated/event-driven beam management >

Issue 1-13: Whether to enhance beam sweeping in event triggered L1 beam reporting?

Agreement: Reuse current beam sweeping factor for UE/event triggered beam reporting.

Issue 1-1: assumption or applicability of event triggered L1 measurement:

<Way forward >

- Option 1: (vivo)

  - Follow the assumption:

    - the condition which triggers the event is remained fixed during the time UE performs L1 measurements.

    - the condition which triggers the event is remained fixed during the time after UE sends the first uplink transmission and before UE is scheduled with the second uplink transmission for the L1 reporting

    - during the time UE performs L1 measurements, no change on the indicated TCI state of current serving cell

    - during the time UE performs L1 measurements, no change on the to-be-measured RS of the new beam.

- Option 2: others are not precluded

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1)?

Agreement:

The agreements under this issue is for event 2, and further discuss whether these agreements can be applicable to other events agreed in RAN1.

TL1-RSRP_event triggered_measurement_Period_SSB or TL1-RSRP_event triggered_measurement_Period_CSI-RS

  - update based on 9.5.4.1 (SSB based), 9.5.4.2 (CSI-RS based) and 9.13.4

    - TReport = 0 or be removed from the measurement period

    - N: Reuse the legacy definition

    - M: FFS pending RAN1 further agreement

    - TSSB (taking SSB as example)

      - For the same periodicity supported in RAN1, TSSB_current, or TSSB_new

      - RAN4 to further discuss the case of different periodicities if different periodicities will be [supported or not excluded] in RAN1

        - Option A: Max{TSSB_current, TSSB_new}

        - Option B: When the RS periodicity is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new.

        - Option C: others are not precluded

Issue 1-3: Same or different periodicities of current beam and new beam(s)?

<Way forward >

- Option 1: (Qualcomm, Apple, CMCC, Samsung, vivo)

  - The same periodicities of current beam and new beam(s) to define the requirements, until further RAN1 agreements on different periodicities.

  - According to RAN1 LS (R1-2410914)

- Option 2: (Huawei)

  - From RAN4 perspective, the periodicity of current beam and new beam(s) can be the same or different which is up to network configurations. Send LS to RAN1

- Option 3: (vivo)

  - RAN4 may further discuss RRM requirements for Alt-2 and the potential restrictions to the requirement applicability if time allows, based on RAN1 further agreements.

Issue 1-4: Components of Event triggered measurement reporting delay?

<Way forward >

- Option 1: (Apple, vivo, Samsung, E///, MTK)

  - The event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst UL channel

    - FFS on whether other components are needed.

    - FFS the definition of Tfirst UL channel

- Option 2: (CMCC, Xiaomi)

  - The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering.

- Option 3: (Nokia)

  - The UE shall be able to send the first PUCCH latest at the PUCCH occasion that is available after the maximum L1 event evaluation delay (TL1-RSRP_measure_period).

Issue 1-6: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with configurable M?

<Way forward >

- Option 1: (Qualcomm, Apple, Ericsson)

  - Only define requirements for the number of events is met at once

- Option 2: (China Telecom, ZTE, Huawei, Samsung, MediaTek)

  - Specify requirements to cover both cases of no time window based and with time window based with configurable M

- Option 2a: (ZTE)

  - Event1&Event 2: Case 2: multiple samples M and timer X should be considered

  - Event1&Event 7: FFS

- Option 2b (Samsung)

  - For within a time window configured case, the L1 evaluation time is defined as multiple times of L1 evaluation time in basic to fulfil M counter is met for the event.

- Option 2c (MediaTek)

  - For multiple events is met by counter:

    - Tevaluate_L1_event = TL1-RSRP_measure * C + Tuncertainty given that Tevaluate_L1_event ≤ Ttime_window

    - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS

    - C: event instance counter is configured by NW.

Issue 1-7: Whether/how to specify the UE behaviour if new L1-RSRP during the UL resource waiting time?

<Way forward >

- Option 1: (Xiaomi)

  - For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting.

- Option 2: (LGE, Nokia)

  - Need to wait RAN1 progress

- Option 3: (vivo)

  - Follow the assumption: the condition which triggers the event is remained fixed during the time after UE sends the first uplink transmission and before UE is scheduled with the second uplink transmission for the L1 reporting. FFS on other cases if time allows

Issue 1-9: CSI-RS based L1-RSRP for inter-cell BM

<Way forward >

- Option 1: (Qualcomm, Nokia, MediaTek(descoped))

  - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

- Option 2: (Apple, Huawei, Ericsson)

  - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

  - Option 2a (Apple): The measurement delay is the same as that in serving cell in legacy L1-RSRP

  - Option 2b (Ericsson): reuse the principle of sharing agreed for SSB based measurement.

- Option 3: (ZTE)

  - RAN4 needs to identify whether additional clarifications on the following points are necessary:

    - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

    - The prerequisite on L3 measurement on NSC.

    - RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- Option 4: (Ericsson)

  - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2

  - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements

Issue 1-10: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM

Agreement:

- Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

Issue 1-11: Whether to specify new unified TCI switching delay requirements?

<Way forward >

- Option 1: (Qualcomm, Apple, China Telecom, Samsung (For known conditions for TCI states), Ericsson, MediaTek)

  - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

- Option 2: (Xiaomi, Nokia)

  - Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress.

- Option 3: (LGE)

  - For unified TCI switching delay requirements, RAN4 needs to discuss whether active TCI state switching delay can be reduced or not

- Option 4: (ZTE)

  - For MAC-CE based DL TCI switching delay: no need to consider TL1-RSRP and Tfirst-SSB+TSSB-proc

- Option 5: (Samsung)

  - Not consider unknown case.

Issue 1-12: Other conditions for L1 measurement and TCI state switching?

<Way forward >

- Option 1: (Samsung)

  - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI.

Issue 1-14: Capabilities for Support of Event Triggering and Reporting Criteria?

<Way forward >

- Option 1: (Apple, Ericsson)

  - Defined by RAN1. To be discussed later if any RAN4 specific UE capabilities need to be added.

- Option 2: (China Telecom)

  - Align the agreement on whether to define the capability for support of L1 event triggering and reporting in TS 38.133 for R19 MIMO WI and R19 mobility WI.

- Option 3: (CMCC, MediaTek)

  - Define capabilities similar as legacy L3 (like legacy L3 in section 9.1.4).

Issue 1-15: Compatibility of capabilities with LTM

<Way forward >

- Option 1: (Apple, Ericsson)

  - Discuss it later.

- Option 2: (CMCC, Huawei, Nokia, Samsung)

  - Do not mix/combine Rel-19 mobility and Rel-19 MIMO. Define the capability separately.

- Option 4: (MediaTek)

  - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

Issue 1-16: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

Agreement

  - The existing accuracy requirements for L1-RSRP measurements can be applied. Not to define new accuracy requirements.