3GPP TSG-RAN WG4 Meeting #114											     R4-2502596

Athens, Greece, February 17 – 21, 2025

Title: 	WF on RRM requirements for NR_MIMO_Ph5_Part2

Agenda Item:	7.22.4

Source: 	MediaTek inc.

Document for:	Approval

# <Topic 1: UE reporting enhancement for CJT calibration>

Issue 1-1: How to decide on the number of samples used for CJT calibration?

Agreement:

- RAN4 to decide on the number of samples based on its impact on the reported interval (granularity of the report) for the given report resolution, while using the following parameters

  - Report resolution (MD = 128, MFO = 128).

  - Offset range (AD and AFO)

    - AD = CP

    - AFO = 0.2 ppm

  - Time offset between the two TRPs: 0.5 CP

  - Frequency offset between the two TRPs: 0.1 ppm

  - Note that report resolution and offset range are network configuration via RRC signalling.

- Simulation results can be provided based on the CDF accuracy at 5%, 50% and 95%.

# <Topic 2: Enhancement for asymmetric DL sTRP/UL mTRP scenarios>

Issue 2-1: Whether RAN4 need to consider the scenario when UE is/isn’t configured with pathloss offset?

Agreement:

For Scenario#1: RAN4 define the timing requirements for FR1, FFS on FR2.

For Scenario#2, FFS on whether to reuse the legacy timing requirements (Rel-18 m-DCI two TA with updated or not).

RAN4 common understanding: the RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs.” are currently only applied for Scenario #1.

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s).

Note 1: the scenario 1 and 2 are based on RAN1 LS.

Note 2: The above agreements apply when 2TA are configured.

Issue 2-2: How to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Agreement:

- For scenario 1, for both TAGs, the uplink transmission(s) timing for PUCCH/PUSCH/SRS take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state.

  - The above agreement can be revisited if not aligned with RAN1 further agreement.

Issue 2-3: Whether to define UL TCI state activation requirement based on SRS?

Way forward:

Further discuss the two proposals and down-select one proposal in the next meeting:

  - Proposal 1 (Qualcomm, Huawei, MediaTek, Samsung, Apple, QC): No RRM impacts on TCI state switching requirement.

  - Proposal 3 (Xiaomi, Nokia):

    - Legacy DCI based UL TCI state activation requirement can be re-used for UL TRP.

    - RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI. The legacy MAC CE based delay requirement can be used as baseline.

    - For DL-TRP, legacy UL TCI state activation requirement based on DL-RS can be re-used. no need to define SRS based ULTCI state activation delay.

    - Further refine the wording for P3 in the WF, if needed