3GPP TSG-RAN WG4 Meeting #114	 														R4-2502601

Athens, Greece, 17th – 21st February, 2025

Agenda item:			7.27.3

Source:	Apple

Title:	WF on RRM requirements for NR_Mob_Ph4_Part1

Document for:	Approval

# Topic #1: General aspects and work plan

#### Issue 1-1: scope of R19 NR mobility enhancements Phase 4

Candidate options:

- Proposals: Not to consider MCG change with SCG change in RAN4 in R19. (MTK)

# Topic #2: Event triggered L1 measurement reporting

#### Issue 2-1: applicability of L1-RSRP reporting delay

Candidate options:

- Option 1: (vivo)

  - RAN4 starts the discussion on L1-RSRP reporting delay with the follow assumptions:

    - No indicate TCI change during the evaluation period.

    - No configuration change w.r.t measurement RS during the evaluation period

    - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

#### Issue 2-2: Event triggered reporting delay for L1 reporting

Agreement:

- Except for periodic MR:

  - Option 1: The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

  - Option 2: The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event.

    - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

- For periodic MR:

  - Option 1: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR.

  - Option 2: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event.

    - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR.

#### Issue 2-3: impact of time-to-trigger (TTT) on RRM

Agreement:

- No need to explicitly capture TTT in core requirements.

#### Issue 2-4: whether to introduce/update requirements for reporting criteria per measurement category

Candidate options:

- Option 1: Yes (Xiaomi, CMCC, CATT, MTK, ZTE, OPPO, CTC, E///)

- Option 1a: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO)

- Option 1b: Number of supported L1 events is to be added on top of the currently supported L3 events. Details FFS. (Nokia)

- Option 2: Open to let RAN1 discuss the number of events supported by UE at first. (MTK)

- Option 2a: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo)

#### Issue 2-5: whether to define a separate capability for supported number of events triggered L1 measurement reporting

Candidate options:

- Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. ([Xiaomi?], ZTE, CMCC, CTC)

- Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///)

#### Issue 2-6: how to define requirements for reporting criteria per measurement category

Candidate options:

- Proposal 1: Regarding whether to count LTM event in a per-layer basis, RAN4 shall wait for further RAN1/2 conclusions regarding the detailed resource configuration for event-triggered L1-RSRP reporting in R19 LTM. (vivo)

#### Issue 2-7: whether and how to coordinate with R19 MIMO

Candidate options:

- Option 1: RAN4 shall develop the two features independently given that UE is allowed to only support one of the two features. (Nokia) Optimization such as joint consideration of UE capabilities can be discussed at later stage. (Apple, CMCC, HW, CTC)

#### Issue 2-8: L1 filtering assumption in event evaluation

Agreement:

- RAN4 should start work assuming no L1 filtering. FFS whether to define requirement with L1 filtering assumed, taking into account the progress in RAN1/2.

#### Issue 2-9: Impact of UL resource waiting time.

Agreement:

- For event triggered L1 measurement reporting, measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting.

#### Issue 2-10: Impact on other LTM requirements

Tentative agreement:

- UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, [and leaving condition of the event is configured and corresponding condition is not met], the related cell switch delay and TCI state activation requirements are applicable.

  - Further discuss how to capture this in RRM spec.

Other proposals for further discussion:

- Proposal:	RAN4 to define TCI state known condition considering the L3 and L1 event triggered reporting.

# Topic #3: CSI-RS based L1 measurement

#### Issue 4-1: Known cell definition in CSI-RS based L1 measurement

Issue 4-1-1: Whether to use detectable or measurable?

Tentative agreement:

  - Use measurable and specify the side condition for RS being measurable.

Issue 4-1-2: Side condition for CSI-RS based L1 measurement for neighbor cell

Candidate options:

  - Option 1: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB (Nokia)

  - Option 2: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Xiaomi, MTK, OPPO, Huawei, Ericsson)

#### Issue 4-2: Measurement procedure

- Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements:

  - Step 1: SSB based L3 measurements and reports at least before LTM configuration.

  - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

    - Further discuss whether Step 2 can be skipped under certain conditions or based on UE capability or based on network RRC configuration. FFS the details.

    - Note: Step 2 is removed for FR1.

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

    - Depending on whether step 2 is skipped, the requirements in step 3 may be different.

  - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

    - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

The following is concluded in RAN4#113 meeting.

Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2

- Option A: Whether step 2 can be skipped or not can be based on UE capability reporting. (Apple, CATT, Nokia, Ericsson, Qualcomm, CMCC - compromise, ZTE, China Telecom, vivo, Xiaomi, OPPO)

- Option B: Always keep step 2 for FR2-1 (MTK, HW)

- Option C: Remove L1 measurement for FR2-1 in the 2nd bullet. (CMCC, OPPO)

#### Issue 4-3: Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement

Candidate options:

  - Option 1: (Apple, ZTE, MTK, CMCC, Huawei, Ericsson)

A measurement is defined as a CSI-RS based intra-frequency measurement provided that: 
- the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and
- the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and
       - It is applied for SCS = 60KHz
- the center frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources within DL BWP (if configured).

  - Option 2: (Qualcomm, MTK, Nokia, E///, vivo, China Telecom, HW, ZTE, CATT)
A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:
- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

#### Issue 4-4: Measurement type

#### Issue 4-7: RTD assumption

Candidate options:

- Option 1: CMCC, ZTE, CATT, China Telecom

- Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature.

- Define requirements for RTD > CP, as optional UE feature. FFS details

- Option 2: E///, HW, CATT, Apple - compromise, CMCC - compromise

- Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature.

- No Ran4 requirements for RTD > CP.

- Option 3:

- Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell.

- Define requirements for support RTD > CP, as optional UE feature. FFS details

- Option 4: QC, Apple, vivo, Xiaomi, Samsung, MTK, OPPO, Nokia - compromise, E/// - compromise

- Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell.

- No Ran4 requirements for RTD > CP.

Agreement:

- Conclude this issue in the next meeting.

#### Issue 4-5: Definition of “inside/outside active BWP”

Candidate options:

  - Option 1: (Apple)

The CSI-RS of candidate cell should be considered inside the active BWP if all the resources are confined within the BWP. Otherwise, it is outside of active BWP.

  - Option 2: (CATT)

RAN4 to firstly align the understanding of ‘the CSI-RS of candidate cell outside active BWP’ among companies. It is easier to define what ‘the CSI-RS of candidate cell within active BWP’ is, and all cases except for ‘within’ are considered as ‘the CSI-RS of candidate cell outside active BWP’.

  - Option 3: (vivo)

From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM:

Option 1: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP.

Option 2: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE.

Option 3: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

#### Issue 4-6: Measurement on CSI-RS with repetition ON/OFF

Candidate options:

  - Option 1: (CATT, CT, vivo)

In FR2, RAN4 will not define the requirements for CSI-RS resources with repetition ON for L1 measurement on neighbor cell. 
In FR2, RAN4 will define the requirements for the following case: 
CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB.

  - Option 2: (Huawei)

Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell.

  - Option 3: (Nokia)

Wait until RAN1 has agreed to support repetition “ON” configuration before agreeing to define requirements. As a baseline, define requirements for the case where the repetition is ‘OFF’. Revisit the agreement if RAN1 decides to support “ON” configuration.

  - Option 4: (Ericsson, Apple, Nokia)

RAN4 to wait for RAN1 conclusion on support of CSI-RS with repetition ON.

#### Issue 4-8: CSI-RS resource type

Candidate options:

  - Option 1: (Apple, OPPO, Nokia, vivo)

RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS and FFS on aperiodic and semi-persistent CSI-RS pending RAN1 decision.

  - Option 2: (CMCC, Ericsson)
Semi-persistent CSI-RS resource for LTM candidate cells is supported.

  - Option 3: (CATT)

RAN4 to define CSI-RS based L1 measurement for periodic and semi-persistent CSI-RS.

#### Issue 4-9: Measurement gap related

Candidate options:

  - Option 1: (Apple)

Decide measurement GAP after RAN4 conclude measurement type. If GAP is needed, then consider type 1 GAP as minimum. Other GAP types are FFS.

  - Option 2: (CATT)

In Rel-19 LTM, if gap is introduced, at least the type 1 MG should be supported for CSI-RS based L1 measurement.

RAN4 at least can follow the same approach as CSI-RS based L3 measurement for gap-based measurement, i.e., requirements applicability for CSI-RS based L1 measurement will be defined to ensure existing gap can cover the CSI-RS configuration in the same frequency layer.

For CSI-RS based L1 with gap measurement, if RAN4 also agree to consider semi-persistent CSI-RS transmission and measurements, may be pre-MG can be introduced for semi-persistent CSI-RS measurements.

  - Option 3: (Ericsson)

RAN4 to define the gap-based CSI-RS L1-RSRP measurement which do not impact the legacy SSB based L1-/L3-RSRP measurement within gap.

RAN4 to define single gap-based CSI-RS L1-RSRP requirement first. RAN4 can further discuss concurrent gaps-based CSI-RS L1-RSRP requirement if time allows.

#### Issue 4-10: Restriction on CSI-RS resource configuration

Agreement:

- In FR1, when CSI-RS based L1 measurement does not cause scheduling restriction, no need to consider restriction on measurement timing configuration.

- In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction:

  - All CSI-RS resources on [one intra-frequency layer] are configured within up to two separate windows where each window is up to [5 ms]

#### Issue 4-11: Definition of frequency layer

Candidate options:

  - Option 1: (Apple)

Definition of frequency layer can be considered after intra-frequency/inter-frequency layer definition and measurement type are concluded.

  - Option 2: (MTK)

If RAN4 concludes not to define CSI-RS based L1 measurement requirements for both RTD>CP case and CSI-RS outside active BWP, it is not necessary to define “frequency layer”.

  - Option 3: (Huawei)

If the center frequency, SCS and CP length of the to-be-measured L1 CSI-RS resources are the same, they are regarded as the same CSI-RS frequency layer.

  - Option 4: (Nokia)
Not to define new terminology of “frequency layer” for CSI-RS based L1-RSRP measurement. Discuss directly the intra/inter-frequency CSI-RS scenarios where CSI-RS resources have similar properties (SCS, CP type, periodicity)

  - Option 5: (Ericsson)

Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and the CSI-RS occasions are fully/partially overlapping in time domain.

#### Issue 4-12: Whether to consider CSI-RS based L1-SINR measurement

Candidate options:

  - Option 1: (CATT, Apple)

RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

  - Option 2: (ZTE, CT)

Focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement.

#### Issue 4-13: CSI acquisition on candidate cell(s) before or during LTM cell switch

Candidate options:

  - Option 1: (ZTE)

Wait for more RAN1 progress to start the discussion on CSI acquisition in RAN4.

  - Option 2: (Nokia)
CSI-acquisition may start before the cell switch (e.g. early CSI-acquisition case) or it may start from the cell switch command (e.g. After SCS CSI-acquisition). RAN4 can start to discuss if and how CSI-acquisition is performed during and after the cell switch.

  - Option 3: (Qualcomm)

For the baseline CSI acquisition (DL CSI acquisition after LTM CSC MAC-CE reception), RAN4 to not define requirements.
For the optional CSI acquisition (DL CSI acquisition before LTM CSC MAC-CE reception), RAN4 to discuss whether and what requirements to define after the following aspects for CSI-RS based L1-RSRP measurement from the candidate cell are concluded:
• Pre-requisite conditions, maximum RTD value, gap-less vs. gap-based, etc.

#### Issue 4-14: CSI-RS accuracy requirements

Candidate options:

- Option 1: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline. (Apple, ZTE)