3GPP TSG-RAN4 Meeting #114	R4-2502611

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5415 | rev | 1 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to UE transmit timing for Satellite Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Eutelsat Group, Thales, European Space Agency, Sharp, CHTTL, Airbus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-02-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The definiton of Te_NTN in Table 7.1C.2-3 for 120 kHz SCS comprises a factor with value 10 in square brackets. This is an open issue that was not resolved for Rel-18. The square brackets need to be removed, and the value cannot exceed 7.5 which is the threshold for ensuring the guard period Tg does not become negative as described in R4-2417511. The maximum aggregate adjustment rate in 200 ms is too small for mobile VSAT served by GSO to accommodate expected UE velocities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The definition of UE Timing Advance adjustment accuracy for VSAT UE in FR2-NTN in Table 7.3C.2.2-2 is missing the “±” symbol which was used for all the other UE Timing Advance adjustment accuracy definitions for TN, REDCAP, FR1-NTN Satellite access and ATG in Tables 7.3.2.2-1, 7.3A.2.2-1 7.3C.2.2-1 and 7.3D.2.2-1 respectively. The value for maximum aggregate adjustment rate in 200 ms is increased from Tq_NTN to 3 * Tq_NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the UL timing requirements for Rel-18 NR NTN are not properly specified, VSAT UEs designed according to 3GPP standards cannot support reliable system operation. If the values for maximum aggregate adjustment rate in 200 ms is not increased then the slew rate limiting of the timing advance changes will not be able to keep up with expected UE veleocities and Te will be unable to stay within Te_NTN leading to a loss of synchronization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1C.2, 7.3C.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is a re-presentation of CR 4960r2 from R4-2420236 which was techncally endorsed at RAN4 #113 but not implmeneted in v18.8.0 due to the wrong revison number on the cover sheet. |  |  |  |  |  |  |  |  |

## 7.1C	UE transmit timing for Satellite Access

### 7.1C.1	Introduction

The UE shall have capability to follow the frame timing change of the reference cell in RRC_CONNECTED state. The uplink frame transmission takes place NTA+NTA offset+NTA,adjcommon+NTA,adjUE×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

### 7.1C.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te_NTN where the timing error limit value Te_NTN:

-	is specified in table 7.1C.2-1 for FR1-NTN.

-	is specified in table 7.1C.2-2 and table 7.1C.2-3 for VSAT UE in FR2-NTN.

This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission.

The UE shall meet the Te_NTN requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc.

The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna.

NTA for PRACH is defined as 0. NTA+NTA offset+NTA,adjcommon+NTA,adjUE×Tc (in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3C was applied. or after the last update in NTA,adjcommon or NTA,adjUE.

The value of NTA offset depends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). NTA offsetis defined in table 7.1.2-2 for FR1-NTN.

NTA offset is defined in table 7.1C.2-4 for VSAT UE in FR2-NTN.

NTA,adjcommon and NTA,adjUE are defined in TS 38.211 [6].

Table 7.1C.2-1: Te_NTN Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |

Table 7.1C.2-2: Te_NTN Timing Error Limit for fixed VSAT is served by GSO and fixed VSAT is served by NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |

Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] Note 2: Te_NTN for 120 kHz UL SCS apply under the following conditions: LOS channel between UE and each GNSS satellite GNSS signal power level level ≥ those defined in Table 6.2-1 of 38.171 GNSS satellite allocation is no less than those defined in Table 6.2-2 of 38.171 The velocity change within one second does not result in the position difference with respect to the projected position, based on the position and velocity obtained one second before, exceeding 10 meters. |  |  |  |  |

Fixed VSAT and mobile VSAT are defined in TS 38.101-5 [43].

Table 7.1C.2-4: The Value of NTA offset for VSAT in FR2-NTN

| Frequency range and band of cell used for uplink transmission | NTA offset(Unit: TC) |
| --- | --- |
| FR2-NTN | 0 |
| Note 1: The UE identifies NTA offset based on the information n-TimingAdvanceOffset as specified in TS 38.331 [2]. If UE is not provided with the information n-TimingAdvanceOffset, the default value of NTA offset is set as 0 for FR2-NTN band. |  |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for PUCCH, PUSCH including PUSCH transmissions in Time Domain Window when pusch-DMRS-Bundling is enabled, and SRS transmission, the UE shall be capable of changing the transmission timing according to the received downlink frame of the reference cell, the updating of NTA,adjcommon and the updating of NTA,adjUE, except when the timing advance in clause 7.3C is applied.

#### 7.1C.2.1	Gradual timing adjustment

When the transmission timing error between the UE and the reference timing exceeds Te_NTN then the UE shall adjust the timing such that timing error is to within Te_NTN. The reference timing shall be NTA+NTA offset+NTA,adjcommon+NTA,adjUE×Tc before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, in one adjustment shall be Tq_NTN.

2)	The minimum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last one second, shall be Tp_NTN per second.

3)	The maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last 200 ms, shall be 3 * Tq_NTN per 200 ms.

Where, the maximum autonomous time adjustment step Tq_NTN and the aggregate adjustment rate Tp_NTN are specified in table 7.1C.2.1-1.

Table 7.1C.2.1-1: Tq_NTN Maximum Autonomous Time Adjustment Step and Tp_NTN Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq_NTN | Tp_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | N/A | N/A |
| FR2-NTN | 60 | 2.5*64*Tc | 2.5*64*Tc |
|  | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

## <Next modified section>

## 7.3	Timing advance

### 7.3.1	Introduction

The timing advance is initiated from gNB to UE in EN-DC, NR-DC, NE-DC and NR SA operation modes, with MAC message that implies the adjustment of the timing advance, as defined in clause 5.2 of TS 38.321 [7].

### 7.3.2	Requirements

#### 7.3.2.1	Timing Advance adjustment delay

UE shall adjust the timing of its uplink transmission timing at time slot n+ k+1 for a timing advance command received in time slot n, and the value of k is defined in clause 4.2 in TS 38.213 [3]. The same requirement applies also when the UE is not able to transmit a configured uplink transmission due to the channel assessment procedure.

#### 7.3.2.2	Timing Advance adjustment accuracy

The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to the UE Timing Advance adjustment accuracy requirement in table 7.3.2.2-1, to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command step is defined in TS 38.213 [3].

Table 7.3.2.2-1: UE Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| --- | --- | --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc | ±10 Tc | ±6 Tc |

## 7.3A	Timing Advance for RedCap

### 7.3A.1	Introduction

The timing advance is initiated from gNB to UE configured with only PCell, with MAC message that implies the adjustment of the timing advance, as defined in clause 5.2 of TS 38.321 [7].

### 7.3A.2	Requirements

#### 7.3A.2.1	Timing Advance adjustment delay

UE shall adjust the timing of its uplink transmission at time slot n+ k+1 for a timing advance command received in time slot n, and the value of k is defined in clause 4.2 in TS 38.213 [3]. The same requirement applies also when the UE is not able to transmit a configured uplink transmission due to the channel assessment procedure.

#### 7.3A.2.2	Timing Advance adjustment accuracy

The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to the UE Timing Advance adjustment accuracy requirement in table 7.3A.2.2-1, to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command step is defined in TS 38.213 [3].

Table 7.3A.2.2-1: UE Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 |
| --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc |

## 7.3C	Timing advance for satellite access

### 7.3C.1	Introduction

The timing advance is initiated by UE configured with only PCell served by SAN, upon initiating a validity timer for NTA,adjcommon  and  NTA,adjUE. The timing advance can be adjusted with MAC message that implies the adjustment of the timing advance, as defined in clause 5.2 of TS 38.321 [7].

### 7.3C.2	Requirements

#### 7.3C.2.1	Timing Advance adjustment delay

UE shall adjust the timing of its uplink transmission timing from the beginning of uplink time slot n+ k+1+2µ ∙Koffset for a timing advance command received in time slot n, and the value of k, µ and Koffset are defined in clause 4.2 in TS 38.213 [3]. The same requirement applies also when the UE is not able to transmit a configured uplink transmission due to the channel assessment procedure.

#### 7.3C.2.2	Timing Advance adjustment accuracy

The UE shall adjust the timing of its transmissions, apart from a change of NTA,adjUE and NTA,adjcommon between the preceding uplink transmission and the current transmission, with a relative accuracy better than or equal to the UE Timing Advance adjustment accuracy requirement in table 7.3C.2.2-1, for FR1-NTN and in table 7.3C.2.2-2 for VAST UE in FR2-NTN to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command step is defined in TS 38.213 [3].

Table 7.3C.2.2-1: UE Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 |
| --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | N/A |

Table 7.3C.2.2-2: UE Timing Advance adjustment accuracy for VSAT UE in FR2-NTN

| UL Sub Carrier Spacing(kHz) | 60 | 120 |
| --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±128 Tc | ±32 Tc |
| NOTE 1: VSAT UE are defined in TS 38.101-5 [43] |  |  |

## <End of changes>