3GPP TSG-RAN WG4 Meeting #114	R4-2502669

Athens, Greece, 17th – 21st February 2025

Agenda item:			7.21.4

Source:	Nokia

Title:	Coffee break discussion minutes for FS_NR_AIML_Mob

Document for:	Approval

# Introduction

This document contains the coffee break discussion minutes for the discussion on AI/ML mobility SI. This meeting discussed some of the topics from the moderator’s summary [1].

# Discussion.

## Topic#2: RAN4 Requirements

### Sub-topic 2-1: General and Scope

#### Issue 2-1-2: Ground Truth Definition for RSRP accuracy

- Background

  - The following agreement was reached in the adhoc session during RAN4#114 Meeting:

Agreement: - The way the ground truth is extracted in the testing may differentiate between FR1 and FR2.

During RRM session, following two alternatives were identified for FR1:

For FR1: - Alt 1: based on the transmitted or reception power (1-1 and 1-5) (MTK, E///, Nokia, CMCC) - Alt 2: based on the reported measurement value under certain conditions (1-2/3/4) (QC, Xiaomi, OPPO, Apple)

- Proposals and Observations:

  - Proposal 1 (ZTE):

    - For FR1, the transmitted power can be used as baseline for the definition on ground truth of L3-RSRP, which is obtained as the legacy way of conducted test.

    - For FR2, the definition of ground truth on R19 AI BM can be reused, that is, ground truth of L3-RSRP is the approximate as the reported RSRP measurement under a certain SNR (the SNR is high enough for sufficient accuracy of reported RSRP).

  - Proposal 2 (Huawei):

    - The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2.

    - The ground truth of L3 RPRP in FR1 is FFS, whether it is reported L3 RSRP measurement under sufficient high SNR or the transmitted RSRP directly from TE (conducted test).

  - Proposal 3 (Apple):

    - In FR2: The ideal received L3-RSRP is assumed to approximate the reported RSRP measurement, provided the SNR is sufficiently high for reliable RSRP accuracy.

    - In FR1: The ideal received L3-RSRP is taken to be equal to the transmitted power (assuming conducted test and AWGN channel during the test)

  - Proposal 4 (Qualcomm):

    - Option 1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP)

      - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE and the configured filter coefficient.

      - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc.

    - Option 2-A: Ground truth based on measurement report from the UE under the SAME radio condition (UE-reported L3-RSRP with noise)

      - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

        - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

        - The radio condition such as S(I)NR remains the same during the target inference time instance and the report instance, i.e., from X – Y ms to X + Y ms.

        - [Testability: to avoid UE from cheating the test, TE can create another cell having the same radio condition to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction]

      - The definition of the ground truth does not differentiate FR1 and FR2.

    - Option 2-B: Ground truth based on measurement report from the UE under the IDEAL radio condition (UE-reported L3-RSRP with noise-free)

      - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

        - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

        - The radio condition such as S(I)NR for the measurement report is set such that the measurement result at the UE can be accurate enough.

        - [Testability: another round of tests would be needed for the L3-RSRP measurement report]

      - The definition of the ground truth does not differentiate FR1 and FR2.

  - Proposal 5 (Xiaomi):

    - If the TE is capable of calculating the ideal cell-level L3-RSRP, RAN4 should discuss the specific conditions and information required for the TE to accurately determine this ideal value.

    - If the ground truth of L3-RSRP is reported by the UE, RAN4 should discuss whether and how to prevent the UE from providing cheating reports.

  - Proposal 6 (MediaTek):

    - For scenario 2 and 3, the “ground truth”, i.e., referenced RSRP to compare with in the test, can be derived based on the transmitted power at TE and the assumed L1 filtering.

  - Proposal 7 (Nokia):

    - RAN4 may consider UE reported L3 RSRP measurement as the approximate of ideal measurement of L3-RSRP and the ground truth for FR2 L3 prediction

    - RAN4 should consider that the test equipment in FR1 has the information of ground truth regarding L3-RSRP (transmit power).

  - Proposal 8 (vivo): For testing, the ground truth definition of L3-RSRP for FR1 and FR2 can be further studied based on the following candidate Options:

    - Option 1: To define the ground truth of L3 RSRP as the reported RSRP measurement under certain SNR

    - Option 2: To define measured RSRP from t1 to t2 as the ground truth under the condition that the channel from t0 to t1(prediction window) is the same as the channel from t1 to t2

  - Proposal 9 (CATT):

    - The definition of ground truth in FR1 can be the ideal L3 filtered cell-level measurement results for both temporal and frequency domain predictions

    - RAN4 to postpone the discussion on the definition of ground truth in FR2.

  - Proposal 10 (Tejas Network Limited)

    - For FR1 conducted tests, the ground truth should be derived from the test equipment.

  - Proposal 11 (OPPO)

    - ground truth is defined as the actual L3-RSRP measurement results based on legacy measurement procedure for both FR1 and FR2.

  - Proposal 12 （CMCC）

    - For FR2, ground truth of L3-RSRP is the approximate as the reported RSRP measurement under a certain SNR (the SNR is high enough for sufficient accuracy of reported RSRP)

    - For FR1, ground truth of L3-RSRP is the transmitted power, which is obtained as the legacy way of conducted test

  - Proposal 13 (Samsung): Two directions can be considered to define ideal RSRP

    - 	Direction 1: Settled by TE. The ideal RSRP is of the transmit power

    - 	Direction 2: Settled by measurement report. The ideal RSRP can be derived based on the RSRP measurements reported by UE for the noise-free or very high SNR conditions

  - For the both directions above, emulation of noise-free/high SNR test conditions and assumption of single signal source are supported

- Candidate options:

For FR1:

- Alt 1: based on the transmitted or reception power (1-1 and 1-5) (MTK, E///, Nokia, CMCC)

- Alt 2: based on the reported measurement value under certain conditions (1-2/3/4) (QC, Xiaomi, OPPO, Apple)

Discussion:

Nokia: We should know the timing of the measurement and prediction.

Qualcomm: There is no ambiguity for prediction but only for measurement

E//: If we don’t know the time instance of prediction, how we can get the ground truth

Qualcomm: Why time window is important for ground truth extraction

- Agreement:

  - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

  - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

  - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as:

    - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction

    - Impact of L3 filtering

    - Number of samples for L1 filtering

    - Timing window, if it exists, where UE can’t measure the ground truth in Alt2

    - channel condition

    - Avoid UE cheat in the test

    - S(I)NR variations in the test at different time instances (for Alt2)

    - Other aspects are not precluded

  - Companies are encouraged to provide analysis on all or some of the aspects in the list

#### Issue 2-1-9: Impacts from observation and prediction windows

- Proposals and Observations:

  - Proposal 1 (OPPO):

    - RAN4 to consider the impact of UE speed, PW length, MRRT or skipping patterns on intra-frequency measurement prediction accuracy requirements.

  - Proposal 2 (Ericsson):

    - RAN4 to study relationship between the OW length and PW length and the impact of the number of measurements performed within the OW on the accuracy of the predicted measurements for the instances inside the PW for temporal prediction cases.

  - Proposal 3 (Huawei):

    - For intra-frequency Intra-cell temporal domain prediction case A, prediction window length (or called as prediction evaluation period) may need to be discussed and specified.

    - For intra-frequency Intra-cell temporal domain prediction case B, RSRP prediction accuracy under the side condition of MRRT with limited PW length may need to be specified.

  - Proposal 4 (Apple):

    - Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy requirements to assess the feasibility of the RAN2 study. This translation is essential for aligning performance metrics and ensuring that mobility optimization goals are met effectively. Additionally, explore the impact of RRM measurement requirements on prediction accuracy across various RRM measurement prediction cases and evaluation scenarios given side conditions of OW and PW, SNR.

  - Proposal 5 (Xiaomi):

    - For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window.

  - Proposal 6 (Samsung):

    - For the side condition consideration, RAN4 to discuss whether to/how to define a certain threshold for OW length to evaluate the L3 measurement prediction accuracy for temporal domain prediction case A.

    - For the RRM impact on prediction window, at least the following may need to be discussed

      - Scheduling restriction

      - Measurement latency for MO (s), MO (s) reduction

  - Proposal 7 (Nokia):

    - RAN4 should investigate impacting factors on the selection of observation and prediction windows for RAN4 requirements and test cases specification.

- Candidate options:

  - Option 1: RAN4 to consider the impact of PW length on prediction accuracy requirements for temporal predictions.

  - Option 2: RAN4 to consider the impact of OW length on prediction accuracy requirements for temporal predictions.

  - Option 3: RAN4 to consider the impact of OW and PW lengths on prediction accuracy requirements for temporal predictions.

  - Option 4: RAN4 to not consider the impact of OW and PW lengths on prediction accuracy requirements for temporal predictions.

- Recommended WF

  - Discuss above listed candidate options.

Discussion:

Agreement:

- RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions.

  - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

# References

[1] R4-2500533, “Topic summary for [114][219] FS_NR_AIML_Mob”, Moderator(Nokia), RAN4#114