3GPP TSG-RAN WG4 Meeting #114	R4-2502701

Athens, Greece, 17 – 21 February, 2025

Title:	reply LS on soft satellite switch with re-sync

Response to:	R2-2410978

Release:	Release 18

Work Item:	NR_NTN_enh-Core

Source:	RAN4

To:	RAN2, RAN1

Cc:

Contact Person:

Name:	Li Zhang

E-mail Address:	zhangli164@huawei.com

Attachments: -

- Overall Description:

RAN4 thanks RAN2 for the information in LS R2-2410978. Regarding the question

RAN2 kindly asks RAN4 and RAN1 whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

Under following assumptions,

- Starting boundaries of SFN0 of the source satellite and the target satellite are aligned at the uplink time synchronization reference point;

- The parameter ssb-TimeOffset is the relative offset between the SSB of the source satellite and the SSB of the target satellite during [t-ServiceStart, t-Service];

RAN4 reached the conclusion that the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

RAN4 would like to note that the temporarily SSB pattern shift (i.e., subject to ssb-TimeOffset) during [t-ServiceStart, t-Service] and shifting back after t-Service was not considered during RAN4 requirement definition and it may require additional UE processing for shifting. Also, it may have impact to UE performing initial access during [t-ServiceStart, t-Service]. It is also noted that the RAN4 requirements are defined assuming the parameter ssb-TimeOffset is the relative time offset between SSB of the source satellite and the SSB of the target satellite with same SSB index.

In addition, RAN4 would like RAN2 to confirm which one of the following is the correct understanding of parameter ssb-TimeOffset,

- Option 1: the relative offset between the SSB of the source satellite and the SSB of the target satellite, and the starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point

- Option 2: the relative offset between the starting boundaries of SFN0 of the source satellite and the target satellite

Regarding the assumptions above, if option 1 is the correct understanding, RAN4 kindly ask RAN1/2:

1) Whether it is feasible to configure ssb-TimeOffset with values other than multiple of 5.

2) How the half-frame indication and SFN indication bits are configured during [t-ServiceStart, t-Service].

2. Actions:

To RAN1/2:

RAN4 respectfully asks RAN2 to take the above information into account in the future work for soft satellite switch with re-sync, and confirm the correct understanding of parameter ssb-TimeOffset.

RAN4 would also respectfully ask RAN1/2 to clarify the two questions as above if option 1 is the correct understanding.

3. Date of Next TSG-RAN4 Meetings:

RAN WG4 Meeting #114-bis		April 07 – April 11, 2025	         TBD, China

RAN WG4 Meeting #115    		May 19 – May 23, 2025	         Malta, MT