3GPP TSG-RAN WG4 Meeting # 114															R4-2502859

# Athens, GR, 17th – 21st February, 2025

Agenda item:			7.21.4

Source:	Moderator (Huawei)

Title:	WF on requirements for A-IoT BS and CW

Document for:	Approval

Introduction

This document captures the agreements about the RF requirements for A-IoT BS and CW on Rel-19 WI solutions for Ambient IoT in NR (RP-243326). The topic summary is R4-2500687 (RAN4#114).

Topic #1: A-IoT BS TX

Background:

Following is the SI conclusions on A-IoT BS TX requirements

| RF Requirement for A-IoT BS- TX requirements |  | Potentially needed or not (SI-->RAN4#114) |
| --- | --- | --- |
| Base station output power |  | YES |
| Output power dynamic | RE power control dynamic range | TBD |
|  | Total power dynamic range | TBD |
| Some requirements like Transmit ON/OFF power | Transmitter OFF power | TBD—>YES |
|  | Transmitter transient period | TBD—>YES |
| Transmit signal quality | Frequency error | YES |
|  | Some requirements like EVM (Modulation quality) | YES |
| Unwanted emissions | Occupied bandwidth | YES |
|  | ACLR | YES |
|  | Operating band unwanted emissions | YES |
|  | Transmitter spurious emissions | YES |
| Transmitter intermodulation |  | TBD |

- Issue 1-1: Base station output power

- Agreement:

  - A-IoT BS output power, Prated,c,AC≤ [38]dBm

- Issue 1-2-1: RE power control dynamic range

- Agreement:

  - Check whether RAN1 supports RE granularity operation for R2D transmission or not.

- Issue 1-2-2: Total power dynamic range

- Agreement:

  - Check whether RAN1 supports PRB granularity operation for R2D transmission or not

- Issue 1-3-1: Transmitter OFF power

- Agreement:

  - Define Transmitter OFF power.

  - values FFS

- Issue 1-3-2: Transmitter transient period

- Agreement:

  - Define Transmitter transient period.

  - FFS values.

- Issue 1-4-1: Frequency error

- Agreement:

  - Reuse the NR micro BS requirements（±0.1 ppm）for A-IoT BS.

- Issue 1-4-2: Modulation quality

- Agreement:

  - FFS whether to define requirements such as modulation Depth, RF Envelope Ripple, RF Envelope Rise Time, RF Envelope Fall Time and RF Pulse width for A-IoT BS.

- Issue 1-5-2: ACLR

- Agreement:

  - Further discuss ACLR requirements considering following aspects:

    - No legacy NR UE indoor assumption

    - Whether 100KHz offset is needed or not

- Issue 1-5-3: OBUE

- Agreement:

  - Postpone OBUE requirements discussion after RAN4 make conclusion on ACLR.

- Issue 1-5-4: Transmitter spurious emissions

- Agreement:

  - Use the FR1 NR BS transmitter spurious emission requirement for A-IoT BS as starting point.

  - FFS on the co-location requirements if the A-IOT BS is co-located with BS operating in other bands.

- Issue 1-6: Transmitter intermodulation

- Agreement:

  - FFS.

Topic #2: A-IoT BS RX

Background:

Following is the SI conclusions on A-IoT BS RX requirements

| A-IoT BS- RX RF Requirement |  | Potentially needed or not |
| --- | --- | --- |
| Reference sensitivity level |  | YES |
| Dynamic range |  | TBD |
| Adjacent Channel Selectivity |  | YES |
| In-band blocking | general blocking | YES |
|  | narrowband blocking | YES |
| Out-of-band blocking |  | YES |
| Receiver spurious emission |  | YES |
| Receiver intermodulation | General intermodulation | TBD |
|  | Narrowband intermodulation | YES |
| In-channel selectivity |  | TBD |

- Issue 2-1-1: Reference sensitivity level

Copied from TR38.769:

Regarding the reference sensitivity level for A-IoT BS, RAN4 reached the following consensus:

- For BS type 1-C,

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]

-	BW is the transmission bandwidth for D2R.

-	NF is the noise figure of A-IoT BS, equal to 10 dB for Medium Range BS.

-	IM is the implementation margin, equal to 2dB.

-	SNR is the SNR value for D2R decoding, the detailed metrics would be discussed in the WI phase.

-	Desens target is considered for CW signal interference cancellation in D1T1 scenario. The desens level for CW signal cancellation might be different for D1T1-A1/D1T1-A2 and D1T1-B. For D1T1-A2, CW signal interference is coming from self-interference and also potential CW interference from other nodes transmitting CW signal. For D1T1-A1, the CW signal interference is coming from other nodes transmitting CW inside the topology. For D1T1-B, the CW signal interference is coming from CW nodes transmitting outside topology.

- Agreement:

  - Define the reference sensitivity level specifically for 15 kHz D2R channel BW at least,

  - Consider the follow aspects

    - Noise figure=10dB

    - IM=2dB

    - SNR

    - Coverage

  - FFS on the definition of channel bandwidth depending on RAN1 conclusions.

- Issue 2-1-2: LLS assumptions and parameters for SNR

- Agreement:

  - Take the following assumptions as a starting point for LLS

  - Companies are encouraged to provide LLS results, assumptions adapted can be reported. The assumptions can be updated based on RAN1 progress.

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 900 MHz |
| SCS | 15 kHz as baseline |
| Block structure | Depend on RAN1 |
| Channel model | AWGN |
| Number of Tx/Rx chains for Ambient IoT device | 1 |
| Number of BS antenna elements | 1 |
| Reference data rate | 1 kbps (M) 5 - 7 kbps (M) |
| Message size | Option 1： {20 bits, 96 bits, 400 bits} are considered for message size. Note 1: companies to report the M value and chip length used for each message size Note 2: CRC is not included for the message size Option 2： 96 bits |
| BLER target | 10% |
| Sampling frequency（SFO） | Randomly select a value from the range of [104 ppm~105 ppm] for device 1 |
| Transmission bandwidth（DSB） | Option 1： X kHz is considered for D2R transmission bandwidth. The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz X = {[15 (M)], [180 (O)]} Option 2: 15kHz |
| [OOK/BPSK chip rate] | Companies to report |
| Receiver bandwidth | FFS |
| Waveform (CW) | unmodulated single tone |
| Modulation | OOK |
| Line code | Manchester encoding |
| FEC | Option 1：No FEC Option 2：1/3 Convolutional Code |
| ADC bit width | Ideal |
| D2R receiver | coherent receiver |
| Repetition | None |

- Issue 2-3: ACS

- Agreement:

  - Evaluate ACS requirements considering no legacy NR UE indoor assumption

- Issue 2-4-1: general blocking

- Agreement:

  - Postpone the blocking requirements discussion after RAN4 make conclusion on ACS requirements.

- Issue 2-4-2: narrowband blocking

- Agreement:

  - Further discuss whether Narrowband blocking requirements is needed or not.

- Issue 2-5: Out-of-band blocking

- Agreement:

  - Reuse the out-of-band blocking requirement from NB-IoT standalone Medium Range BS for A-IoT BS

- Issue 2-6: Receiver spurious emission

- Agreement:

  - The receiver spurious emissions for A-IoT BS can reuse the NR BS requirements specified in Table 7.6.2-1 of TS 38.104.

- Issue 2-7-1: General intermodulation

- Agreement:

  - Further discuss whether general intermodulation requirement is needed or not.

- Issue 2-7-2: Narrowband intermodulation

- Agreement:

  - Further discuss whether narrowband intermodulation requirement is needed or not.

- Issue 2-8: ICS

- Agreement:

  - FFS

Topic #3: CW

| CW RF Requirement | Potentially needed or not |
| --- | --- |
| Output power | YES |
| ON/OFF power | TBD |
| Frequency error | YES |
| Phase noise | TBD for outside topology |
| Unwanted emission | YES |
| Spurious emissions | YES |
| Intermodulation | TBD |

- Issue 3-1: CW Output power (for ahoc discussion)

- Agreement:

  - CW output power [33] dBm.

- Issue 3-2: ON/OFF power

- Agreement:

  - FFS for OFF power

- Issue 3-3: Frequency error

- Agreement:

  - Reuse Micro BS frequency error ±0.1 ppm for A-IoT CW.

- Issue 3-4: Phase noise

- Agreement:

  - Further discuss phase noise requirements

- Issue 3-5: Unwanted emission

- Agreement:

  - Specify emission requirements, values FFS

- Issue 3-6: Spurious emissions

- Agreement:

  - NR spurious emission requirement can be considered as baseline for R19 AIoT CW node

- Issue 3-7: Intermodulation

- Agreement:

  - FFS