3GPP TSG-RAN WG4 Meeting # 114	R4-2502892

# Athens, Greece, 17th – 21st February, 2025

Source:	Huawei, HiSilicon

Title:	TP for TR 37.719-21-11: DC_8_n40-n77

Agenda item:	6.2.3

Document for:	Approval

1 Background

This contribution provides text proposal on the NR band combination DC_8A_n40A-n77A.

2 Text Proposal

---Start of changes---

7.X	DC_8_n40-n77

7.X.1	Configurations for DC

Table 7.X.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE X) |
| --- | --- |
| DC_8A_n40A-n77A | DC_8A_n40A DC_8A_n77A |

7.X.2	Co-existence analysis for DC

Table 7.X.2-1: Band 8 and Band n77 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 3300 | 4200 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2385 – 3320 |  | 4180 – 5115 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1470 – 2440 |  | 5685 – 7520 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5060 – 6030 |  | 7480 – 9315 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 555 – 1560 |  | 8985 – 11720 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4770 – 6640 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5940 – 6945 |  | 10780 – 13515 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8360– 10230 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12285 – 15920 |  | 360 – 680 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8070 – 10840 |  | 3855 – 5760 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14080 – 17715 |  | 6820 – 7860 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11660 – 14430 |  | 9240 – 11145 |  |

Table 7.X.2-2: Band 8 and Band n40 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 2300 | 2400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1385 – 1520 |  | 3180 – 3315 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 640 – 470 |  | 3685 – 3920 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4060 – 4230 |  | 5480 – 5715 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 240 – 445 |  | 5985 – 6320 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3040 – 2770 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4940 – 5145 |  | 7780 – 8115 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6360– 6630 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8285 – 8720 |  | 1120 – 1360 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5070 – 5440 |  | 1855 – 2160 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10080 – 10515 |  | 5820 – 6060 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8660 – 9030 |  | 7240 – 7545 |  |

Based on Table 7.x.2-1 and Table 7.x.2-2, the 3rd IMD product of band 8 and band n40 may fall into Rx frequencies of band n77, and the 2nd and 3rd IMD product of band 8 and band n77 may fall into Rx frequencies of band n28.

7.X.3	∆TIB and ∆RIB values

Table 7.X.3-1: ΔTIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| DC_8_n40-n77 | 0.6 | 0.5 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

Table 7.X.3-2: ΔRIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| DC_8_n40-n77 | 0.2 | 0.4 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

7.X.4	Analysis of MSD requirements

Table 7.x.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD3 product of band 8 and band n40 may fall into Rx frequencies of band n77, IMD2 and IMD3 product of band 8 and band n77 may fall into Rx frequencies of band n28. The MSD values for CA_n8-n40-n77 are reused.

Table 7.x.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8_n40-n77 | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n40 | N/A | 10 | N/A | 2395 | 28 | IMD21 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n40 | 2395 | 10 | 50 | 2395 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3305 | 28.8 | IMD21 |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |

---End of changes---

Reference

[1]	RP-241786, “Revised WID for for Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR Supplementary Uplink (SUL) bands”, Nokia, CHTTL, LGE, Samsung