3GPP TSG RAN WG4 Meeting #114		 R4-2502925

Athens, GR, 17th – 21nd February, 2025

Agenda Item:	7.6.1

Source: 		MediaTek Inc., Nokia, OPPO, vivo [Apple, Ericsson, Huawei, Skyworks,…]

Title: 	TP for TR38.755

Document for:	Approval

1. Introduction

The DL fragmented carrier study has been discussed several meetings and reached agreements on baseline architecture, evaluation assumptions and test conditions[11][12][15]. The contribution try to provide text proposal for the discussion of the study with courtesy of company contributions.

2. Discussion

Proposed TP for TR38.755

<Start of proposed TP>

5	Study of methods for reducing the number of UE Rx chains

An example to present the case of fragmented carriers are shown in the figure below from [6][7].

Figure 5-1: Examples of fragmented carriers [8]

Fragmented carriers is not something completely new for RAN4, since it is essentially non-contiguous intra-band carrier aggregation (NC IB CA). However, proponents of the new SI have chosen to name it Fragmented Carriers, since this is how it is perceived mainly from an operator perspective. Adding a specific name also makes sense to associate the non-contiguous intra-band carrier aggregation with the request for UEs to support downlink reception of the fragmented spectrum block with a single “receive chain”, “receive path” or “receive branch”.

DL fragmented carriers containing more than one non-contiguous CC in the same band consuming multiple Rx RF chains may limit the UE CA capabilities as the maximum supported number of component carriers (CCs) in a CA combination is limited by the number and capabilities of the Rx RF chains as well as the baseband processor capabilities in the UEs.

From [9], the reference RF architecture of non-contiguous CA was initially introduced in LTE Rel-11, and captured in TR 36.823 as below.

Figure 5-2: Reference RF architecture of non-contiguous CA (Figure 6.2.3.1-1 of TR 36.823)

According to this reference RF architecture, the LNA is shared by two non-contiguous carriers, while the mixer, LPF and A/D are independent for each carrier. This reference RF architecture continues to be used in deriving the NR requirements for non-contiguous CA. Therefore, the current RF receiving requirements in TS 38.101-1 for non-contiguous CA are based on this partially shared architecture.

The motivation of current study item is to reduce the number of UE Rx chains that are used to receive the fragmented carriers. To serve this motivation, different RF architectures were mentioned during early discussions, e.g. from separate RF chains per CC to shared RF chains.

Apple has in [10] provided what in their view is meant by a single Rx chain: “A single Rx chain consists of one main Rx and one diversity Rx for a UE with 2Rx”.

In [11] MediaTek shows a block diagram of a single Rx chain and appllying Apple's definition in [13] of a Rx chain to the main and diversity branch in MediaTek’s block diagram results in a definition of a single Rx chain as shown in Fig. 5-3.

Type 1 – Single RX chain

| Single RX chain (consists of one main Rx and one diversity Rx) |  |
| --- | --- |
| Main receiver with RF carrier cantered between CCs | Diversity receiver with RF carrier centred between CCs |

Figure 5-3: Single RX chain as presented in [11], where DAC must mean ADCs, since this is at the receiver.

- It can be observed a single Rx chain for a 2Rx UE consists of one main Rx and one diversity Rx branch according to previously shared understandings from UE vendors.

This is illustrated in Fig. 5-4 with the main and diversity receiver branch.

Figure 5-4: Single Rx Chain for a 2Rx UE consisting of a main and diversity receiver branch (FDD).

It would be beneficial for further discussion to agree what the underlying assumptions are, as a basis for a common understanding, to align terminology which can help companies to ensure they discuss the same thing.

Similar to the previous discussion for 2Rx UEs, it becomes even more inconsistent what is meant by a Rx chain for 4Rx UEs when reviewing literature and previous contributions in RAN4. However, it seems the most predominant interpretation is to have 1 main receiver branch and 3 diversity receiver branches. Meaning, when using Fig. 5-4 as a reference, there will be an addition of two diversity receiver branches.

For the current UE implementation, the descriptions in [10] and [11] assume a separate UE RF chain per fragmented carrier which results in the limitations of the maximum number of supported CCs. For the current UE implementation, a separate UE RF chain per fragmented carrier is assumed.

This study item challenges the need for applying a separate UE Rx chain per fragment. In other words, the ambition is to reduce the number of needed Rx chains for fragmented carrier operation.

As previously mentioned, the concept of fragmented carries is in some way similar to non-contiguous intra-band carrier aggregation (NC IB CA). The reception at the UE of non-contiguous intra-band carrier aggregation in a system with a single Rx chain will be impacted by interference especially in the “gap” between the fragments of the carrier as illustrated with a different colour in Fig. 1. The impact on UE requirements is further discussed in R4-2413340 while the following focuses on the UE receiver architectures.

The in-gap interference will, in a single analogue Rx chain system, pass un-attenuated as the UE does not have the ability to suppress/attenuate the in-gap interference via analogue filtering. Suggested solutions are shown in [9] where the UE, by utilizing additional Rx chains, can attend to this presence of an in-gap interferer. In [9] the single Rx chain system, as also shown in Fig. 5-3, is denoted Type 1 while two other multi-chain systems are denoted Type 2 and Type 3. It can be observed Type 1 UE receiver does not have the ability to suppress/attenuate in-gap interference via analogue filtering.

The differences between the Type 2 and Type 3 UE architectures of [10] may be a bit detailed to follow. Our understanding from [10] is that there is a split at the eLNA (external transceiver LNA) towards two iLNA (internal transceiver LNA) (Type2) and a split only at the DACs (Edit: They must mean ADCs, since this is at the receiver) for CC0 and CC1 (Type 3). However, it is not explaining how the UE actually may receive the two carriers individually.

To extend the understanding of the different choices for transceiver implementation, it would have been chosen to follow the principal presented by other companies and also split into three different types with emphasis on Type 2 and Type 3 in the following. This as Type 1 is the hardware state it is wanted to free up hardware resources i.e. Rx chains as described in the introduction.

It can be noted that there is also the possibility to suppress some of the in-gap interference in the digital domain. However, this does not solve the dynamic range issues given by the near-far problem. It can mitigate some of the adjacent channel interference but only when within proper dynamic range provided from the analog and mixed signal Rx chain. Hence, the focus of this work is on solutions related to the analog Rx path. Even though there may be a possibility to suppress some of the in-gap interference in the digital domain, the objective of the SI is to free up Rx resources hence the focus is on the analog Rx chains.

Type 2 – Internal RX path split:

There are two ways that the signal can be treated by separate Rx chains. It may be split at the front-end components (eLNA), which is seen as Type 3, and it may be split inside the transceiver, which is seen as Type 2. For Type 2 here would like to show a full picture without discussing the iLNA (internal LNA) split. The UE has two Rx chains active, and this allows the UE to center the LO1 and LO2 at the respective centers of each CC, so that the following filtering (channel filters) may attenuate also the in-gap interferer as shown here. In the example of Fig. 4 it is consider non-collocation, so the in-gap interferer is seen as a “worst case” condition. In the collocated case, the wanted to interference ratio will be less.

Figure 5-5: Type 2 – Internal Rx path split. A single Rx chain after frontend filtering is marked with a dashed square.

If this UE is to transition to a single Rx chain state (Type 1), it will have to accept an in-gap interferer as shown in the next view of Type 2, that visualizes the “Extra Receiver chain” as toned-out colours:

Figure 5-6: Type 2 – Internal Rx path split. Unused Rx chain is toned-out. Type 2 becomes Type 1.

In this state, the UE is without analogue filtering to suppress the interferer, and RAN4 will have to discuss the relaxation of the receiver requirements for the UE to tolerate some level of in-gap interference presence. Since the requirement is based on the presence of an interfering signal, it is closer related to the ACS requirement than to the Reference sensitivity requirement for non-contiguous intra-band carrier aggregation, and a new set of requirements must capture the conditions and the relaxation for the UE state that uses a single Rx chain for the reception of the two fragmented carriers in the presence of an in-gap interferer.

This means that when a Type 2 implementation is using a subset of its Rx chains when operating in fragmented carrier mode the new set of requirements will apply.

Type 3 – External Rx path split:

The second way that the signal can be treated by separate Rx chains is a split at the front-end components by multiple external LNAs (eLNA). This can either be within the main/diversity module, or at an extension from the main/diversity module that allows connection to another receive pin at the transceiver. For Type 3 the study would like to show a full picture and without discussion of detail whether front-end modules splits internally at the eLNA or if there’s a separate front-end module that accompanies the main/diversity modules to extend the connection possibilities towards the transceiver(s). Some solutions also have 2 transceivers that allow split of reception in NC IB CA between the two transceivers. The full system view is that the UE has two Rx chains active, and this allows the UE to center the LO1 and LO2 at the respective centers of each CC, so that the following filtering (channel filters) may attenuate also the in-gap interferer as shown here.

Figure 5-7: Type 3 – External Rx path split. A single Rx chain after frontend filtering is marked with a dashed square.

If this UE is to transition to a single Rx chain state (Type 1) it will have to tolerate an in-gap interferer as shown in the next view of Type 3, that visualizes the “Extra Receiver chain” as toned-out colours:

Figure 5-8: Type 3 – External Rx path split. Unused Rx chain is toned-out. Type 3 becomes Type 1.

In this state, the UE is in the same conditions as in Type 2, bringing both Type 3 and Type 2 into the state of Type 1 single Rx chain reception, so the relaxation of the requirements is the same for Type 2 and Type 3, and the same set of requirements can apply regardless of how the UE supports non-contiguous intra-band carrier aggregation.

As presented previously it has been shown that with both a Type 2 and a Type 3 transceiver implementation it is possible to separate the Rx chains (i.e. operate as Type 1). When separating or freeing up Rx chains in the Type 2 and Type implementations it is possible to apply these now idle Rx chains for reception of other carries if new RAN4 requirements for this type of operation is introduced to the specification.

Reference architecture for the study item:

In the RAN4#112bis meeting, it was agreed the reference architecture is based on fully shared RF chain[12].

The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter

  - To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers

Figure 5-9: Reference architecture of one Rx RF chain

  - A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823)

Discussion on the baseband (BB) assumptions

There are also some discussion on baseband assumptions i.e., shared/separated BB filter that may relate to rejection ability toward unwanted signal for the study as well as “single FFT” or “separate FFT” assumption evaluation.

Referring to [13], there may be different digital BB chain approaches:

o	Separate digital BB chains per CC after digital down conversion which would enable a dedicated digital BB filter and thus good ACS.

o	Common digital BB chain assuming that CCs are SCS aligned such that a shared FFT/digital filter are used. In this case it may be difficult to maintain good ACS and blocking performance, especially in the gap.

•	Note that if a single FFT is used to receive two non-contiguous CCs/cluster that are SCS aligned, if the CCs are 15kHz SCS the maximum BW is 50MHz and if 30kHz it is 100MHz.

Figure 5-9 illustrates some of the above aspects and terminology. The CCs in the illustration are the co-located operator non-contiguous CCs/clusters to be received.

Figure 5-10 illustration of shared Analog path and shared/separate digital BB paths

For the baseband signal processing, referring to[14], separate FFT is legacy implement to handle DL NCCA carriers since it provides better rejection on interference and unwanted signal. Single FFT that can receive both NCCA carriers including in-gap interference (with poor rejection) at a time. There are some constraint for single FFT. The channel raster of PCC and SCC shall align to orthogonal grid that can avoid unnecessary ICI after FFT. This would need to be guaranteed by network scheduling. And due to the max FFT size is 4096, for the fragment carriers that span larger than 50MHz, only 30KHz or 60KHz SCS would be applicable for the case.

With above assumptions in observations 2 & 3, the evaluated RF performance when there exists in-gap interference applying single FFT together is shown below. The fc of interference was assumed in the middle of two FFT grid as worst-case simulation.

Figure 5-11: SCC: 5M, PCC: 5M + ACS single FFT simulated spectrum plot

Figure 5-12: SCC: 5M, PCC: 5M + IBB1 single FFT simulated spectrum plot

Figure 5-13: SCC: 5M, PCC: 5M + IBB2 single FFT simulated spectrum plot

It can be seen from simulated results, the spectrum of interference was spread out due to allocated on non-orthogonal FFT grid. UE would be able to pass ACS1/2 and IBB1 requirements without relaxation but would fail on IBB2. And due to wide frequency span for a single FFT, the baseband power consumption could be higher. It is expected single FFT may have worse rejection ratio to interference signal and this is not common baseband implementation for DL NCCA carriers.

If UE apply separate FFT baseband, it is foreseen the digital filter can provide better rejection on the in-gap interference signal so that performance of wanted CCs would not be degraded except when the interference falls in the Rx image range of wanted signal. Separate FFT for DL NCCA carriers has been widely used for modern modem design.

Agreed baseband assumption:

It has been agreed in [15] single FFT or separate FFT can be left to UE implementation, and however the condition of single FFT compared to separate FFT can be captured in the technical report.

< Other contents are to be added by contribution driven >

6	Study on power spectral density difference between carriers of co-located adjacent channel operators

It is also intended to study on power spectral density difference between carriers of collocated adjacent channel inter-operators impact on the UE in the objective of current SI. The PSD difference between operators is for further study. Here evaluation results contributed by different companies are captured as below.

Evaluation from company A:

Referring to [16], according the discussion in RAN4#112, it’s very difficult to ensure different operators are always co-located. Even in some cases the operators share the same site, it’s very common that the infrastructures are not shared. As a result, the coverage of different operators is not fully aligned. Below Figure 6-1 shows a possible multi-operator deployment case.

Figure 6-1: Multi-operator deployment case

In this example, the UE is at the edge of its serving cell, while almost in the center of the neighbouring operator cell. Taking the BS antenna pattern from TR 38.769 (fixed beam direction) as the assumption for both cells, the UE’s maximum receiving RSRP difference is equal to the antenna gain difference between horizontal angle = 0 degree and +/-60 degree. The Figure 8 shows the RSRP difference could achieve 10dB in this case.

Figure 6-2: Delta RSRP Multi-operator deployment case

If the deployment includes both macro cell and micro cell, the possible RSRP difference will increase. Also the fast fading will bring more fluctuation to the RSRP difference. For the purpose of study, it can be considered a higher level RSRP difference, e.g. 20dB.

Evaluation from company B:

Referring to [14], some system level simulation results which follows coexistence frame work considering large-scale parameters were also provided. The environment and configuration assumptions are as following bullets:

- Co-site case

- Same BW/Power/ANT # of 2 operators at n25 (1.9GHz)

    - 10 MHz(wanted)+10 MHz(unwanted) BW

- Care interference from the same site.

- Log RSRP difference between 2 operators à CDF

The detailed parameters are listed in the table below:

Table 6-1: Simulation parameters of PSD difference from UE reception point of view

| Parameter | setting |
| --- | --- |
| Scenario | Dense Urban |
| Channel model | UMa |
| ISD | 200m |
| Number of operators | 2 |
| Number of sites | 7/operator, each with 3 sectors |
| Number of UEs | 10/sectors |
| Carrier frequency | 1.9GHz |
| Channel BW | 10MHz |
| Antenna pattern | AAS is assumed (same as that in coexistence study) |
| BS antenna configuration | 32 ANT# : (M,N,P) = (4,4,2) 128 ANT# : (M,N,P) = (8,8,2) |
| Note: The simulation follows coexistence framework, which considers large-scale parameters only. |  |

Figure 6-3: Antenna patterns for the simulation

Figure 6-4: Simulated PSD difference at UE receiving antenna

From simulation result, it can be seen there’s no “interference level larger than wanted CC case” if the BW of both wanted CCs and unwanted CC(=Wgap) are same. The maximum ratio of unwanted CC level to wanted CC level is 0dB in this case. Other different BW/Wgap configurations can be scalable estimated that existing requirements at in-gap ACS1/ACS2/IBB1/IBB2/NBB can fulfil the need for coexistence. On the other hand, collocation inter-operator assumption might be too much simplified and different with real deployment. Some company propose to keep same ACS/IBB requirements and the proposal can be further considered for non-collocated BS cases. And even applying the new UE capability that RF parts can support higher order CA configurations with more CCs, the baseband processing capability may need to extend to handle higher order CA combos including more DL carriers as well as more MIMO-layer processing capability.

<Other contents are to be added by contribution driven>

<End of proposed TP>

Reference:

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”.

[3] 	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4] 	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5] 	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[6] 	R4-2413339 Discussion on methods for reducing the number of UE Rx chains for Fragmented Carriers, RAN4#112, Nokia

[7] 	R4-2415390 Discussion on methods for reducing the number of UE Rx RF chains for Fragmented Carriers, RAN4#112bis, Nokia

[8] 	RP-233374, Fragmented carriers in the DL, TELUS, Bell Mobility, T-Mobile USA, Telstra Limited, US Cellular Corporation, Nokia, Nokia Shanghai Bell

[9] 	R4-2411691 On general aspects of fragmented carriers, RAN4#112, Huawei

[10] 	RP-241269, On Fragmented Carriers in Rel-19, Apple

[11] 	RP-241361, Recommended WF on Fragmented Carriers, MediaTek

[12] 	R4-2417201 WF on NR FR1 DL fragmented carriers study, RAN4#112bis

[13] 	R4-2413031 On architecture options for fragmented spectrum reception, RAN4#112, Skyworks

[14] 	R4-2416138 Discussion on UE Rx chain architecture and network assumption, RAN4#112bis, MediaTek

[15] 	R4-2420519 WF on NR FR1 DL fragmented carriers study, RAN4#113

[16] 	R4-2415804 On methods for reducing the number of UE Rx chain, RAN4#112bis, Huawei