3GPP TSG-RAN WG4 Meeting #114	R4-2502994

# Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:			5.33.1

Source:	Huawei, HiSilicon

Title:	WF on FR2 Power Class 8

Document for:	Approval

Way forward

Issue 1: Introduction of UE FR2 Power Class 8

- Way Forward:

  - Option 1: As a compromise, RAN4 to consider adopting the following requirements for n257/n258/n261:

    - min pk. EIRP: 16.4 dBm

    - Improved EIRP spherical coverage requirements @ 50th% ile direction than PC7 (5.5 dBm)

      - Option 1-1: 7.5 dBm

      - Option 1-2: 11.5 dBm

      - Option 1-3: Other values

  - Option 2: As a compromise, RAN4 introduces the similar access control scheme as 2Rx XR and RedCap for FR2 PC8 with the min peak EIRP = 16.4 dBm and EIRP spherical coverage@50%-ile CDF = 5.5 dBm

    - intraFreqReselectionFR2PC8

    - cellBarredFR2PC8

  - Option 3: Option 1 + Option 2

  - Option 4: Based on the observations made in R4-2501569 (see Section 1.1), PC8 is not introduced

Issue 2: Procedure for introduction of FR2 PC8

- Agreements:

  - Step 1: Continue the discussion in TEI_18 in RAN4 as per the guidance from RAN#106 meeting (Dec. 2024)

  - Step 2: RAN4

    - Agree formal CRs for Rel-18 including RF requirements in TS 38.101-2, RRM requirements in TS 38.133 and release independence in TS 38.307 by TEI_18 if RAN4 agrees to introduce FR2 PC8

      - Add an editor’s note in the specification: This is for early implementation purposes only

    - Send LS to RAN2 to add the UE capability of FR2 PC8 by TEI_18.

      - Request RAN2 to add an editor’s note in the specification: This is for early implementation purposes only

  - Step 3: RAN

    - A new Rel-19 WI for introduction of the FR2 PC8 is proposed exceptionally with no TU assumption.

  - Step 4: RAN4

    - Agree formal CRs for Rel-19 including RF requirements in TS 38.101-2, RRM requirements in TS 38.133 and release independence in TS 38.307 with the allocated WI code

      - Remove the editor’s note in the Rel-19 specifications to complete the WI and keep the editor’s note in the Rel-18 specifications

- Reference

- R4-2417183, WF on FR2 Power Class 8, RAN4#112bis, Huawei, HiSilicon

- R4-2414420, WF on introduction of new FR2 Power Class 8, RAN4#112, Huawei, HiSilicon

- R4-2415629, Updated system simulation assumption for introduction of FR2 PC8, Huawei, HiSilicon