3GPP TSG-RAN WG4 Meeting #114			    								               R4-2503016

Athens, Greece, February 17-21, 2025

Agenda item:			7.8.4

Source:	Apple

Title:	WF on low-low band switching

Document for:	Approval

# Way Forward

Issue 1: Scope clarification

-	Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI

-	It is not precluded that such two low bands are further combined with the additional band

-	The combination of those two low bands with other band can be further discussed in the future release

Issue 2: Switching period values

Summary of company proposals (for information):

5us: 3 (Spreadtrum, Huawei, Ericsson) 13us, assuming no LO retuning: 2 (Ericsson, Huawei) 35us: 6 (Huawei, Qualcomm, ZTE, CATT, Spreadtrum, Xiaomi) 70us: 1 (Samsung) 100us: 2 (Apple, Murata) 140us: 9 (Apple, Qualcomm, ZTE, CATT, MediaTek, Spreadtrum, Samsung, Xiaomi, Murata) 140 + FFS us: 1 (Skyworks) 200us: 1 (Google) 210us: 4 (Qualcomm, ZTE, Samsung, Xiaomi)

-	Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations

-	35us, 140us

-	RAN4 to further discuss 13us

-	LS will capture the currently confirmed values

-	It is not precluded to send another LS with more values in future meetings

Issue 3: Switching period capability

-	Discuss whether the switching delay capability can be per band combination

Issue 4: Time mask for switching

-	Discuss the time mask for switching based on the following options:

-	Option 1: Switching period can be either within carrier 1 (FDD) or within carrier 2 (SDL)

-	Option 2: Switching period is always within the switch-from carrier

-	Option 3: Switching period is contained within the switching gap configured by the network

-	Other options are not precluded

Issue 5: Switching period location

-	Discuss the switching period location based on the following options:

-	Option 1: Switching period location is entirely up to RAN1 design

-	Option 2: Switching period location is defined relative to the slot boundary

-	Other options are not precluded

Issue 6: UE reference architecture

-	Companies are encouraged to contribute proposals on the UE reference architecture as a basis for developing RF requirements for the feature

Issue 7: Test methodology

-	Companies are encouraged to contribute proposals related to the testability of the time mask requirement