# 3GPP TSG-RAN WG4 Meeting # 114                                                         R4-2503022

# Athens, Greece, 17 – 21 February 2025

# Title:	Input to RAN4 PRD on rules for 1UL CC MSD requirements

# Agenda Item:	6.2.6

# Source:	Skyworks Solutions, Inc., Murata

# Document for:	Approval

# Introduction

This document proposes to capture the legacy rules on 1UL REFSENS exceptions (MSD) from TR 38.846 and the recently updated rules on near-miss UL harmonic MSD [1,2], and rules presented at this meeting on near-miss harmonic mixing MSD [3].  This proposal is based upon the PRD clause 5.5 structure of [4].

# Discussion

## Background

The TP for PRD is submitted for structure proposed in [4] for clause 5.5 “Guidelines on MSD”. A single proposal combines different sets of rules that fall into two groups:

- New rules on UL harmonic MSD, and on near-miss harmonic MSD test points [1,2,3].

- Legacy MSD rules imported from TR 38.846. For guidelines on MSD due to UL harmonic interference, the TR refers to the agreed WF [4]. We propose merging these rules with those of agreed WF [2].

- Guidelines on handling any new “highest/lowest” CBW due to BCS4/5 requirements are copied from the last agreed WF [6]. The compilation of all these rules shows that the terminology on these new CBW has not been consistent across all WF and past agreements. We therefore propose to “normalize” the terminology by:

  - replacing the term “lowest” with the term “smallest” in cases where a new “greater” CBW is supported in a given band, and

  - replacing the term “highest” with the term “greatest” in cases where a new “smaller” CBW is supported in a given band.

- Guidelines on MSD for intra-band non-contiguous CA with 1UL CC are updated based on [8].

## 1UL CC MSD TP from TR 38.846 legacy rules

Proposal : Adopt the following TP for RAN4 PRD.

******************************* Start of TP to RAN4 PRD 0.2.0 *******************************

### 5.5	Guidelines on MSD

### 5.5.1	Intra-band CA

#### 5.5.1.1	One Uplink CC

#### Guidelines for intra-band reference sensitivity with one uplink carrier:

- It is sufficient to specify only one MSD test point per BCS.

- If a test point can only be supported by a given BCS, a footnote is needed to indicate this restriction.

Additionally,

For NR-CA intra-band contiguous reference sensitivity with one uplink CC:

- The MSD is specified for one DL SCC for which the reference sensitivity is increased by ΔRIBC.

- For NR_CA, the downlink PCC carrier center frequency shall be configured closer to the uplink operating band than any of the downlink SCC center frequency.

- The PCC/SCC CBW and the UL band RB allocation are selected to specify the worst-case (highest) MSD level,

- For UL RB allocation,

  - The PCC UL band parameter “LCRB” is equal to the UL RB configuration specified in Table 7.3.2-3 Uplink configuration for reference sensitivity for the PCC DL channel bandwidth.

  - The RBstart is specified to maximize the DL SCC overlap by the lowest IMD order.

For NR-CA intra-band non-contiguous reference sensitivity with one uplink CC:

- The MSD is specified for one DL SCC for which the reference sensitivity is increased by ΔRIBNC.

- Unless otherwise noted, the downlink SCC carrier center frequency shall be configured closer to the uplink operating band than the downlink PCC center frequency.

- The PCC/SCC CBW and the UL band RB allocation are selected to specify the worst-case (highest) SCC MSD level, and the parameter “Wgap” is specified at the maximum value that can be configured to account for the DL band BW, PCC CBW and SCC CBW, i.e. Wgap = DL_BandBW – (PCCCBW + SCCCBW).

#### Guidelines for intra-band contiguous EN-DC reference sensitivity with one uplink carrier:

For intra-band contiguous EN-DC within FR1 with one uplink CC:

- Reference sensitivity requirements are verified with the downlink carrier(s) from the cell group without uplink configured closer to the uplink operating band than any of the downlink carriers from the cell group with uplink.

- The UL/DL CBW and the UL band RB allocation are selected to specify the worst-case (highest) MSD level.

#### 5.5.1.2	Two Uplink CCs

### 5.5.2	Inter-band CA

#### 5.5.2.1	Two-band Inter-band Downlink CA

#### 5.5.2.1.1	One Uplink Band

#### 5.5.2.1.1.1	One Uplink CC

#### Guideline on co-existence studies for harmonic interference and cross-band isolation:

Table 1 summarizes the frequency ranges where harmonics and/or harmonics mixing occur for CA_ nX-nY.

Table 1: UL/DL harmonics/harmonic mixing analysis.

| UL/DL harmonics |  | nX | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow |  |  |  |  |  |  |
| nY | fLow | fHigh |  |  |  |  |  |  |
| DL1 |  |  | N/A |  |  |  |  | UL harmonic |
| DL23 |  |  |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  |  |  | N/A |  | N/A |  |
| DL4 |  |  |  | N/A | N/A | N/A | N/A |  |
| DL54 |  |  |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| UL/DL harmonics |  | nY | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow |  |  |  |  |  |  |
| nX | fLow | fHigh |  |  |  |  |  |  |
| DL1 |  |  | N/A |  |  |  |  | UL harmonic |
| DL23 |  |  |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  |  |  | N/A |  | N/A |  |
| DL4 |  |  |  | N/A | N/A | N/A | N/A |  |
| DL54 |  |  |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 2 summarizes the frequency ranges where cross band isolation may occur for CA_nX-nY.

Table 2 Cross-band isolation analysis

| Bands | nX |  |  | nY |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency limit | flow |  | fhigh | flow |  | fhigh |
| fUL (MHz) |  |  |  |  |  |  |
| fDL (MHz) |  |  |  |  |  |  |
| UL CBW (MHz)2 | Minimum CBW |  | Maximum CBW | Minimum CBW |  | Maximum CBW |
| ACLR1 range | fxULlow-maxULCBWx |  | fxULhigh+maxULCBWx | fyULlow-maxULCBWy |  | fyULhigh+maxULCBWy |
| ACLR1 (MHz) |  |  |  |  |  |  |
| ACLR2 range | fxULlow-2*maxULCBWx |  | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy |  | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) |  |  |  |  |  |  |
| ACLR3 range | fxULlow-3*maxULCBWx |  | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy |  | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) |  |  |  |  |  |  |
| ACLR4 range | fxULlow-4*maxULCBWx |  | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy |  | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) |  |  |  |  |  |  |
| ACLR5 range1 | fxULlow-5*maxULCBWx |  | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy |  | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) |  |  |  |  |  |  |
| Analysis |  |  |  |  |  |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |  |  |

#### Guidelines on handling new “greatest/smallest” CBW due to BCS4/5 [6] for MSD due to hamonic and cross-band isolation interference:

Table 3 and Table 4 indicate cases in which the MSD test point must be re-visited, and in cases where the MSD test point may be revisited on a case-by-case basis for:

- NR-CA combinations when a BCS4/5 request introduces a new greatest CBW or a new smallest CBW,

- EN-DC combinations when a new greatest CBW or a new smallest CBW is introduced in one of the constituent frequency band,

- Legacy BCS4/5 NR-CA combinations when a new greatest CBW or a new smallest CBW is introduced in one of the constituent frequency bands.

Table 3: Handling of MSD test points due to cross-band isolation interference

|  | New greatest CBW | New smallest CBW |
| --- | --- | --- |
| UL band | MUST be revisited | May be revisited on case-by-case |
| DL band | May be revisited on case-by-case | MUST be revisited |

Table 4: Handling of MSD due to harmonic interference (uplink and/or Rx harmonic mixing)

|  | New greatest CBW | New smallest CBW |
| --- | --- | --- |
| UL band | May be revisited on case-by-case | May be revisited on case-by-case |
| DL band | May be revisited on case-by-case | MUST be revisited |

It is recommended that proponents provide justification for cases where the MSD test point may need to be revisited.

#### Handling of legacy UEs that may not support the new greatest/smallest CBW introduced by BCS4/5:

For MSD test points due to harmonic interference and cross-band isolation interference:

- To ensure that the conformance testing of legacy UEs is not impacted, the so-called “first MSD test point” remains unchanged, i.e., it remains the baseline mandatory MSD test point.

- When the MSD test point must be or may be revisited on case-by-case basis, the new MSD test point becomes the second optional MSD test point in the current Release where BCS4/5 is introduced,

- If the second optional test point is already specified, the revisited MSD test point replaces/overwrites the legacy agreed second optional test point.

#### Guidelines for MSD Test Points due to direct-hit harmonic interference:

#### These guidelines apply to both MSD due to UL harmonic interference and harmonic mixing interference.

Guidelines for the DL band configuration:

- For MSD due to direct-hit harmonic interference, a maximum of two test points can be specified:

- the first test point captures the smallest DL channel bandwidth (CBW) MSD,

- the second test point is optional and may capture the greatest DL channel bandwidth (CBW) MSD. For this second test point, other DL configurations are not precluded.

- The “smallest” and the “greatest” CBW shall not exceed the CBW specified in the BCS configuration tables of clause 5.5A.

Guidelines for the UL band configuration:

UL RB allocation:

- The parameter “LCRB” is specified as follows for SCS 15kHz / SCS 30kHz respectively,

- For the UL2/DL1 test point, LCRB = 12RB / 6RB,

- For the UL3/DL1 test point, LCRB = 8RB / 4RB,

- For the UL4/DL1 test point, LCRB = 6RB / 4RB,

- For the UL5/DL1 test point, LCRB = 5RB / 2RB.

- Additionally, LCRB = 25RB / 12RB for UL1/DLx harmonic mixing MSD.

- The parameter “RBstart” is specified with the following footnote:

“The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RBs for the specified UL band channel bandwidth and the UL band subcarrier spacing”.

UL band CBW:

- The UL band is configured with the smallest UL CBW and the smallest SCS that can accommodate the specified UL LCRB,

- The “smallest” and the “greatest” CBW shall not exceed the CBW specified in the BCS configuration tables of clause 5.5A.

Guidelines for the UL/DL harmonic order and collision type:

- The generic notation “ULn/DLm” shall be used, where “m” indicates the DL band harmonic order, “n” indicates the UL band harmonic order, and the source of interference is specified as “direct-hit”. For example, “UL2/DL1 direct-hit” is used to indicate a DL band MSD test point due to direct hit of the UL band 2nd harmonic.

- The following definition of a “direct-hit” collision is captured in a footnote:

“The direct-hit requirements apply when there is at least one individual RE within the uplink transmission

bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the

downlink transmission bandwidth of a victim (higher) band.”

#### Guidelines for MSD Test Points due to near-miss uplink harmonic interference.

- Near-miss MSD test points are specified only for UL2/DL1 harmonic interference and only when the band combination does not meet the direct-hit criteria.

- For near-miss MSD test points:

  - Only one MSD requirement is specified for near-miss,

  - The MSD is specified with:

    - UL CBW = 5MHz,

    - UL LCRB = 25RB (fully allocated),

    - The Note 6 of Table 7.3A.4-1/ Note 6 of TS 38.101-3 Table 7.3B.2.3.1-1 is modified as follows:

#### NOTE 6: The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from  in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively.

Guidelines for MSD Test Points due to cross-band isolation.

Guidelines for the DL band configuration:

- The affected DL band carrier centre-frequency shall be configured closest to the UL band.

- A maximum of two test points can be specified:

- the first test point captures the smallest DL channel bandwidth (CBW) MSD,

- the second test point is optional and may capture the greatest DL channel bandwidth (CBW) MSD. For this second test point, other DL configurations are not precluded to account for say, regional spectrum allocations specificities, or proponent concerns on specific CBW of interest etc...

Guidelines for the UL band configuration:

- UL band CBW:

- The UL band is configured with the highest supported CBW. This ensures that the UL band smallest IMD order has a maximum reach towards the DL affected band.

- UL band carrier frequency and SCS:

- The UL band carrier centre frequency is configured closest to the affected DL band.

- The UL band SCS is specified for the lowest SCS that can be supported for the specified UL CBW.

- UL RB allocation:

- The parameter “LCRB” is that specified in Table 7.3.2-3 (UL configuration for UL Band REFSENS) for the corresponding UL band CBW.

- The parameter “RBSTART” is specified to ensure the UL RBs are positioned closest to the DL affected band.

- For SUL, the requirements are specified with the LCRB of the NR band counterpart as defined in Table 7.3.2-3 (UL configuration for UL Band REFSENS) for the specified SUL band CBW. A SUL-NR counterpart look-up is provided in Table 5.

Table 5: Handling SUL-NR counterpart lookup table

| SUL band | NR UL Band counterpart | FUL_low – FUL_high (MHz) |
| --- | --- | --- |
| n80 | n3 | 1710 – 1785 |
| n81 | n8 | 880 – 915 |
| n82 | n20 | 832 – 862 |
| n83 | n28 | 703 – 748 |
| n84 | n1 | 1920 – 1980 |
| n86 | n66 | 1710 – 1780 |
| n89 | n5 | 824 – 849 |
| n95 | n34 | 2010 – 2025 |
| n97 | n40 | 2300 – 2400 |
| n98 | n39 | 1880 – 1920 |
| n99 | n24 | 1626.5 – 1660.5 |

Guidelines on specifying the “source of interference”:

- The interference source can be either “ACLR1”, “ACLR2”, or greater than ACLR2 (“>ACLR2”).

- For “ACLR1”, or “ACLR2” test points:

  - MSD may be neglected if the order of the IMD product of the modulated RB and its image that overlaps the affected DL band is greater than [9],

  - For RB/Image IMD order less or equal to [7], the MSD needs to be analysed taking into consideration RF-FE complexity (quadplexer, triplexer, etc...),

- For “>ACLR2” test points, aka “flat-band noise cases” / large separation distances:

  - MSD may need to be evaluated if the Tx noise rejection in the DL affected band is less than [40] dB. The underlying assumption is that the flat Tx noise PSD is -130dBm/Hz and DL affected CBW is 5MHz.

#### 5.5.2.1.1.2	Two UL CCs

#### 5.5.2.1.2	Two UL Bands

#### 5.5.2.1.2.1	One UL CC in each band

#### 5.5.2.1.2.2	One UL CC in one band and two UL CCs in the other band

#### 5.5.2.2	Three-band DL CA

#### 5.5.2.2.1	Two UL Bands

#### 5.5.2.2.1.1	One UL CC in each band

###### 5.5.2.2.1.2   One UL CC in one band and two UL CCs in the other band

******************************* END of TP to RAN4 PRD 0.2.0 *******************************

# Conclusion

To the best of our knowledge, this contribution summarizes the most up-to-date guidelines on MSD requirements for 1UL CC configurations. Thus, porting the text proposal 1 into the RAN4 PRD is proposed.

# References

- R4-2413063 UL harmonic MSD clean-up, 3GPP TSG RAN Meeting #112, Maastricht, The Netherlands, Skyworks Solutions, Inc.

- R4-2410651 WF on Harmonic MSD clean-up, 3GPP TSG RAN Meeting #111, Fukuoka, Japan, Skyworks Solutions, Inc., AT&T, Qualcomm.

- R4-2501987 On near-miss for Rx harmonic mixing, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Anritsu, Qualcomm France, Oppo.

- R4-2419725 Input to PRD clause 5.5 and TP on rules for harmonic MSD, 3GPP TSG RAN Meeting #113, Orlando, US, Skyworks Solutions, Inc.

- R4-2210565 WF on criteria on Rel-17 enhanced MSD table format, 3GPP TSG RAN Meeting #103e, electronic meeting, Skyworks Solutions, Inc.

- R4-2310321 WF on guidelines on checks/specification updates required when BCS4/5 introduces a new smallest or largest CBW, 3GPP TSG RAN Meeting #107, Incheon, South Korea, Skyworks Solutions, Inc.

- R4-2314864 WF on intra-band contiguous UL-CA MSD, 3GPP TSG RAN Meeting #108, Toulouse, France, Skyworks Solutions, Inc.

- R4-2501994 1UL CC MSD clean-up for intra-band non-contiguous NRCA, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc., T-Mobile USA.