3GPP TSG-RAN4 Meeting #114	R4-2503027

# Athens, GR, 17th – 21st February, 2025

Source:	vivo

Title:	TP for TR 38.774 on LP-WUS UE RF

Agenda item:			7.26.2.1

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.774 to capture a few general agreements for LP-WUS RF from WF [2-5].

2. Reference

[1] TR 38.774 v0.1.0

[2] R4-2417112, WF on UE RF requirements for LP-WUS, vivo, RAN4#112bis

[3] R4-2414309, WF on LP-WUS UE RF requirements, vivo, RAN4#112

[4] R4-2410569, WF for 111136 NR_LPWUS_UERF, vivo, RAN4#111

[5] R4-2420528, WF on UE RF requirements for LP-WUS, vivo, RAN4#113

[6] Renzhi Liu, et al., An 802.11ba-Based Wake-Up Radio Receiver with Wi-Fi Transceiver Integration, IEEE JOURNAL OF SOLID-STATE CIRCUITS, VOL. 55, NO. 5, pp. 1151-1164, MAY 2020

# 3. Text Proposal

<Start of Text Proposal>

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Main radio (MR): the Tx/Rx module operating for NR signals/channels apart from signals/channel related to low-power wake-up.

LP-WUR (LR): The Rx module operating for receiving/processing signals/channel related to low-power wake-up.

<End of Text Proposal>

<Start of Text Proposal>

5	System parameters

5.1	General

The channel arrangements presented in this clause for LP-WUS are based on the operating bands and channel raster defined in sub-clause 5.2 and 5.3 below.

5.2	Operating bands

LP-WUS is designed to operate in the operating bands defined in Table 5.2-1 of TS 38.101-1 [3] and TS 38.101-2 [4].

5.3	Channel raster

The PRB grid between LR and MR is aligned and the LP-WUS RBs can be flexibly allocated within the wider NR carrier. How to make sure the RB alignment between LP-WUS RB and MR RB grid is UE implementation specific.

<End of Text Proposal>

<Start of Text Proposal>

6.2	Simulation assumption

6.2.1	General

To evaluate the RF performance of LP-WUS/WUR, the link level simulation is performed in this WI. This clause captures the simulation assumption for LP-WUS UE RF and BS RF analysis.

6.2.2	Simulation parameters

For RF performance analysis, the following simulation assumptions are considered.

Table 6.2.2-1 Link level simulation assumption for FR1 LP-WUS

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |  |  |
| Channel structure | Total 8/16 bits |  |  |
| Chip rate | M=1 | M=1/2/4 |  |
| Coding | 1/2 rate Manchester coding |  |  |
| Time error | 0 |  |  |
| Residual Frequency error | 0/10/20 ppm |  |  |
| SCS | 30kHz |  |  |
| UE Channel BW | 20MHz (51 RB)-case 1 10MHz (24 RB)-case 2 5MHz (11 RB)-case 3 |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |  |  |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |  |  |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [20~33] dB NOTE: decide the interference level depending on SNR |  |  |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS |  |  |
| Sampling rate | 7.68MHz |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |  |  |
| Non-linearities | Not modelled |  |  |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |
| Channel Model | Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  |
| Performance metric | - 1% MDR/BLER as baseline and 5% MDR/BLER as optional - The following false alarm rate can be considered - 1% Providing the information whether the false alarm rate is considered or not |  |  |
| Note: the detailed Channel structure for LP-WUS is under discussion in RAN1, some parameters may be updated accordingly. |  |  |  |

<End of Text Proposal>