3GPP TSG-RAN WG4 Meeting #114	R4-2503028

Athens, GR, 17th – 21st February, 2025

Agenda item:			7.26.5

Source:	vivo

Title:	WF on UE RF requirements for LP-WUS

Document for:	Approval

Introduction

This is way forward for Rel-19 LP-WUS UE RF.

Topic #1: General and system parameters

Sub-topic 1-1 General

Issue 1-1-1: How to define two LP-WUR types

Agreement:

- Define two set of RF requirements as the baseline, considering the following aspects including

  - Noise figures

  - Interference performance

  - Modulation

- Consider to merge two sets of requirements into one if the requirements are close to each other

Sub-topic 1-2 System parameters

Issue 1-2-1: Whether operation band clause for LP-WUS/WUR is needed (5.2M	Operating bands for LP-WUS)?

Agreement:

- Operation band clause for LP-WUS/WUR is needed in Rel-19

  - Keep the band with DL operating band defined, and SDL band is precluded

Issue 1-2-2: Whether Channel bandwidth clause for LP-WUS/WUR is needed (5.3M Channel Bandwidth for LP-WUS)?

Agreement:

- [Transmission bandwidth configuration] for LP-WUS/WUR signal is needed to specify.

Issue 1-2-3: Whether Channel raster clause for LP-WUS/WUR is needed (5.4M Channel raster for LP-WUS)?

Way Forward:

  - No update of core spec for this part.

Topic #2: REFSENS, ASCS and ACS requirements

Sub-topic 2-1 FR1 SNR, NF and IM for REFSENS requirements

Issue 2-1-1: SNR, NF and IM value for FR1 LP-WUR

Agreement:

- For the FR1 requirements targeting at bands <2.5GHz

  - The IM+NF values are

    - Set 1: 18dB

    - Set 2: 13.5dB

- Send an LS to RAN1 with RAN4 decision of two sets of : NF (assume IM is 2.5dB)

- Ask RAN1 to check whether coverage target can be met based on above NF information. And ask for detailed signal configuration (e.g., chip rate, length of sequence) of LP-WUS.

- LS should also cc RAN2

Issue 2-1-3: Timing error impacts for OOK-based and OFMD-based SNR

Agreements

- Timing error contribution to RAN4 RF simulation can be contribution driven.

Sub-topic 2-2 FR2 SNR, NF and IM for REFSENS requirements

Issue 2-2-1: Combining strategy for FR2 LP-WUR

Agreements:

  - For Rel-19, confirm no combination gain across two Rx chains is considered for FR2 LP-WUR. FFS whether and how to capture the impacts of Rx selection in REFSENS evaluation.

  - Confirm 1Rx at baseband for FR2 LP-WUR.

Issue 2-2-2: NF for FR2 LP-WUR

Way forward:

  - Adopt similar approach to FR1 LP-WUR, encourage companies to input analysis on IM, NF and SNR together.

Issue 2-2-3: Spherical coverage for FR2 LP-WUR

Way forward:

  - RAN4 further discuss how to specify spherical coverage requirements for FR2 LP-WUR.

Sub-topic 2-3 ASCS simulation and requirements

Issue 2-3-2: in-band power level of ASCS Test case

Agreements:

- For ASCS, allowable de-sense due to introduction of NR RBs is [0-1dB].

- [0] guard RB is the baseline.

Issue 2-3-3: channel bandwidth of ASCS Test case

Agreements:

- For ASCS core requirements, it should be applied to all NR CBW.

- For conformance test, for NR CBW can be further discussed and down-selected.

- For ASCS test configuration, the NR RBs should be fully allocated with the same PSD of LP-WUS within the channel except for 11 LP-WUS RBs and potential guard RBs (if any).

Sub-topic 2-4 ACS simulation and requirements

Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB

Agreements

- Same interference signal level as MR for LP-WUR ACS requirements

  - FFS at least for one set of Rx requirements.

- The LP-WUS level is LP-WUS REFSENS+[X]dB, X=14 as starting point

  - FFS whether two sets of LP-WUS wanted signal level can be defined.

- Final ACS will be decided by LP-WUR REFSENS and wanted signal relaxation

- FFS number of guard RBs.

Issue 2-4-2: ACS requirements (same interference) and required guard RB

Way forward:

- FFS, also depends on discussion for Issue 2-4-1

Issue 2-4-3: ACS requirements (lower interference level) and required guard RB

Way forward:

- FFS, also depends on discussion for Issue 2-4-1

Issue 2-4-3: Case 1 ACS test parameter

Way forward:

- FFS LPWUS ACS test case.

- ACS requirement applies to all NR CBW

Topic #3: Other RF requirements

Sub-topic 3-1 IBB, OBB and Intermodulation requirements for UE RF

Issue 3-1-1: IBB, OBB requirements

Way forward:

- Same interference level as MR

  - FFS at least for one set of Rx requirements.

  - FFS on wanted signal in blocking test.

Issue 3-1-2: detailed table for test parameters of IBB and OBB test case

Way forward:

- Encourage companies to provide input and analysis on IBB and OBB requirements

Sub-topic 3-2 Other Rx requirements

Issue 3-2-2: Maximum input level requirements for FR1 LP-WUR

Way forward:

- FFS.

Issue 3-2-3: Maximum input level test case for FR1

Way Forward:

  - Further check and decide whether the max. input power level is specified for all channel bandwidths.

Issue 3-2-4: spurious response requirements for FR2

Way Forward

  - Further check next meeting on whether need to specify spurious response requirements for FR2 LP-WUR.

Topic #4: Testability issues

Sub-topic 4-1 Testability for UE RF requirements

Issue 4-1-1: General framework for LP-WUR testability

Way Forward:

    - FFS

Issue 4-1-2: Separate RF test case for idle and connection mode

Way forward:

- RAN4 further discuss and check whether all UE types can support IDLE and/or CONNECTED mode?