3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503114

Wuhan, China, 07th – 11th April. 2025

3GPP TSG-RAN WG2 Meeting #129	R2-2501388

Athens, Greece, Feb. 17th – 21st, 2025

Title:	Reply LS on LP-WUS subgrouping

Response to:	R2-2500050/S2-2412876

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	RAN2

To:	SA2, RAN3, CT1

Cc:	RAN1, RAN4

Contact Person:

#### Name:	Yiru Kuang

E-mail Address:	kuangyiru@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 thanks SA2 for the LS in S2-2412876 and provides the following feedback:

- On the terminology and naming of “LP-WUS”:

It’s not necessary that LP-WUS is transmitted with high power. Higher power is just one possible way to ensure the coverage performance. The terminology has been used since R18 Study Item on LP-WUS and the terminology is clear in RAN without any misunderstanding. Thus, RAN2 confirmed that the “LP-WUS” terminology can be used.

- On the issue of UE radio capability for paging information, RAN2 had the following agreement:

- Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container).

Whether there is issue for Release 15 gNB and Release 16 gNB and for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the coming RAN2 meetings based on company contributions.

2. Actions:

To SA2, RAN3, CT1

RAN2 respectfully asks SA2, RAN3, CT1 to take the above answer into account.

3. Date of Next TSG-RAN WG2 Meeting:

3GPP RAN2#129bis	                       07 - 11 April, 2025		        Wuhan, China

3GPP RAN2#130	                       19 - 23 May, 2025		        St Julian, Malta