3GPP TSG-RAN WG4 Meeting #114bis	R4-2503140

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.18.2

Source:	MediaTek Inc.

Title:	Discussion on UE requirements with 8Rx MMSE-IRC receiver

Document for:	Discussion

# Introduction

A WF for UE 8Rx MMSE-IRC receiver requirements was agreed upon at the RAN4#114 meeting [1]. In this contribution, we share our perspectives on the remaining issues.

- Discussion

## 2.1 General

Issue 1-1-1: Definition of MMSE-IRC receiver capability for 8Rx UE Agreement: - If agreed to change the receiver definition, it also needs to be changed in Rel-18 maintenance. - If receiver definition is agreed to be changed, an LS will be sent to RAN2 for updating TS38.306. Proposals: - Option 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 2: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 3: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE declares supporting SU-MIMO 8Rx receiver capability Issue 1-1-2: 8Rx UE MMSE-IRC receiver coverage Agreement: - Criteria for decision if one or two sets of PDSCH requirements are to be introduced: - Secure the span is <=2.5 dB for each receiver type by removal of outliers - If difference in the average of each receiver type for a specific configuration is > [TBD:limit] then define two sets of requirements for the specific configuration. - Further discuss the actual value for the [limit]. - Proposals: - Option 1: Set [limit] to 0.5dB - Option 2: Set [limit] to 1.0dB - Option 3: Set [limit] to 1.2dB - Other options not precluded.

Based on the simulation results presented in RAN4#114 [2], a significant performance disparity is observed between the baseline and the simplified receiver. In terms of establishing a "threshold" for deciding whether to adopt two requirement sets, we recommend a value of 1.0dB. This threshold is consistent with the one used to distinguish between different receiver implementations, such as MMSE and MMSE-IRC, during the discussions for Release 17. Consequently, we advocate for the creation of two distinct sets of requirements, mirroring the approach taken by RAN4 in the Release 18 8Rx Work Item. Given the introduction of two requirement sets, we propose to revise the definition of the MMSE-IRC receiver capability for 8Rx UEs, opting for Option 1.

Proposal 1: Introduce two set of requirements if difference in the average of each receiver type for a specific configuration is larger than 1.0dB.

Proposal 2: Update the definition of MMSE-IRC receiver capability for 8Rx UE as:

- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

## 2.2 PDSCH requirements with intra-cell inter-user interference

Issue 1-3-1: MCS for Rank 2+2 Agreement: - For Rank 2+2, use MCS17 for requirement definition. Issue 1-3-2: Rank 4+4 Proposals: - Option 1: Do not define Rank 4+4 requirements. - Option 2: Define requirements for Rank 4+4 MCS 13 for baseline receiver.

The remaining issue for PDSCH requirements with intra-cell inter-user interference is whether to introduce requirements for the Rank 4+4 scenario. We do not think Rank 4+4 is a typical scenario in real filed. To support Rank 4+4 MU-MIMO transmissions, both UEs must have channel conditions that support four spatial streams with sufficient orthogonality. Finding two UEs with such ideal channel conditions simultaneously can be difficult. Besides, it is also a challenging task for a gNB to identify suitable UE pairs for rank 4 MU-MIMO transmissions.

Proposal 3: Regarding PDSCH requirements with intra-cell inter-user interference, we propose not to define Rank 4+4 requirements.

## 2.3 CQI requirements with inter-cell interference

At RAN4#113 meeting, RAN4 agreed to reuse the same test metrics from Rel-17 2/4Rx wideband CQI reporting with inter-cell interference, which are:

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ g .

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Based on our simulation results [3], the BLER is greater than 0.02 for all SINR values for both rank 1 and rank 2. Besides, it is possible to find some SNIR point for rank 1 and rank 2 with gamma larger than 2. Therefore, aaccording to the agreement at the last meeting, we propose using rank 2 to define requirements.

Issue 1-4-2: Rank for serving cell Agreement: - If simulation results show both rank 1 and rank 2 to be feasible for requirement definition, then Rank 2 will be selected.

As for the SINR testing point, gamma value and BLER, we propose to use SINR = -1 dB, gamma = 2 and BLER = 0.02.

| Figure 1. Throughput ratio (gamma) versus SINR | Figure 2. BLER versus SINR |
| --- | --- |

Proposal 4: Use Rank 2 in the serving cell to define CQI requirements.

Proposal 5: Use SINR = -1dB, gamma =2 and BLER = 0.02 to define CQI requirements.

## 2.4 Applicability rule and release independent

Issue 1-5-1: Test applicability to skip 2/4Rx requirements Proposals: - Option 1: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not. - Option 2: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. TBD: Clarification of what “corresponding 8Rx requirements” covers. Issue 1-5-3: Release independency Proposals: - Option 1: Discuss release independence requirements at a later stage in the WI. - Option 2: The new Rel-19 requirements for 8Rx should be release independent from Rel-17. - Option 3: The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17. - Option 4: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-18. - Option 5: The requirement should at least be released independent from Rel-18 if no new signalling or UE capability introduced.

Given that processing for 8Rx is more complex compared to 2Rx/4Rx, we support Option 1 as it reduces the testing load on the UE.

Proposal 6: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

For release independent, we support new requirements for 8Rx to be release independent from Rel-18.

Proposal 7: The performance requirements for 8Rx MMSE-IRC receiver to be release independent from Rel-18.

# Conclusion

In this contribution, we present our views on UE requirements for devices equipped with an 8Rx MMSE-IRC receiver. Our proposals are outlined below:

Proposal 1: Introduce two set of requirements if difference in the average of each receiver type for a specific configuration is larger than 1.0dB.

Proposal 2: Update the definition of MMSE-IRC receiver capability for 8Rx UE as:

- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Proposal 3: Regarding PDSCH requirements with intra-cell inter-user interference, we propose not to define Rank 4+4 requirements.

Proposal 4: Use Rank 2 in the serving cell to define CQI requirements.

Proposal 5: Use SINR = -1dB, gamma =2 and BLER = 0.02 to define CQI requirements.

Proposal 6: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 7: The performance requirements for 8Rx MMSE-IRC receiver to be release independent from Rel-18.

# Reference

- R4-2502275, Way forward for [114][324] NR_demod_Ph5_Part2_UE, Nokia

- R4-2500267, Simulation result summary for ICI and MU-MIMO with 8RX, Apple

- R4-2503141 Simulation results for UE demodulation requirements with 8Rx MMSE-IRC receiver, MediaTek