3GPP TSG- WG4 Meeting # 114bis	R4-2503153

# Wuhan, China, April 07 – 11, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on HPUE MSD

Agenda Item:	7.1.1.1.1

Document for:	Approval

Introduction

In RAN4#114, it was agreed that

Agreement: - Eligibility criteria to HPUE MSD requirements specified using LUTs: - A valid 1UL HPUE configuration is specified in the two-band inter-band CA configuration tables (Table 5.5A.3.1-1a ~ 5.5A.3.1-1n), and, - A default power class 3 (1Tx) 1UL MSD test point is specified, and, - A 1UL HPUE MSD test point is not already specified in either Table 7.3A.4-2a, Table 7.3A.4-4a-1, Table 7.3A.4-4b, Table 7.3A.6-1a-1, or Table 7.3A.6-1b.

And it was almost come to consensus that

When the CR for HPUE MSD is implemented (after RAN#109) - All specified PC2 MSD tables and PC1.5 MSD tables remain untouched in TS 38.101-1 and TS38.101-3

This paper provides analysis of HPUE MSD simplification methods based on the assumptions above.

In RAN4#113, the WF[1] for MSD rules was approved with simplifications to replace the existing HPUE MSD tables.

Agreement: - For new Rel-19 inter-band CA combinations within FR1, and for MSD due to harmonic, receiver harmonic mixing and cross-band isolation interference, the 1UL HPUE MSD requirements are specified using a delta MSD relative to the default power class 3 MSD as: PCx MSD = PC31Tx MSD + PCx, where x = 2 for PC2 or x=1.5 for PC1.5, - Companies are invited to evaluate two options, - option 1: RAN4 keeps on specifying two sets PC2 requirements [4,5]: - one LUT/equation for PC2 1Tx: PC21Tx MSD = PC31Tx MSD + PC21Tx, e.g. plateau at ~3dB, - one LUT/equation for PC2 2Tx: PC22Tx MSD = PC31Tx MSD + PC22Tx, e.g. plateau at ~8dB, - option 2: RAN4 specifies a single set PC2 requirement [3]: - one PC2 LUT/equation PC2 MSD = PC31Tx MSD + PC2, e.g. plateau at ~6dB. - To ensure that 1UL PC1.5 MSD is greater than 1UL PC2 2Tx MSD, companies are invited to derive the HPUE delta MSD LUT/Equations values assuming: - a [9dB] 1Tx Main/Div interference imbalance. Imbalance assumptions less than 9dB are not precluded, - [0.5]dB RevIM (Rtx) and 3dB interferer increase for harmonic/harmonic mixing and for crossband isolation. . - Handling of band combination without MSD for PC3 but with small or moderate PC2/PC1.5 MSD: - option 1: FFS LUT rule and FFS on test point configuration - option 2: Legacy HPUE MSD tables may still be used to capture an exception MSD test point - option 3: no HPUE MSD requirement for those cases. - For proponents of LUT simplification approach, FFS the LUT granularity (e.g. 1dB steps) Agreement: - Companies are invited to - Provide 2UL HPUE MSD simplification rules to cover the Rel-19 supported 2UL configurations using either equations/LUT or single value de-ratings relative to the default power class 3 MSD, - Study means to minimize the number of LUTs / set of equations that cover all cases. - Check if all Rel-19 cases have been correctly captured and to look at the initial proposals in [4] and [7]. - FFS if only IMD orders are sufficient to design LUT/ set of equations or if both the IMD order and the IMD indexes should be accounted for. - FFS to account for other IMD contributors than just the power amplifier.

This paper is to provide further thinking concerning the HPUE MSD rules in Rel-19.

Simplification for 1UL HPUE MSD

General consideration

An example LUT table was proposed in last RAN4 meeting during the offline discussion. The basic idea is to use the unified LUT for all 1UL MSD types in terms of PC2 and PC1.5 with different Tx architectures.

Table 1 (Example): 1UL PC21Tx , PC22Tx , PC1.5 LUT

| PC3 MSD (dB) | PC21Tx (dB) | PC22Tx (dB) | PC1.5 (dB) |
| --- | --- | --- | --- |
| <1 | 0.6 | 0.9 | 2.0 |
| 1 | 0.8 | 1.2 | 2.7 |
| 2 | 1.2 | 2.1 | 4.1 |
| 3 | 1.6 | 2.7 | 5.1 |
| 4 | 1.8 | 3.3 | 5.9 |
| 5≤ <7 | 2.0 | 4.0 | 7.0 |
| 7≤ ≤9 | 2.5 | 5.0 | 8.0 |
| ≥10 | 3.0 | 6.0 | 9.0 |

In general, we support the use of a simplified LUT for HPUE MSD, However, we are uncertain whether the same LUT can be universally applied across different MSD types, as our previous studies indicate that the underlying mechanisms and determining factors for final MSD values may vary considerably. Therefore, in the subsequent analysis, we will examine typical band combinations for harmonic, harmonic mixing, and crossband isolations with both 1Tx and 2Tx configurations. This examination aims to determine whether a unified LUT can adequately align with existing requirements or if modifications to the LUT proposal are necessary.

It is impractical to explore all possible reference architectures for every band combination to establish a definitive rule for HPUE MSD, especially given that not all MSD requirements in the specifications are accurate. Nonetheless, we can analyze trends by increasing output power while keeping the primary parameters involved in MSD calculations constant. This approach will help us identify whether a unified LUT is feasible or if adjustments are required to accommodate different MSD types and their specific characteristics.

For MSD types, the following cases are defined with specific HPUE requirements,

- Harmonic MSD:

  - PC2 1Tx, PC2 2Tx (mainly for FDD bands)

- Harmonic mixing MSD:

  - PC2 1Tx, PC2 2Tx, PC1.5 2Tx

- Crossband isolation MSD:

  - PC2 1Tx, PC2 2Tx, PC1.5 2Tx

Currently, there are no PC1.5 HPUE MSD requirements for harmonic cases. However, such scenarios may be addressed in future releases. If the HPUE LUT is adopted, it should be designed to accommodate potential future cases, ensuring its applicability as specifications evolve.

HPUE Harmonic

As per R4-2502864, for Rel-19 inter-band CA combinations within FR1 introduced after RAN#109, the 1UL CC HPUE MSD requirements can be specified using a delta MSD “PCx” relative to the default power class 3 MSD as:

PCx MSD = PC3 MSD + PCx (Equation 1)

Currently, only analysis for PC21Tx and PC22Tx can be referred to. No requirements for PC1.5 harmonic MSD until now.

Example architectures for HPUE harmonic MSD with 1Tx and 2Tx

Here CA_n3A-n78A is used to see the impact of 1Tx and 2Tx for harmonic MSD.

As per [8], different UE RF front-end architectures result in different value of power imbalance, which is calculated by PC2 MSD to PC3 MSD. And PC21Tx is not significantly impacted by different power imbalance.

One of the possible UE architectures illustrated for 1Tx harmonic is as follows in Figure 1. The H2 at LNA (HB n78) in main path and diversity path from PC2 PA (LB n3) via PCB isolation could be the same, while H2 at LNA from Tx to Rx chain main & diversity path routed from Xplexers/filters and/or antenna are quite different. Both the factors are impacted by the PA output power. PC3 to PC2 1Tx with 3dB increasement lies in the PC21Tx of 3dB with margin averaged from the calculations by companies.

Figure 1 Illustration for 1Tx PC2 harmonic

Observation 1: Across different interference paths, the power imbalance between the main and diversity branches tends to be lower than the typical antenna isolation value of 10 dB.

Observation 2: For 1Tx PC2 harmonic case, the increased output power is reflected in the same level of increased interferences to both the main and diversity paths, thus 3dB MSD degradation is expected.

One of the UE architectures illustrated for 2Tx harmonic is as follows in Figure 2. There are two PC3 PAs with one in main path and the other in diversity path. The link analysis from band n3 PA in main path to band n78 LNA in main & diversity path is same as that for PC3 harmonic MSD. And quite similar to band n3 PA in diversity path to band n78 LNA in main & diversity path. The composite H2 interference at LNA port in main path and diversity path can be considered as same.

Figure 2 Illustration for 2Tx PC2 harmonic

Observation 3: It is obvious from our calculation that the HPUE harmonic MSD values are quite different for 1Tx and 2Tx configurations.

Observation 4: For 2Tx PC2 harmonic case, when the composite H2 interference at the LNA port is equal in both the main and diversity paths (i.e., no interference imbalance between the paths), the calculated MSD is approximately 7 dB higher compared to PC3 1Tx for the example band combination in our analysis.

Observation 5: It is observed that for most harmonic cases, the PC3 MSD values are larger than 7dB.

In RAN4#113 WF [1], it was discussed that whether the two sets PC2 requirement considering both PC21Tx and PC22Tx should be specified for harmonic. It was proposed that PC21Tx = 3dB, PC22Tx= 8dB for harmonic MSD can be applied to all of the HPUE Harmonic MSD. According to above analysis, the proposed values are aligned with our consideration.

Proposal 1: Consider to define two values for PC2 harmonic requirements with PC21Tx and PC22Tx.

Proposal 2: The values for harmonic MSD could be PC21Tx = 3dB, PC22Tx= 7~8dB respectively.

Though there are no PC1.5 2Tx harmonic MSD defined yet, for the same architecture in Figure 3, if keeping all parameters unchanged but with increased 3dB power compared to PC2 2Tx, the expected PC1.5 MSD would be further increased by 3dB.

Proposal 3: The harmonic MSD for PC1.5 2Tx should be future proof, which could be 3dB higher than PC2 2Tx.

Simplification for HPUE Harmonic mixing

For harmonic mixing with the order of UL1/DLx, the uplink is fundamental harmonic, and DLx is up to the harmonic rejection of local oscillator. PCx MSD = PC3 MSD + PCx (Equation 1).

Example architectures for HPUE 1Tx and 2Tx

Here CA_n3A-n78A is also used to see the impact of 1Tx and 2Tx for harmonic mixing MSD.

Only the different output power should be considered for 1Tx PC2 harmonic mixing as in Figure 3.

Observation 6: Harmonic mixing MSD values for UL1/DLx are relatively high, and most of them are above 5dB for PC3.

Observation 7: For 1Tx PC2 harmonic mixing case, ~3dB increasement of MSD is observed for most UL1/DLx cases.

For 2Tx HPUE harmonic mixing cases, theoretically, it should be similar to that of the harmonic cases, at least for UL1/DLx. As illustrated in Figure 4, the output from PA in main path is same as that in diversity path, the resulted MSD should be larger than 6dB compared to 1Tx owing to the different interference imbalance. However, the increased MSD for PC1.5 2Tx is just 5~6dB compared to PC3 MSD. And there is only one PC2 2Tx case for harmonic mixing with increased MSD of 7.7dB, which is similar to harmonic MSD analysis.

Observation 8: For 2Tx PC2 harmonic mixing case, 7~8dB increasement of MSD is observed for UL1/DLx cases, which is aligned with harmonic case.

Observation 9: For 2Tx PC1.5 harmonic mixing case, 5~6dB increasement of MSD is observed for UL1/DLx cases, which is even lower than the case of 2Tx PC2. Similar rule for harmonic cases may not be accurate for 2Tx PC1.5.

Referring to Table 10 in Annex, for the harmonic mixing order of ULx/DLy, there are no obvious rules could be observed. Similar to UL1/DLx, most 2Tx cases are for PC1.5. But the values in the spec are quite diversified.

Observation 10: For ULx/DLy harmonic mixing, most HPUE MSD cases are for 1Tx PC2 and 2Tx PC1.5. No obvious rule could be identified, which are not aligned with the analysis of harmonic MSD and harmonic mixing for UL1/DLx.

Figure 3 Illustration for 1Tx PC2 harmonic mixing

Figure 4 Illustration for 2Tx PC2/PC1.5 harmonic mixing

Proposal 4: It is proposed to investigate whether the harmonic mixing rules applicable to UL1/DLx configurations can be extended to ULx/DLy scenarios.

Simplification for HPUE cross band isolation

For cross band isolation, one uplink transmission is assumed. The MSD is highly depended on filter/Xplexer capability for different bands.

For cross-band interference source of ACLR1 and ACLR2 and even some cases >ACLR2, the dominant component to MSD value is IM2 induced by IIP2. The IM2 product generated after the non-linear receiving channel falls at twice the uplink transmission bandwidth near direct current, degrading maximum sensitivity in DL. The value of IM2 product is PIM2=2Pin-IIP2. The PA output power also impact noise induce by Rx reciprocal mixing and Tx LO phase noise. PC21Tx = 3dB can be considered for cross-band interference source of ACLR1/ACLR2, which is aligned with the existing requirements, though it is not obvious if we just take IM2 interference into account.

Observation 11: PC21Tx = 3dB can be considered for cross-band interference source of ACLR1/ACLR2.

For cross-band interference source > ACLR2, the total interference calculated by couple noise from Tx thermal, IM2 induced by IIP2, noise induced by Rx reciprocal mixing and Tx LO phase noise is quite closed to noise floor. Given that PA noise floor at far end is flat, for those band combinations with Rx frequency range of one band far away from ACLR2 region, the MSD value would be relatively low, such as lower than 2dB. The increasement of PA output power has limited impact to the PC. PC21Tx < 3dB should be considered, for example, 1dB or so.

However, for some band combinations with Rx frequency range of one band close to ACLR2 region if wide channel BW is allocated, e.g. 100MHz CBW for n40, n41 and n78, n79. The PC21Tx value similar to that for ACLR2 can be applied. The figure below shows the typical PA performance with both adjacent channel leakage as well as the far end noise floor which are widely used in the cross band isolation analysis.

Figure 5: Example PA performance

It is noted that the cross-band isolation MSD depends on the proportion of different interference components falling within the victim bands, e.g., IM2, Tx residual noise floor after filter rejection, etc. These cases may be more complicated than harmonic and harmonic mixing. We observed that for a specific band combination with a >ACLR2 MSD greater than 20 dB, regardless of whether the output power is increased by 3 dB or 6 dB, the MSD value remains almost constant for one of the constituent bands, while MSD changes obviously with increased output power for the other victim band for the same combination.

Additionally, we see cross-band isolation MSD includes most small value cases, which makes the analysis more complicated.

Observation 12: Cross-band isolation covers most cases of small value MSDs.

Observation 13: PC21Tx = 3dB could be considered for cross-band interference source of > ACLR2 for all available cases, but not that accurate.

Observation 14: For cross-band isolation HPUE MSD with 2Tx PC2 and PC1.5, no obvious rules could be observed.

Observation 15 : Define separate LUTs could be more accurate for band combinations with cross band isolation source of ACLR1/ACLR2, and with Rx frequency range of one band close to and far away from ACLR2 region.

Proposal 5: For cross band isolation source of ACLR1/ACLR2: PC21Tx = 3dB, PC22Tx = 6dB, PC1.5 = 9dB.

[for cross-band interference source > ACLR2:

- PC2 = 3dB, PC1.5 = 6dB for band combinations with Rx frequency range of one band close to ACLR2 region.

- PC2 = 1dB, PC1.5 = 2.5dB for band combinations with Rx frequency range of one band far away from ACLR2 region.]

Based on the above analysis and observations, it is evident that theoretical rules can be established for certain cases, although some of these rules may rely on simplified assumptions. Nevertheless, the derived HPUE MSD requirements should remain theoretically sound. We acknowledge that it is difficult to derive a generic LUT which could be accurately applicable to all MSD types and cases. The starting point of the HPUE MSD work is to derive applicable rules upon the existing requirements, though some of them are correct by themselves, we are open to have a simple LUT table for all MSD types in terms of PC2 1Tx, PC2 2Tx and PC1.5 2Tx.

Simplification for HPUE IMD

In [2][3][4][5], plenty of impressive observations and various ways to simplifying the HPUE IMD are presented, which is quite helpful to analyse the HPUE MSD simplification issues.

When calculating the IMD MSD value, the parameters listed in Table 2 are unchanged for different power class in CA_nA-nB.

Table 2 Parameters for MSD analysis (CA_nA-nB)

| Parameter | value |
| --- | --- |
| IP2/3/4/5: Antenna switch (dBm) | 115/68/55/53 |
| IP2/3/4/5: Duplexer (dBm) | 100/74/55/53 |
| IP2/3/4/5: PA forward (dBm) | 28/30/28/27 |
| IP2/3/4/5: PA reverse (dBm) | 38/28/32/32 |
| IP2/3/4/5: LNA (dBm) | 6/-6/-6/-10 |
| Antenna isolation (dB) | 10 |
| PCB isolation (dB) | 65 |
| Diplexer isolation (dB) | 15 |
| L-H diplexer IL (dB) | 0.5 |
| M-H diplexer IL (dB) | 1 |
| Antenna switch IL (dB) | 1 |
| Duplexer IL (dB) | 4 |
| Band nA Tx-Rx duplexer rejection (dB) | xx |
| Band nB Tx-Rx duplexer rejection (dB) | xx |
| PA gain | 25 |
| nA Channel BW (MHz) | 5 |

The coefficient related to IMD order for each uplink component applies to the value of diplexer, antenna switch, duplexer for both main path and diversity path, and band A/B PA forward /reverse mixing, resulting in MSD value for the band combination. Compared to PC3 IMD calculation, the main difference lies in output power for the two uplink components with PC2/PC1.5 in one band combination. It is noted that for a band combination with two bands, it could be implemented either with 2Tx or 3Tx.

Table 7.3A.5-1: IMD for PC3 CA NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 Table 7.3A.5-1a: IMD for PC2 CA NOTE 1: Both of the transmitters shall be set min(+23 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 Table 7.3A.5-1b: IMD for PC1.5 CA NOTE 1: This band combination is specified for inter-band UL CA with UL MIMO or Tx diversity capabilites, and the transmitter shall be set at min (+23 dBm, PCMAX_L,f,c) for the band with single Tx antenna connector as defined in clause 6.2A.4, and set at min (+27.8 dBm, PCMAX_L,f,c) for the band with two Tx antenna connectors as defined in clause 6.2H.3 or 6.2L.3.4.

For PC2 IMD MSD, it was agreed in Rel-18[7] that the requirements are same for 3Tx and 2Tx.

- 3Tx MSD for PC2 IMD: The requirements specified based on 1Tx-1Tx UL configuration are applicable for 1Tx-2Tx UL configuration. - It is allowed for companies to check the requirements for particular cases. - 23+27.8 is used as the power configuration in the 3Tx PC1.5 IMD calculation where band A is 23dBm and band B is 24.8dBm+24.8dBm with 2Tx

For PC 1.5 IMD with 3Tx, according to the RF front end architecture in [6], compared to 2Tx PC2 MSD analysis, there is one more forward IMD and reverse IMD from the 2nd PA of band n78 in the other branch for 3Tx PC1.5 CA_n3-n78. The isolation of antenna, PCB and diplexers, etc, between the two branch is quite large, and there is barely a little contribution from the 2nd PA to the PC1.5 MSD value. Therefore, only 24.8dBm output power from the band with 2Tx is considered.

Figure 6 IMD MSD for CA_n3-n78 with 1Tx or 2Tx @ n78

Observation 16: For 3Tx IMD analysis, there is barely a little contribution from the 2nd PA to the PC1.5 MSD value. Therefore, only 24.8dBm output power from the band with 2Tx is considered.

In the existing specification, not PA configuration but the power of each uplink component is illustrated. With 3dB power increasement for transmitter of PC2 than that of PC3, and (24.8dBm-20dBm) improvement for each Tx antenna connector of PC1.5 than that of PC3. The delta_MSD between different power classes is listed in Table 3 below.

Table 3 Delta_MSD for HPUE and the coefficient for IMD order

| IMD order | Sum of the coefficient for the two uplink components | Proposed Delta_MSD btw PC2 and PC3 | Proposed Delta_MSD btw PC1.5 and PC2 |
| --- | --- | --- | --- |
| IMD2 | 2 | 6dB, i.e., 2*(23dBm-20dBm) | 1*(24.8dBm -23dBm) |
| IMD3 | 3 | 9dB, i.e., 3*(23dBm-20dBm) | 1*(24.8dBm -23dBm), or 2*(24.8dBm -23dBm) |
| IMD4 | 4 | 12dB, i.e., 4*(23dBm-20dBm) | Min:1*(24.8dBm -23dBm), or Max:3*(24.8dBm -23dBm) |
| IMD5 | 5 | 15dB, i.e., 5*(23dBm-20dBm) | Min:1*(24.8dBm -23dBm), or Max:4*(24.8dBm -23dBm) |

For delta_MSD of PC2 and PC3, (6, 9, 12, 15) corresponded to IMD2/3/4/5 should be considered.

Margin could also be considered based on the Delta_MSD btw PC1.5/PC2 and the default power class for testing. 1.5dB can be considered.

Proposal 6: For delta_MSD of PC2, [(6, 9, 12, 15) +margin] dBm corresponded to IMD2/3/4/5 should be considered.

- Margin = 1.5 can be considered.

The delta_MSD between PC1.5 and PC2 does not exhibit a clear or consistent pattern based on the existing requirements. Unlike PC2, the MSD for PC1.5 may vary depending on whether 3Tx or 2Tx is used, primarily due to differences in PA configurations for the constituent bands. Additionally, particularly for high-order intermodulation distortion (IMD), such as IMD4, which can result from components like 2f2-2f1 or 3f2-f1, the positions of the IMD products and the PA output power are key factors that contribute to the significant differences in delta_MSD between PC1.5 and PC2.

Proposal 7: For delta_MSD between PC1.5 and PC3, on top of the value between PC2 and PC3, additional value with worst case could be considered, i.e. (N-1)*1.8, where N is the order of IMD.

Moreover, for higher order IMD, the MSD value with PC3 is 0, such as CA_n8-n77 with IMD4, and CA_n7-n20 with IMD3, and the delta_MSD of PC2/PC1.5 may not be fully applied. Case by case analysis may still be required for higher order IMD.

Proposal 8: For higher order IMD, the MSD value with PC3 is 0, and the delta_MSD of PC2/PC1.5 may not be fully applied. Case by case analysis may still be required for higher order IMD.

Conclusion

In this contribution we discussed on HPUE MSD rules, according to the analysis, we have the following observations and proposals:

Observation 1: Across different interference paths, the power imbalance between the main and diversity branches tends to be lower than the typical antenna isolation value of 10 dB.

Observation 2: For 1Tx PC2 harmonic case, the increased output power is reflected in the same level of increased interferences to both the main and diversity paths, thus 3dB MSD degradation is expected.

Observation 3: It is obvious from our calculation that the HPUE harmonic MSD values are quite different for 1Tx and 2Tx configurations.

Observation 4: For 2Tx PC2 harmonic case, when the composite H2 interference at the LNA port is equal in both the main and diversity paths (i.e., no interference imbalance between the paths), the calculated MSD is approximately 7 dB higher compared to PC3 1Tx for the example band combination in our analysis.

Observation 5: It is observed that for most harmonic cases, the PC3 MSD values are larger than 7dB.

Observation 6: Harmonic mixing MSD values for UL1/DLx are relatively high, and most of them are above 5dB for PC3.

Observation 7: For 1Tx PC2 harmonic mixing case, ~3dB increasement of MSD is observed for most UL1/DLx cases.

Observation 8: For 2Tx PC2 harmonic mixing case, 7~8dB increasement of MSD is observed for UL1/DLx cases, which is aligned with harmonic case.

Observation 9: For 2Tx PC1.5 harmonic mixing case, 5~6dB increasement of MSD is observed for UL1/DLx cases, which is even lower than the case of 2Tx PC2. Similar rule for harmonic cases may not be accurate for 2Tx PC1.5.

Observation 10: For ULx/DLy harmonic mixing, most HPUE MSD cases are for 1Tx PC2 and 2Tx PC1.5. No obvious rule could be identified, which are not aligned with the analysis of harmonic MSD and harmonic mixing for UL1/DLx.

Observation 11: PC21Tx = 3dB can be considered for cross-band interference source of ACLR1/ACLR2.

Observation 12: Cross-band isolation covers most cases of small value MSDs.

Observation 13: PC21Tx = 3dB could be considered for cross-band interference source of > ACLR2 for all available cases, but not that accurate.

Observation 14: For cross-band isolation HPUE MSD with 2Tx PC2 and PC1.5, no obvious rules could be observed.

Observation 15: Define separate LUTs could be more accurate for band combinations with cross band isolation source of ACLR1/ACLR2, and with Rx frequency range of one band close to and far away from ACLR2 region.

Observation 16: For 3Tx IMD analysis, there is barely a little contribution from the 2nd PA to the PC1.5 MSD value. Therefore, only 24.8dBm output power from the band with 2Tx is considered.

1UL – Harmonic MSD

Proposal 1: Consider to define two values for PC2 harmonic requirements with PC21Tx and PC22Tx.

Proposal 2: The values for harmonic MSD could be PC21Tx = 3dB, PC22Tx= 7~8dB respectively.

Proposal 3: The harmonic MSD for PC1.5 2Tx should be future proof, which could be 3dB higher than PC2 2Tx.

1UL – Harmonic mixing MSD

Proposal 4: It is proposed to investigate whether the harmonic mixing rules applicable to UL1/DLx configurations can be extended to ULx/DLy scenarios.

1UL – Crossband isolation MSD

Proposal 5: For cross band isolation source of ACLR1/ACLR2: PC21Tx = 3dB, PC22Tx = 6dB, PC1.5 = 9dB.

[for cross-band interference source > ACLR2:

- PC2 = 3dB, PC1.5 = 6dB for band combinations with Rx frequency range of one band close to ACLR2 region.

- PC2 = 1dB, PC1.5 = 2.5dB for band combinations with Rx frequency range of one band far away from ACLR2 region.]

2UL – IMD MSD

Proposal 6: For delta_MSD of PC2, [(6, 9, 12, 15) + margin] dBm corresponded to IMD2/3/4/5 should be considered.

- Margin = 1.5 can be considered.

Proposal 7: For delta_MSD between PC1.5 and PC3, on top of the value between PC2 and PC3, additional value with worst case could be considered, i.e. (N-1)*1.8, where N is the order of IMD.

Proposal 8: For higher order IMD, the MSD value with PC3 is 0, and the delta_MSD of PC2/PC1.5 may not be fully applied. Case by case analysis may still be required for higher order IMD.

- References

[1] R4-2420380, WF on MSD rule, Skyworks, Murata, Nokia, Qualcomm, RAN4#113, November, 2024

[2] R4-2417784, Further discussion on HPUE MSD rules, Murata, RAN4#113, November, 2024

[3] R4-2417794, Simplification of HPUE 2UL IMD MSDs, Skyworks Solutions, Inc., RAN4#113, November, 2024

[4] R4-2417999, HPUE MSD requirements, Nokia, RAN4#113, November, 2024

[5] R4-2419082, MSD for HPUE, Qualcomm France, RAN4#113, November, 2024

[6] R4-2307146, On UE MSD requirements for FWA UE supporting 3T for two bands, Huawei, RAN4#107, May, 2023

[7] RP-232937 SR of 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC, OPPO, RAN#102, December, 2023

[8] R4-2407156, Guidelines for FDD PC2 Harmonic MSD Analysis, Skyworks Solutions, Inc, RAN4#111, May, 2024

- Annex

A.1 IMD delta_MSD comparisons

Reference Table 1: Delta_MSD comparison for IMD2

|  |  | Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | PC3 MSD | PC2 MSD (dB) | PC2 DeltaMSD to PC3 | PC1.5 MSD (dB) | PC1.5 DeltaMSD to PC3 | Duplex mode |  |
| CA_n1-n77 | n1 | 1950 | 5 | 25 | 2140 | 29.8 | 35.8 | 6 |  |  | FDD | IMD2 |
|  | n77 | 4090 | 10 | 50 | 4090 |  | N/A |  |  |  | TDD | N/A |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 26 | 32.10 | 6.1 | 35.2 | 9.2 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 |  | N/A |  |  |  | TDD | N/A |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 26 | 32.1 | 6.1 | 37.5 | 11.5 | FDD | IMD24 |
|  | n77 | 3790 | 10 | 50 | 3790 |  | N/A |  |  |  | TDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 31 | 34.33 | 3.33 | 40 | 9 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 |  | N/A |  |  |  | TDD | N/A |
| CA_n70-n77 | n70 | 1702.5 | 5 | 25 | 2002.5 | 31 | 37 | 6 | 41.8 | 10.8 | FDD | IMD2 |
|  | n77 | 3705 | 10 | 50 | 3705 |  | N/A |  |  |  | TDD | N/A |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 | 26 | 31.9 | 5.9 |  |  | FDD | IMD2 |
|  | n77 | 3575 | 10 | 50 | 3575 |  | N/A |  |  |  | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | 31.9 | 5.9 |  |  | FDD | IMD2 |
|  | n78 | 3575 | 10 | 50 | 3575 |  | N/A |  |  |  | TDD | N/A |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 | 26 | 32.10 | 6.1 |  |  | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 |  | N/A |  |  |  | TDD | N/A |

Reference Table 2: Delta_MSD comparison for IMD3

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | PC3 MSD | PC2 MSD (dB) | PC2 DeltaMSD to PC3 | Duplex mode |  |
| CA_n18-n41 | n18 | 820 | 5 | 25 | 865 | 15.6 | 24.6 | 9 | FDD | IMD3 |
|  | n41 | 2505 | 5 | 25 | 2505 |  | N/A |  | TDD | N/A |

Reference Table 3: Delta_MSD comparison for IMD4

|  |  |  | Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) |  | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | PC3 MSD | PC2 MSD (dB) | PC2 DeltaMSD to PC3 | PC1.5 MSD (dB) | PC1.5 DeltaMSD to PC3 | Duplex mode |  |
| CA_n1-n774 | n1 | 1950 |  | 5 | 25 | 2140 | 8.0 | 17.8 | 9.8 |  |  | FDD | IMD4 |
|  | n77 | 3710 |  | 10 | 50 | 3710 |  | N/A |  |  |  | TDD | N/A |
| CA_n1-n78 | n1 | 1950 |  | 5 | 25 | 2140 | 8 | 17.8 | 9.8 |  |  | FDD | IMD4 |
|  | n78 | 3710 |  | 10 | 50 | 3710 |  | N/A |  |  |  | TDD | N/A |
| CA_n2-n77 | n2 | 1900 |  | 5 | 25 | 1980 | 5 | 19.10 | 14.1 | 26.4 | 21.4 | FDD | IMD4 |
|  | n77 | 3720 |  | 10 | 50 | 3720 |  | N/A |  |  |  | TDD | N/A |
| CA_n5-n77 | n5 | 844 |  | 5 | 25 | 889 | 8.3 | 18.6 | 10.3 | 25.5 | 17.2 | FDD | IMD44,13 |
|  | n77 | 3421 |  | 10 | 50 | 3421 |  | N/A |  |  |  | TDD | N/A |
| CA_n7-n77 | n7 | 2540 |  | 5 | 25 | 2660 | 7.1 | [15.8] | 8.7 | 29.7 | 22.6 | FDD | IMD4 |
|  | n77 | 3870 |  | 10 | 50 | 3870 |  | N/A |  |  |  | TDD | N/A |
| CA_n25-n77 | n25 | 1900 |  | 5 | 25 | 1980 | 8.0 | 19.1 | 11.1 | 25.8 | 17.8 | FDD | IMD4 |
|  | n77 | 3720 |  | 10 | 50 | 3720 |  | N/A |  |  |  | TDD | N/A |
| CA_n41-n71 | n41 | 2614 |  | 5 | 25 | 2614 |  | N/A |  |  |  | TDD | N/A |
|  | n71 | 665 |  | 5 | 25 | 619 | 11 | 16.3 | 5.3 | 25.4 | 14.4 | FDD | IMD4 |
| CA_n3-n41 | n3 | 1740 |  | 5 | 25 | 1835 | 8.2 | 18.4 | 10.2 |  |  | FDD | IMD4 |
|  | n41 | 2657.5 |  | 10 | 50 | 2657.5 |  | N/A |  |  |  | TDD | N/A |
| CA_n3-n77 | n3 | 1765 |  | 5 | 25 | 1860 | 8 | 18.5 | 10.5 |  |  | FDD | IMD4 |
|  | n77 | 3435 |  | 10 | 50 | 3435 |  | N/A |  |  |  | TDD | N/A |
| CA_n3-n78 | n3 | 1765 |  | 5 | 25 | 1860 | 8 | 18.5 | 10.5 |  |  | FDD | IMD4 |
|  | n78 | 3435 |  | 10 | 50 | 3435 |  | N/A |  |  |  | TDD | N/A |
| CA_n5-n78 | n5 | 844 |  | 5 | 25 | 889 | 8.3 | 18.6 | 10.3 |  |  | FDD | IMD4 |
|  | n78 | 3421 |  | 10 | 50 | 3421 |  | N/A |  |  |  | TDD | N/A |
| CA_n8-n77 | n8 | 897.5 |  | 5 | 25 | 942.5 | void | 15.5 |  |  |  | FDD | IMD4 |
|  | n77 | 3635 |  | 10 | 50 | 3635 |  | N/A |  |  |  | TDD | N/A |
| CA_n8-n78 | n8 | 897.5 |  | 5 | 25 | 942.5 | 8.3 | 15.5 | 7.2 |  |  | FDD | IMD4 |
|  | n78 | 3635 |  | 10 | 50 | 3635 |  | N/A |  |  |  | TDD | N/A |
| CA_n18-n77 | n18 | 827.5 |  | 5 | 25 | 872.5 | 8.4 | 17.5 | 9.1 |  |  | FDD | IMD416 |
|  | n77 | 3355 |  | 10 | 50 | 3355 |  | N/A |  |  |  | TDD | N/A |
| CA_n26-n78 | n26 | 836.5 |  | 5 | 25 | 881.5 | 11.1 | 23.8 | 12.7 |  |  | FDD | IMD4 |
|  | n78 | 3391 |  | 10 | 50 | 3391 |  | N/A |  |  |  | TDD | N/A |
| CA_n30-n77 | n30 | 2310 |  | 5 | 25 | 2355 | 8.0 | 17.6 | 9.6 |  |  | FDD | IMD4 |
|  | n77 | 3487.5 |  | 10 | 50 | 3487.5 |  | N/A |  |  |  | TDD | N/A |

Reference Table 4: Delta_MSD comparison for IMD5

|  |  | Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | PC3 MSD | PC2 MSD (dB) | DeltaMSD | PC1.5 MSD (dB) | PC1.5 DeltaMSD to PC3 | Duplex mode |  |
| CA_n2-n774 | n2 | 1885 | 5 | 25 | 1965 | 8 | 20.0 | 12 | 28 | 20 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 |  | N/A |  |  |  | TDD | N/A |
| CA_n5-n772 | n5 | 826.5 | 5 | 25 | 871.5 | 5.5 | Void | Void | 24.3 | 18.8 | FDD | IMD5 |
|  | n77 | 4177.5 | 10 | 50 | 4177.5 |  |  |  | N/A |  | TDD | N/A |
| CA_n25-n77 | n25 | 1885 | 5 | 25 | 1965 | 5 | Void | Void | 29.8 | 24.8 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 |  |  |  |  |  | TDD | N/A |
| CA_n66-n77 | n66 | 1760 | 5 | 25 | 2160 | 5.0 | 11.27 | 6.27 | 26 | 21 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 |  | N/A |  |  |  | TDD | N/A |
| CA_n70-n77 | n70 | 1697.5 | 5 | 25 | 1997.5 | 5.0 | 18.4 | 13.4 | 27.9 | 22.9 | FDD | IMD5 |
|  | n77 | 3545 | 10 | 50 | 3545 |  | N/A |  |  |  | TDD | N/A |
| CA_n71-n77 | n71 | 681.5 | 5 | 25 | 635.5 | 5.5 | 11.4 | 5.9 | 16 | 10.5 | FDD | IMD513 |
|  | n77 | 3361.5 | 10 | 50 | 3361.5 |  | N/A |  |  |  | TDD | N/A |

A.2 Harmonic delta_MSD comparisons (only the 1st test points are illustrated)

Reference Table 5: Delta_MSD comparison for Harmonic

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | PC2 MSD_1Tx | DeltaMSD | PC2 MSD_2Tx | DeltaMSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) | (dB) | (dB) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 12 | 10 | 23.9 | 27.1 | 3.2 | 32.3 | 8.4 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 12 | 10 | 23.9 | 26.9 | 3 | 31.9 | 8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 12 | 10 | 23.9 | 26.9 | 3 | 31.9 | 8 | NOTE 2 | UL2/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 8 | 10 | 13 | 15.9 | 2.9 | 20 | 7 | NOTE 2 | UL3/DL1 direct-hit |
| n71 | n25 | 5 | 15 | 8 | 5 | 6.9 | 9.4 | 2.5 | 12.9 | 6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 6 | 10 | 10.8 | 13.6 | 2.8 | 17.8 | 7 | NOTE 4 | UL4/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 5 | 10 | 12 | 14.9 | 2.9 | 19 | 7 | NOTE 2 | UL5/DL1 direct-hit |
| n71 | n77 | 5 | 15 | 5 | 10 | 10.4 | 13.2 | 2.8 | 17.4 | 7 | NOTE 5 | UL5/DL1 direct-hit |

A.3 Harmonic mixing delta_MSD comparisons (only the 1st test points are illustrated)

Reference Table 6: Delta_MSD comparison for Harmonic mixing UL1/DL2

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | MSD_1Tx | deltaToPC3 | MSD_PC1.5 | deltaToPC3 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |  |  |  |  |
| n77 | n2 | 10 | 15 | 25 | 5 | 6.7 | 9.2 | 2.5 | 11.8 | 5.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 5 | 5.7 | 8.1 | 2.4 | 10.7 | 5 | NOTE 4 | UL1/DL2 |
| n77 | n25 | 10 | 15 | 25 | 5 | 5.6 | 7.9 | 2.3 | 10.6 | 5 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 10 | 15 | 25 | 5 | 5.7 | 8.1 | 2.4 | 10.7 | 5 | NOTE 4 | UL1/DL2 |

Reference Table 7: Delta_MSD comparison for Harmonic mixing UL1/DL3

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | MSD_1Tx | deltaToPC3 | MSD_2Tx | deltaToPC3 | MSD_PC1.5 | deltaToPC3 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |  |  |  |  |  |  |
| n25 | n713 | 5 | 15 | 25 | 5 | 26.8 | 29.5 | 3 | 34.5 | 7.7 |  |  | NOTE 4 | UL1/DL3 |
| n40 | n28 | 5 | 15 | 6 | 5 | 37.8 | 40.8 | 3 |  |  | 43.8 | 6 | NOTE 7 | UL1/DL3 |
| n41 | n186 | 10 | 15 | 25 | 5 | 26.3 | 29.3 | 3 |  |  |  |  | NOTE 7 | UL1/DL3 |

Reference Table 8: Delta_MSD comparison for Harmonic mixing UL1/DL4

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | MSD_1Tx | deltaToPC3 | MSD_PC1.5 | deltaToPC3 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |  |  |  |  |
| n77 | n5 | 10 | 15 | 25 | 5 | 5.7 | 8.1 | 2.4 | 10.7 | 5 | NOTE 5 | UL1/DL4 |
| n77 | n8 | 10 | 15 | 25 | 5 | 5.7 | 8.1 | 2.4 |  |  | NOTE 5 | UL1/DL4 |
| n78 | n5 | 10 | 15 | 25 | 5 | 5.7 | [8.1] | 2.4 |  |  | NOTE 5 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 5 | 5.7 | 8.1 | 2.4 | 10.6 | 4.9 | NOTE 5 | UL1/DL4 |
| n78 | n26 | 10 | 15 | 25 | 5 | 5.4 |  |  | 10.1 | 4.7 |  | UL1/DL4 |

Reference Table 9: Delta_MSD comparison for Harmonic mixing UL1/DL5

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | MSD_1Tx | deltaToPC3 | MSD_PC1.5 | deltaToPC3 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |  |  |  |  |
| n77 | n12 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n77 | n85 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n78 | n28 | 10 | 15 | 25 | 5 | 31 | 34 | 3 | 37 | 6 | NOTE 1 | UL1/DL5 |
| n79 | n8 | 10 | 15 | 25 | 5 | 25 | 28.0 | 3 | 31 | 6 | NOTE 1 | UL1/DL5 |

Reference Table 10: Delta_MSD comparison for Harmonic mixing ULx/DLy

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | MSD_1Tx | deltaToPC3 | MSD_2Tx | deltaToPC3 | MSD_PC1.5 | deltaToPC3 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  | (dB) |  |  |  |  |  |  |  |
| n41 | n39 | 10 | 15 | 8 | 5 | 4.3 | 6.1 | 1.8 |  |  | 10.5 | 6.2 | NOTE 9 | UL3/DL4 |
| n25 | n41 | 5 | 15 | 25 | 10 | 1.3 | 2.2 | 0.9 | 3.2 | 1.9 |  |  | NOTE 11 | UL4/DL3 |
| n39 | n41 | 5 | 15 | 6 | 5 | 9.3 | 10.6 | 1.3 |  |  |  |  | NOTE 8 | UL4/DL3 |
| n41 | n77 | 10 | 15 | 6 | 10 | 8.9 | 11.6 | 2.7 |  |  | 14.5 | 5.6 | NOTE 8 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 10 | 9.3 | 12.1 | 2.8 |  |  | 14.9 | 5.6 | NOTE 8 | UL4/DL3 |
| n77 | n7 | 10 | 15 | 12 | 5 | 14.7 | 17.7 | 3 |  |  | 17.5 | 2.8 | NOTE 3 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 12 | 5 | 10.4 | 13.2 | 2.8 |  |  | 16.1 | 5.7 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 10 | 11.7 | 14.5 | 2.8 |  |  | 16.1 | 4.4 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 10 | 11.7 | 14.5 | 2.8 |  |  | 17.5 | 5.8 | NOTE 3 | UL2/DL3 |
| n78 | n40 | 10 | 15 | 12 | 10 | 14.7 | 13.2 | -1.5 |  |  | 16.1 | 1.4 | NOTE 3 | UL2/DL3 |

Reference Table 11: Delta_MSD comparison for Cross band isolation

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | PC3 MSD | PC2-1Tx MSD | Delta_PC2-PC3 | PC2- 2Tx MSD | Delta_PC2-2Tx-PC3 | PC1.5 MSD | Delta_PC1.5-PC3 | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) |  | (dB) |  |  |  |  |  |  |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 23 | 26 | 3 | 32.3 | 9.3 |  |  | ACLR1 |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 28.1 | 31.1 | 3 |  |  | 37.9 | 9.8 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 31.4 | 34.4 | 3 |  |  | 41.2 | 9.8 | ACLR2 |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2 | 10.9 | 2.7 | 15.9 | 7.7 |  |  | ACLR2 |
| n2 | n66 | 1900 | 20 | 15 | 50 (RBstart=56) | 2112.5 | 5 | 0 | 0.7 | 0.7 | 0.9 | 0.9 |  |  | >ACLR2 |
| n3 | n1 | 1760 | 50 | 15 | 50 (RBstart=220) | 2112.5 | 5 |  | 0.8 |  | 1.1 |  |  |  | >ACLR2 |
| n3 | n7 | 1760 | 50 | 15 | 50 (RBstart=220) | 2622.5 | 5 |  | 0.5 |  | 0.7 |  |  |  | >ACLR2 |
| n7 | n1 | 2525 | 50 | 15 | 45 (RBstart=0) | 2167.5 | 5 |  | 0.8 |  | 1.1 |  |  |  | >ACLR2 |
| n7 | n3 | 2525 | 50 | 15 | 45 (RBstart=0) | 1877.5 | 5 | 0.6 | 1.1 | 0.5 | 1.5 | 0.9 |  |  | >ACLR2 |
| n7 | n40 | 2525 | 50 | 15 | 45 (RBstart=0) | 2397.5 | 5 | 3.7 |  |  |  |  |  |  | >ACLR2 |
| n25 | n41 | 1760 | 40 | 15 | 40 (RBstart=176) | 2501 | 10 |  | 0.8 |  | 1 |  |  |  | >ACLR2 |
| n25 | n66 | 1895 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 |  | 0.7 |  | 0.9 |  |  |  | >ACLR2 |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 3.3 | 4.7 | 1.4 |  |  |  |  | >ACLR2 |
| n40 | n3 | 2350 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 |  | 0.6 |  |  |  | 1.2 |  | >ACLR2 |
| n41 | n1 | 2546 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | 18.1 | 20.8 | 2.7 |  |  | 23.5 | 5.4 | >ACLR2 |
| n41 | n3 | 2546 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 | 0.6 | 2.3 | 1.7 |  |  | 3.9 | 3.3 | >ACLR2 |
| n41 | n25 | 2546 | 100 | 30 | 270 (RBstart=0) | 1992.5 | 5 | 0.6 | 1.6 | 1 |  |  | 2.8 | 2.2 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 1.6 | 2.7 | 1.1 |  |  | 6.7 | 5.1 | >ACLR2 |
| n41 | n66 | 2546 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 10.5 | 13.1 | 2.6 |  |  | 7.7 | -2.8 | >ACLR2 |
| n41 | n77 | 2640 | 100 | 30 | 270 (RBstart=3) | 3305 | 10 | 8.3 | 10.5 | 2.2 |  |  | 13.3 | 5 | >ACLR2 |
| n41 | n79 | 2640 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 |  | 3.1 |  |  |  |  |  | >ACLR2 |
| n66 | n2 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 1.2 | 1.9 | 0.7 | 3.3 | 2.1 |  |  | >ACLR2 |
| n66 | n25 | 1757.5 | 45 | 15 | 240 (RBstart=2) | 1932.5 | 5 | 1.4 | 2.2 | 0.8 | 3.8 | 2.4 |  |  | >ACLR2 |
| n66 | n41 | 1760 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 0.4 | 0.8 | 0.4 | 1 | 0.6 |  |  | >ACLR2 |
| n66 | n70 | 1760 | 40 | 15 | 216 (RBstart=0) | 1997.5 | 5 |  | 1.9 |  | 3.3 |  |  |  | >ACLR2 |
| n70 | n66 | 1702.5 | 15 | 15 | 75 (RBstart=4) | 2112.5 | 5 |  | 0.4 |  | 0.5 |  |  |  | >ACLR2 |
| n77 | n2 | 3350 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 |  | 1.0 |  |  |  | 1.8 |  | >ACLR2 |
| n77 | n7 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n77 | n25 | 3350 | 100 | 30 | 270 (RBstart=0) | 1992.5 | 5 |  | 1.0 |  |  |  | 1.8 |  | >ACLR2 |
| n77 | n30 | 3350 | 100 | 30 | 270 (RBstart=0) | 2357.5 | 5 |  | 1.0 |  |  |  | 1.8 |  | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 10 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 4.5 | 6.5 | 2 |  |  | 2.2 | -2.3 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n77 | n66 | 3350 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 |  | 1.0 |  |  |  | 1.8 |  | >ACLR2 |
| n78 | n7 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2395 | 10 | 4.5 | 6.5 | 2 |  |  | 9.0 | 4.5 | >ACLR2 |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 4.5 | 1.2 | -3.3 |  |  | 2.2 | -2.3 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 2 | 5 | 3 |  |  | 8 | 6 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4450 | 100 | 2 | 5 | 3 |  |  | 8 | 6 | >ACLR2 |
| n79 | n41 | 4450 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 |  | 3.5 |  |  |  |  |  | >ACLR2 |
| n79 | n783 | 4450 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | 2.6 | 5.6 | 3 |  |  | 8.6 | 6 | >ACLR2 |
| n79 | n783 | 4450 | 100 | 30 | 270 (RBstart=0) | 3750 | 100 | 2.6 | 5.6 | 3 |  |  | 8.6 | 6 | >ACLR2 |