3GPP TSG-RAN WG4 Meeting #114-Bis	R4-2503175

Wuhan, CN, Apr. 2025

Source	:	Qualcomm Incorporated

Title	:	On the proposed FR2 PC8 (new power class) UE

Agenda item	:	5.3.1

Document for	:	Approval

# 1.	Introduction

Emerging FR2 markets may well have denser deployment and more use cases than current practice in North America. A previous proposal [1] suggested that these new scenarios will be better served by a new UE type that combines the RF front end of a PC7 (RedCap UE) with the full featured baseband capabilities of PC3 (handheld UE). There is justifiable concern that PC8 will become the default power class (making PC3 obsolete). In this contribution, technical feasibility for an improved spherical coverage is provided.

# 2. 	Discussion

It was previously proposed to create an FR2 UE (‘PC8’) by combining a full-featured baseband section with an FR2 Redcap UE front end (i.e. PC7, which has  diminished RF capability compared to PC3). In a previous contribution [2], we showed the incompatibility of the stated use cases [1] and the proposed low EIRP capability.

Further discussions showed that there are other scenarios where the low EIRP suffices [3]. While these scenarios may exist, their limited nature and special conditions are cause for concern with standardizing this type of low EIRP UE for non-RedCap eMBB devices for more general usage.

On the one hand we think the FR2 eco-system will benefit from a UE with a different (lower) performance and price point from legacy power classes, so we support the PC8 initiative. On the other hand, we also recognize that PC7 spherical coverage ‘gain drop’ leaves PC8 with an UL that may be useful only in extremely select situations.

For this reason, a compromise would be to improve the ‘gain drop’ to the 50th % direction for PC8. To largely minimize the impact of introduction of these devices in existing networks, PC8 should have the same EIRP spherical coverage requirement as PC3. The improved performance can be realized in multiple ways. The first way focuses mainly on the PA and improved antenna performance [4]. A second way is to use multiple legacy PC7 modules, which trades PA quality and sophistication for complexity in layout, but makes important gains in diversity (hand grip problem). Note that beam management simplicity is retained due to retention of 2x1 modules.

In the figure to the left, the multi-module impact is captured as the relative EIRP spherical coverage performance. The values are normalized to the peak EIRP expectation from a legacy 4x1 module.

While some impairments are left out for this relative study, it is evident that the EIRP drop (dB) to the 50th% ile direction (EIRP50) for a UE with two 2x1 modules reduces by 45-50% compared to the single module drop. For n258, the EIRP50 value is 10.9 dB, which suggests that a UE with two 2x1 modules would have an EIRP50 drop of 6 dB. Note that this improvement comes from just adding the second module, with no increases in PA power. As noted previously [4], PAs have come a long way from Rel-15 assumptions, and these PAs make further improvement in spherical coverage feasible. (shifting the entire curve to the right)

Observation 1: Multiple avenues exist to reasonably improve spherical coverage beyond that of PC7, for example module count, antenna design and increase in PA power.

Proposal 1: To make the PC8 proposal more palatable to all stakeholders, RAN4 to consider adopting the following requirements for n257/n258/n261:

- min pk. EIRP: 16.4 dBm

- EIRP spherical coverage requirement: 11.5 dBm @ 50th% ile direction

# 3.	References

[1] 	R4-2408997, ‘Discussion on extension FR2 UE power class 7 to non-RedCap UE’,  Huawei, RAN4#111, May. 2024

[2] 	R4-2413227, ‘Discussion on feasibility of FR2 UEs with low EIRP’, Qualcomm, RAN4#112, Aug. 2024

[3] 	R4-2414420, ‘WF on introduction of new FR2 Power Class 8’, Huawei, RAN4#112, Aug. 2024

[4] 	R4-2500482, ‘On the proposed FR2 PC8 (new power class) UE’, Qualcomm, RAN4#114, Feb. 2025