3GPP TSG-RAN WG4 Meeting #114-bis	                               R4-2503215

Wuhan, China, April 7 – April 11, 2025

Source: 	Murata Manufacturing Co Ltd.

Title: 	Further discussion on HPUE MSD rules

Agenda item:	7.1.1.1.1

Document for:	Discussion

Introduction

In the RAN4#113 meeting, the general MSD simplification rules for HPUE was discussed and WF [1] was approved. In this WF, it was agreed that RAN4 specifies or provides equations/LUT/simplified guidelines identifying the HPUE MSD either as normative or informative requirements and the simplification rules provided by proponent companies were analyzed.

In the RAN4#114 meeting, there were many discussions, and it was agreed to adopt eligibility criteria to HPUE MSD requirements using LUTs, but WF was withdrawn due to diverse opinions from interested companies. Therefore, the detailed rules and methods need further discussion and consensus within RAN4.

In this contribution we listed the issues which needs to be compromised within the RAN4#114-bis meeting and make proposals so that the discussion moves forward.

Discussion

One UL HPUE MSD calculation (Harmonic, Harmonic mixing, Crossband isolation)

The basic starting point is that the 1Tx PC3 MSD is used as a reference when deriving HPUE MSD.

The first input variable for the delta MSD between HPUE and PC3 is interferer imbalance between PRX and DRX when calculating PC3 MSD. However, this number is quite diverse from different companies since they have used different RFFE architectures and RF parameters. So RAN4 is averaging the MSD based on companies’ input for final specification decision.

The second one is how much interferer for HPUE is increased compared to PC3 operation. For example, 3dB harmonic distortion increase from 1Tx PC3 to 1Tx PC2 operation. It should be noted that the interferers of 2Tx PC2 and 2Tx PC1.5 are referring 1Tx PC3 and 1Tx PC2 interferers respectively.

These two variables determine the delta MSD curves of 1Tx HPUE and 2Tx HPUE differently.

For 1Tx HPUE, the imbalance is not affecting the plateau of the curve but making slight differences at low PC3 MSD region. On the other hand, the interferer increase plays a major role for plateau level. Figure 1 shows the 1Tx PC2 delta MSD curve with 3dB interferer increase with 3/6/9dB interferer imbalance of PC3.

Figure 1. Delta MSD between 1Tx PC2 and PC3 (interferer noise increase is 3dB with 3/6/9dB interferer imbalance)

To our understanding, it is reasonable and safe to assume that the interferer increase of 1Tx PC2 is 3dB for harmonic, harmonic mixing and crossband isolation. So, we can use the curve in Figure 1 for LUT of delta MSD of 1Tx PC2, which means the plateau is 3dB. For small PC3 MSD region, the delta MSD is dependent on the interferer imbalance, but the difference is not much large, so LUT for this region could be compromised between companies.

Proposal 1: For 1Tx PC2 delta MSD, RAN4 to agree to use 3dB for the plateau. The delta MSD LUT for small PC3 MSD region should be compromised during the RAN4 114-bis meeting.

However, there are diverse opinion on the 2Tx PC2 and 2Tx PC1.5 delta MSD based on the contributions from the previous meeting as listed below.

- Assumed interferer imbalance level for PC3 MSD calculation, equivalently plateau level.

- Whether the same LUT could be used for high order harmonic mixing and crossband isolation (>ACLR2).

Figure 2 and Figure 3 show the 2Tx PC2 delta MSD and 2Tx PC1.5 delta MSD inputs from various companies in last meeting contribution. For reference, the existing MSD requirements for three types are also plotted with dotted mark.

Figure 2. 2Tx PC2 delta MSD inputs

Figure 3. 2Tx PC1.5 delta MSD inputs

Unlike the 1Tx PC2 case, the interferer imbalance of PC3 MSD determines the plateau level of 2Tx HPUE. This is because the PC3 MSD is calculated as MRC with diversity gain and both receivers of 2Tx PC2 (PC1.5) HPUE see the same interferer level which is combined with PRX and DRX interferer from 1Tx PC3 (PC2) UE. This is depicted in Figure 4.

Figure 4. 2Tx delta MSD plateau vs interferer imbalance of PC3 (including 0.5dB Rtx or reverse IMD effect)

Our proposal is to use 9dB interferer imbalance of PC3 which results in 8.4dB plateau for 2Tx PC2 and 11.4dB plateau for 2Tx PC1.5 (Rtx or reverse IMD 0.5dB included). In the previous meeting, some companies proposed the interferer imbalance for delta MSD calculation and resulting plateau ([2], [6], [8]), and other companies proposed the plateau only ([3], [5], [7]). However, the plateau of 2Tx PC2 delta MSD ranges from 6dB to 9dB from companies’ input as shown in Figure 2.

In our opinion, 8.4dB plateau (9dB interferer imbalance) for 2Tx PC2 is well aligned with the current existing 2Tx PC2 MSD requirements as shown in Figure 2. Therefore, we prefer to use 9dB imbalance or 8.4dB plateau but we are fine to average the plateau of 2Tx PC2 delta MSD as the conventional way of determining the MSD requirements.

After agreeing on the interferer imbalance or plateau first, RAN4 can discuss and determine the LUT for small PC3 MSD region.

Proposal 2: For 2Tx PC2 delta MSD, we propose to use 9dB interferer imbalance or 8.4dB plateau. However, it is fine for us to average the plateau level based on companies’ input. After that, RAN4 can discuss and determine the LUT for small PC3 MSD region.

Regarding 2Tx PC1.5 delta MSD, there are many points to be addressed.

The first one is that the current 2Tx PC1.5 has no harmonic MSD. This is because the aggressor UL band is FDD LB-MB currently so there is no 2Tx PC1.5 of FDD. However, it is expected that 2Tx FDD PC1.5 might be introduced for future release. Therefore, it is proposed that the interferer imbalance or plateau agreed in Proposal 2 should be reused for 2Tx PC1.5 delta MSD of harmonic distortion.

Proposal 3: For 2Tx PC1.5 delta MSD of harmonic distortion, RAN4 to reuse the agreed interferer imbalance or plateau in Proposal 2 for the calculation. After that, RAN4 can discuss and determine the LUT for small PC3 MSD region.

Secondly, some companies have concern on the unified delta MSD LUT for all one UL types of MSD, especially for harmonic mixing. This is because one of 2Tx PC2 delta MSD is 7.7dB while most of 2Tx PC1.5 delta MSD plateau is 6dB in the current existing spec. However, it should be noted that the harmonic mixing MSD of 2Tx PC2 cases are when the aggressor band is n25 FDD band (CA_n25-n71 and CA_n25-n41) with interferer imbalance. On the other hand, the harmonic mixing MSD of 2Tx PC1.5 cases are when the aggressor band is TDD band. To the best of our knowledge, the current PC1.5 harmonic mixing MSDs have been derived based on the assumption that the interferer imbalance for PC3 MSD is 0dB. That means the plateau of 2Tx PC1.5 delta MSD would be 6dB as shown in Figure 4 (excluding 0.5dB Rtx or reverse IMD effect).

Moreover, there have not been 2Tx PC1.5 harmonic mixing MSD cases from FDD aggressor up to now but it is expected to be requested by operators in future release, too.

Therefore, two options can be considered for 2Tx PC1.5 delta MSD of harmonic mixing, which uses the agreed plateau or imbalance in Proposal 2 as the calculation of 2Tx PC1.5 delta MSD for

- all harmonic mixing cases

or

- harmonic mixing cases when the aggressor band is FDD only and applies 6dB plateau when the aggressor band is TDD.

Proposal 4: For 2Tx PC1.5 delta MSD of harmonic mixing, RAN4 to reuse the agreed interferer imbalance or plateau in Proposal 2 for the calculation. However, it needs further discussion whether all harmonic mixing cases are applied or only FDD aggressor cases are applied and 6dB plateau is used for TDD aggressor cases. After that, RAN4 can discuss and determine the LUT for small PC3 MSD region.

Lastly, 2Tx PC1.5 delta MSD for crossband isolation shows very diverse tendency in Figure 3. It is believed that some cases have not considered the interferer imbalance impact on the 2Tx PC1.5 MSD calculation. Also, it is reasonable to assume that most of 2Tx PC1.5 delta MSD are regarded to have interferer imbalance for PC3 MSD. So, we propose to use the same LUT as harmonic distortion and harmonic mixing.

Proposal 5: For 2Tx PC1.5 delta MSD of crossband isolation, RAN4 to reuse the same LUT as 2Tx PC1.5 harmonic distortion in Proposal 3.

Table 1 indicates LUT for one UL HPUE delta MSD based on our initial proposal of 9dB interferer imbalance as an example.

Proposal 6: RAN4 to consider Table 1 for one UL HPUE delta MSD LUT as an example.

Table 1 One UL ΔMSD LUT

| PC3 MSD (dB) | ΔPC2 1Tx (dB) | ΔPC2 2Tx (dB) | ΔPC1.5 2Tx (dB) |
| --- | --- | --- | --- |
| <1 | 0.8 | 1.5 | 3.5 |
| 1-2 | 1 | 2.5 | 4.5 |
| 2-3 | 1.3 | 3.5 | 6 |
| 3-5 | 1.7 | 5 | 8 |
| 5-7 | 2.1 | 6 | 9 |
| 7-9 | 2.4 | 7 | 10 |
| 9-13 | 2.8 | 8 | 11 |
| >13 | 3 | 8.5 | 11.5 |

Two UL HPUE MSD calculation (IMD)

In the previous meetings, there have not been discussions about two UL HPUE MSD calculation (IMD) due to lack of time. Moreover, unlike one UL HPUE MSD calculation, two UL HPUE MSD calculation due to IMD from two aggressors is not simple and needs more analysis. This is because there are many power class combinations for one single band, different number of Tx, different IMD order and coefficient and different linearity between PC3 PA and PC2 PA.

The two UL HPUE delta MSD also has two input variables – interferer imbalance for PC3 MSD and interferer increase. Figure 5 shows the delta MSD graph with various interferer increase (6/9/12/15dB) and interferer imbalance of PC3 (3/6/9 dB). Like one UL 1Tx PC2, interferer increase determines the plateau and interferer imbalance makes slight difference at low PC3 MSD region. Therefore, in our opinion, it is important to determine the interferer increase for various two UL IMD cases as a first step and the interferer imbalance which was agreed in Proposal 2 can be reused for LUT for small PC3 MSD region.

Figure 5. Delta MSD between HPUE due to 2UL IMD and PC3 (interferer noise increase is 6/9/12/15dB with 3/6/9dB interferer imbalance)

Proposal 7 For delta MSD due to two UL IMD, RAN4 to agree the interferer increase for each PC and PA combinations as a first step. Then RAN4 to reuse the agreed interferer imbalance in Proposal 2 to determine the LUT for small PC3 MSD region.

Table 2 summarizes five categories of power class and number of Tx – PC3 2Tx (baseline), PC2 2Tx, PC2 3Tx, PC1.5 2Tx and PC1.5 3Tx. The third and the fourth column means power combination to achieve 2 UL CA power class and PA power capability for each band respectively. The fifth column denotes the case number in WF [1]. It should be noted that some cases (5, 6, 7, 8, 10) were not specified until Rel-18 and we do not yet have specified MSD for those cases.

Table 2 Two UL MSD categories of PC3, PC2 and PC1.5 with power class combinations

|  | Combination | Band A | Band B |  | Case # | Rel |
| --- | --- | --- | --- | --- | --- | --- |
| PC3 2Tx | Power | 20 | 20 |  |  |  |
|  | PA | PC3 PA | PC3 PA |  | 1 | R15 |
| PC2 2Tx | Power | 23 | 23 |  |  |  |
|  | PA | PC3 PA | PC3 PA |  | 2 | R17 |
|  |  | PC3 PA | PC2 PA |  | 3 | R17 |
|  |  | PC2 PA | PC2 PA |  | 5 | R19 |
| PC2 3Tx | Power | 23 | 23 |  |  |  |
|  |  |  | 20 | 20 |  |  |
|  | PA | PC3 PA | PC3 PA | PC3 PA | 4 | R18 |
|  |  | PC2 PA | PC3 PA | PC3 PA | 6 | R19 |
| PC1.5 2Tx | Power | 26 | 26 |  |  |  |
|  | PA | PC2 PA | PC2 PA |  | 7 | R19 |
| PC1.5 3Tx | Power | 23 | 27.8 |  |  |  |
|  |  |  | 24.8 | 24.8 |  |  |
|  | PA | PC3 PA | PC2 PA | PC2 PA | 9 | R18 |
|  | Power | 26 | 26 |  |  |  |
|  |  |  | 23 | 23 |  |  |
|  | PA | PC2 PA | PC3 PA | PC3 PA | 8 | R19 |
|  |  | PC2 PA | PC2 PA | PC2 PA | 10 | R19 |

We investigated two UL HPUE MSD rules for each category based on PC3 MSD (case #1) with different IMD orders in [2]. In this document, we just summarize and propose the interferer increase for each two UL IMD categories based on the analysis result of [2] as in Table 3.

Table 3 Interferer increase for HPUE PC and PA combinations with IMD order

| HPUE PC | PA combinations | IMD order | Interferer increase (dB) |  |
| --- | --- | --- | --- | --- |
| PC2 2Tx/3Tx | 23dBm + 23dBm | n | 3*n |  |
|  | 23dBm + 2*20dBm |  |  |  |
| PC1.5 2Tx/3Tx | 26dBm + 26dBm | n | TBD ~ 6*n |  |
|  | 26dBm + 2*23dBm |  |  |  |
|  | 23dBm + 2*24.8dBm | 2 (1,1) | 11 | or 11 |
|  |  | 3 (1,2) | 19 | or 19 |
|  |  | 3 (2,1) | 14 |  |
|  |  | 4 (1,3) | 27 | or 27 |
|  |  | 4 (2,2) | 22 |  |
|  |  | 4 (3,1) | 17 |  |
|  |  | 5 (1,4) | 35 | or 35 |
|  |  | 5 (2,3) | 30 |  |
|  |  | 5 (3,2) | 25 |  |
|  |  | 5 (4,1) | 20 |  |

Proposal 8: For delta MSD due to two UL IMD, RAN4 to consider the interferer increase for each HPUE PC and PA combinations in Table 3 as a starting point.

Conclusion

In this contribution, we analyzed and provided our view on the HPUE MSD derivation rules based on the existing PC3 MSD. With this analysis result, we make the following proposals.

- One UL HPUE MSD calculation

Proposal 1: For 1Tx PC2 delta MSD, RAN4 to agree to use 3dB for the plateau. The delta MSD LUT for small PC3 MSD region should be compromised during the RAN4 114-bis meeting.

Proposal 2: For 2Tx PC2 delta MSD, we propose to use 9dB interferer imbalance or 8.4dB plateau. However, it is fine for us to average the plateau level based on companies’ input. After that, RAN4 can discuss and determine the LUT for small PC3 MSD region.

Proposal 3: For 2Tx PC1.5 delta MSD of harmonic distortion, RAN4 to reuse the agreed interferer imbalance or plateau in Proposal 2 for the calculation. After that, RAN4 can discuss and determine the LUT for small PC3 MSD region.

Proposal 4: For 2Tx PC1.5 delta MSD of harmonic mixing, RAN4 to reuse the agreed interferer imbalance or plateau in Proposal 2 for the calculation. However, it needs further discussion whether all harmonic mixing cases are applied or only FDD aggressor cases are applied and 6dB plateau is used for TDD aggressor cases. After that, RAN4 can discuss and determine the LUT for small PC3 MSD region.

Proposal 5: For 2Tx PC1.5 delta MSD of crossband isolation, RAN4 to reuse the same LUT as 2Tx PC1.5 harmonic distortion in Proposal 3.

Proposal 6: RAN4 to consider Table 1 for one UL HPUE delta MSD LUT as an example.

- Two UL HPUE MSD calculation

Proposal 7 For delta MSD due to two UL IMD, RAN4 to agree the interferer increase for each PC and PA combinations as a first step. Then RAN4 to reuse the agreed interferer imbalance in Proposal 2 to determine the LUT for small PC3 MSD region.

Proposal 8: For delta MSD due to two UL IMD, RAN4 to consider the interferer increase for each HPUE PC and PA combinations in Table 3 as a starting point.

References

[1]	R4-2420380, “WF on MSD rule”, Skyworks, Murata, Nokia, Qualcomm, 3GPP TSG RAN WG4 Meeting #113, Orlando, US, November 18th – 22nd, 2024

[2] R4-2500117, “Further discussion on HPUE MSD rules”, Murata Manufacturing Co. Ltd, 3GPP TSG RAN WG4 Meeting #114, Athens, Greece, November 17th – 21st, 2025

[3] R4-2500140, “Discussion on HPUE MSD”, Huawei, 3GPP TSG RAN WG4 Meeting #114, Athens, Greece, November 17th – 21st, 2025

[4] R4-2500315, “On Rel-19 HPUE CA DC MSD consideration”, Apple, 3GPP TSG RAN WG4 Meeting #114, Athens, Greece, November 17th – 21st, 2025

[5] R4-2500731, “Discussion on HPUE MSD simplification”, vivo, 3GPP TSG RAN WG4 Meeting #114, Athens, Greece, November 17th – 21st, 2025

[6] R4-2501579, “MSD beyond PC3”, Qualcomm France, 3GPP TSG RAN WG4 Meeting #114, Athens, Greece, November 17th – 21st, 2025

[7] R4-2501685, “Duty cycle solutions and MSD rules for HPUE inter-band UL CA and EN-DC”, LG Electronics, 3GPP TSG RAN WG4 Meeting #114, Athens, Greece, November 17th – 21st, 2025

[8] R4-2501955, “1UL MSD simplifications rules for Rel-19”, Skyworks Solutions, 3GPP TSG RAN WG4 Meeting #114, Athens, Greece, November 17th – 21st, 2025