3GPP TSG-RAN WG4 Meeting #114bis	R4-2503228

Wuhan, China, 7th– 11th, Apr, 2025

Title:	LS to RAN2 on Type-4 for non-collocated deployment

Response to:

Release:	Rel-19

Work Item:	NonCol_intraB_ENDC_NR_CA_Ph2-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name:  Petri Vasenkari

E-mail Address:  petri.j.vasenkari@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN4 wants to share the current situation of the NonCol_intraB_ENDC_NR_CA_Ph2 WI.

On network indication:

- When maxMIMO-layers ＞4, Type 1(collocated). No new BS signaling is needed.

- maxMIMO-layers=4, New R19 network indication is needed to indicate if UE follows collocated (Type 1) or non-collocated (Type 4) deployment. RAN4 asks RAN2 to define this network indication and UE behavior when this new R19 network indication is absent.

- maxMIMO-layers=2, follow Rel-18 specification

UE capability for R19 Type 4 support is also needed, if UE indicates support for R19 Type 4 it also must support R18 Type 2.

RAN4 respectfully ask RAN2 to define the necessary UE capabilities and NW signalling.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully ask RAN2 to devise the necessary UE capabilities and NW signaling.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #115	18 - 24 May, 2025		                   Malta

TSG-RAN WG4 Meeting #116	25 – 29 Aug, 2025		                  Indian