3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503232

Wuhan, China 7th – 11th April 2025

Source: 	Nokia

Title: 					n26 7 MHz A-MPR simulations and proposal

Agenda Item: 			7.8.2

Document for:	Approval

# 1	Introduction

We simulate the A-MPR for 7 MHz CBW at fixed position of Fc = 820.5 MHz on operating band n26. We consider the additional spurious emission masks signaled via NS_12 and NS_15, and we make a proposal for the A-MPR.

# 2	Simulation assumptions and scenario

## 2.1	Simulation parameters

- Operating band n26 (UL 814 MHz – 849 MHz)

- Fixed center frequency Fc = 820.5 MHz according to planned deployment

- 7 MHz CBW with NRB = 35 as defined in [1]

- Power class PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc

- Only additional spurious emission masks from NS_12 and NS_15 are simulated. Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

## 2.2	NS_12 mask

Table 6.5.3.3.17-1: Additional requirements "NS_12"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 10 MHz |  |
| 806 ≤ f ≤ 813.5 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 814 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

## 2.3	NS_15 mask

Table 6.5.3.3.20-1: Additional requirements "NS_15"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |
| 851 ≤ f ≤ 859 | -53 | 6.25 kHz |
| NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

# 3	Simulation results

## 3.1	NS_12

In Fig. 1 we show examples of the simulated A-MPR. We present the complete set of the A-MPR triangles at Fc = 820.5 MHz in the Appendix.

Figure 1: Examples of the simulated A-MPR to satisfy the NS_12 additional spurious emission limit for 7 MHz channel at Fc = 820.5 MHz.

## 3.2	NS_15

A-MPR is not needed for NS_15 with Fc = 820.5 MHz.

# 4	A-MPR proposal

We make the following A-MPR proposal based on our simulation results. We highlight the new entries in the tables with yellow color. In Fig. 2 we show examples of the proposed A-MPR triangles. The complete set of triangles is presented in the Appendix.

Proposal 1: RAN4 shall consider defining A-MPR for n26 PC3 NS_12 as shown in tables 6.2.3.21-1 and 6.2.3.21-2 below.

Table 6.2.3.21-1: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7MHz | Fc = 820.5 MHz | ≤ 0.72 | ≤ 0.9 | A3 |
|  |  | ≥ 0 | ≥ 1.5*12*SCS*RBstart + 4.8 | A4 |
| 10MHz |  | ≤3.6 | >0 | A1 |

Table 6.2.3.21-2: A-MPR for NS_12

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 | ≤ 2 | ≤ 1 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 | ≤ 2 | ≤ 1.5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 | ≤ 2.5 |  |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 |  |  |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 |  |  |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 | ≤ 3.5 |  |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 | ≤ 3.5 |  |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 |  |  |
| CP-OFDM 256 QAM | ≤ 9.5 |  |  |  |

Figure 2: Examples of the proposed A-MPR for n26 PC3 NS_12 at Fc = 820.5 MHz.

# 5	Conclusions

We have simulated the A-MPR for 7 MHz CBW on n26 at Fc = 820.5 MHz due to additional emission requirements signaled via NS_12 and NS_15. No A-MPR is needed for NS_15 according to our simulations. For NS_12 we have made the following proposal.

Proposal 1: RAN4 shall consider defining A-MPR for n26 PC3 NS_12 as shown in tables 6.2.3.21-1 and 6.2.3.21-2 below.

Table 6.2.3.21-1: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7MHz | Fc = 820.5 MHz | ≤ 0.72 | ≤ 0.9 | A3 |
|  |  | ≥ 0 | ≥ 1.5*12*SCS*RBstart + 4.8 | A4 |
| 10MHz |  | ≤3.6 | >0 | A1 |

Table 6.2.3.21-2: A-MPR for NS_12

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 | ≤ 2 | ≤ 1 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 | ≤ 2 | ≤ 1.5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 | ≤ 2.5 |  |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 |  |  |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 |  |  |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 | ≤ 3.5 |  |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 | ≤ 3.5 |  |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 |  |  |
| CP-OFDM 256 QAM | ≤ 9.5 |  |  |  |

# 6	References

[1] R4-2502870, WF on requirements for 7MHz channel bandwidth, Ericsson

# 7	Appendix: A-MPR triangles

## 7.1	NS_12 simulations

## 7.2	NS_12 proposal