3GPP TSG RAN WG4 Meeting #114-bis                                                	      R4-2503235

Wuhan, China, April 7 – 11, 2025

Title:	Fair coexistence between SBFD and TDD networks

Agenda item:	 7.23.1 - General RAN4 aspects for SBFD system parameters

Source:	Charter Communications

Document for:	 Discussion/Agreement

# Introduction

In Rel 18 study, SBFD performance was evaluated, and it was concluded in TR 38.858 [1] that SBFD provides benefits of reduced latency and increased UL throughput over legacy TDD. It was concluded in the study phase that SBFD may cause UE-to-UE interference to the UEs of a legacy TDD network operating in the adjacent channel. Rel. 19 normative phase started with the specific objective in the WID [2] for adjacent channel coexistence between the two operators.

In the last meeting in Athens, we presented [6] how a single operator owning both networks could mitigate this UE-to-UE CLI, by reducing the PCMAX of the SBFD UE by XdB. The RAN4 vice chair decision was to wait until after RAN1 approves this solution.

RAN1 decision: Our RAN1 team in Charter is presenting this week the TDOC [6] for agrrement in RAN1.

In this meeting, we present more ideas on the CLI mitigation options. I would like to thank Richard Tang of Verizon for educating Charter on this topic.

# Discussion

It was agreed that UE-to-UE CLI is worst at cell edge, when the two UEs are in a cluster location (same area), as shown in Rel. 18 study, TR 38.858 [1] section 11.3.1-1.

The next diagram shows that DUD (40%, 20%, 40%) is optimal to mitigate this issue, as its IM7 noise is reduced much more then IM3 and IM5. Additionally, its IM3 and IM5 do not reach the adjacent channel.

DUD is much better than DU, since DU protects only the left side carrier, but not the right-side carrier. Also, DU creates IM3, which is much more powerful noise than IM7.  Therefore, a single operator owning both networks could mitigate this UE-to-UE CLI, by using DUD (40%, 20%, 40%), and by reducing PCMAX of the SBFD UE by XdB.

## Limiting Maximum UE Transmit power in SBFD symbols/slots.

As mentioned above, the benefit of allowing UEs to transmit at the maximum power level during the SBFD symbols/slots may not be beneficial to the UEs and may increase UE-to-UE CLI in the adjacent networks. This UE-to-UE CLI is worse when the UEs of adjacent legacy TDD network are at their cell edge and SBFD-Aware UEs are close to it as shown in Rel. 18 study, TR 38.858 [1] section 11.3.1-1.

To limit this increase in UE-to-UE CLI in the adjacent legacy TDD networks, it will be more efficient to the limit maximum transmit power of UEs during SBFD symbols/slots, while keeping the maximum transmit power of UEs during the legacy UL symbols/slots same as in case of legacy TDD systems as shown in the following figure:

The figure above shows that SBFD aware UEs will transmit at a higher power level than the legacy TDD UEs. This higher power is caused by the interference factor, caused by gNB-to-gNB interference, where the aggressor is TDD, and the victim is SBFD. To overcome this interference at the SBFD gNB receiver, the UE will need to boost its power. This may cause UE-to-UE CLI, where now the aggressor is the SBFD UE and the victim is TDD UE. To mitigate this UE-to-UE CLI in the adjacent networks the maximum UE transmit power should be limited to a value lower than the maximum transmit power allowed in the legacy TDD networks. Based on the above figure we propose

Transmit power=minPCMAX, Ptx,  &non-SBFD symbols/slotsminPCMAX,SBFD, Ptx,  &SBFD symbols/slots

Where:

Ptx = transmit power as defined in 38.213 [3] for different Channels (PUSCH/PUCCH/RACH/SRS)

PCMAX = UE configured maximum output power as specified in [8-1, 38.101-1], [8-2, 38.101-2], and [8-3. 38.101-3]

PCMAX,SBFD = PCMAX – XdB where XdB could be a backoff as determined by gNB supporting SBFD.

Limiting the UE transmit power during SBFD symbols/slots will help in mitigating UE-to-UE CLI in the adjacent legacy TDD networks. It will also save the SBFD system in unsuccessful retransmissions of screaming UEs at cell edge which cannot benefit from SBFD anyway. The gNB will have full cell range for TDD symbols, and smaller range for SBFD symbols.

The SBFD operator may set XdB = 0 to completely turn off this proposal. Alternatively, the operator may set XdB to a value that will turn off only 5% or 10% of the SBFD UE population by using a dynamic algorithm which is based on Tx power headroom reports from the UEs, as we propose at the end of this TDOC.

Proposal 1: A single operator owning both networks, SBFD and legacy TDD, could mitigate UE-to-UE CLI, by using DUD (40%, 20%, 40%), and by reducing PCMAX of the SBFD UE power by XdB.

# Conclusions

Based on the discussion above we propose the following:

Proposal 1: A single operator owning both networks, SBFD and legacy TDD, could mitigate UE-to-UE CLI, by using DUD (40%, 20%, 40%), and by reducing PCMAX of the SBFD UE power by XdB.

# Reference

[1] 3GPP TR 38.858, “Study on Evolution of NR Duplex Operation”

[2] RP-242772 Revised WID - Evolution of NR duplex operation Sub-band full duplex (SBFD)

[3] 3GPP TS 38.213, “Physical Layer Procedures for Control”

[4] 3GPP TS 38.101-1, “User Equipment (UE) radio transmission and reception: Part 1: Range 1 Standalone”

[5] 3GPP TS 38.101-2, “User Equipment (UE) radio transmission and reception: Part 2: Range 2 Standalone”

[5] 3GPP TS 38.101-3, “User Equipment (UE) radio transmission and reception: Part 3: Range 1 and Range 2 Interworking operation with other radios”

[6] Charter TDOC R4-2500474