3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503240

Wuhan, China, April 7 – 11, 2025

# Title:	Cross-band isolation exception cases and corrections

# Agenda Item:	6.3.3

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

The contribution flags cross-band isolation MSD requirements that do not comply with the recently agreed rules for the RAN4 Permanent Reference Document (PRD) [1]. We also propose updating the PRD rules for 1) the so-called “second optional” MSD test point, and 2) to account for Rel-19 new smallest/new greatest CBW that may impact inter-band combinations specified with BCS4/5.

- Discussion

  - Background on PRD rules for cross-band isolation MSD

Key PRD rules for cross-band isolation MSD test points are reproduced and highlighted in yellow below from [1].

Key PRD rules on handling new greatest/smallest CBW are reproduced and highlighted in yellow below from [1].

  - Handling of previous Release “optional CBW”

To our understanding, channel bandwidth (CBW) that are specified as “optional” in Rel-18 with Note 4 and Note 12 in Table 5.3.5-1 are supposed to have “mandatory” support in Release-19. To illustrate the impact this may have on the Rel-19 cross-band isolation MSD requirements due to BCS4/5 configurations, we highlight in Table 1 in yellow the new greatest CBW, in blue the new smallest CBW, and in green new CBW that do not have any impact.

Table 1: Rel-18 optional CBW ported as in Rel 19.0.0 Table 5.3.5-1.

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n26 | 15 | 34 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |  |
| n28 | 15 | 34 | 5 | 10 | 15 | 207 | 257 | 307 |  | 404,7 |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 207 | 257 | 307 |  | 404,7 |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n31 | 15 | 34 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n41 | 15 |  | 54,11 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n46 | 15 |  |  | 105 |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  |  | 105 |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
|  | 60 |  |  | 105 |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
| n71 | 15 |  | 5 | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |
| n72 | 15 | 34 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n7910 | 15 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 704 | 80 | 90 | 100 |
|  | 60 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 704 | 80 | 90 | 100 |
| n85 | 15 | 34 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n90 | 15 |  | 54 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n96 | 15 |  |  |  |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
|  | 60 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
| n100 | 15 | 34 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n102 | 15 |  |  |  |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
|  | 60 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 1004 |
| n106 | 15 | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. NOTE 7: For UEs supporting up to 30 MHz channel bandwidth, the minimum requirements are specified for any NR UL channel bandwidth confined to 703-733 MHz or 718-748 MHz. For UEs supporting 40 MHz channel bandwidth, the minimum requirements are specified for any NR UL channel bandwidth confined to 703-743.04 MHz or 718-748MHz. NOTE 8: This UE channel bandwidth is applicable only to uplink. NOTE 9: Void. NOTE 10: For this band, UE channel bandwidths which are applicable to sidelink operation are specified in Table 5.3E.1-1. NOTE 11: Not all frequency positions of 5 MHz carriers are possible due limitations of the SSB position relative to the 5 MHz channels. 5 MHz channels with Fc such that 2499+N*1.2 ≤Fc<2499.3+N*1.2MHz for 0≤N<157 are not compatible with SSB positions and cannot be used for 5 MHz n41. NOTE 12: This UE channel Bandwidth is optional for uplink in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

New greatest CBW:

- Band n46 100MHz: no impact yet since the greatest supported UL CBW remains 80MHz for CA_n46-n48, CA_n46-n77/n78 in current BCS requests.

- Band n71 35MHz impact:

  - CA_n12-n71, CA_n20-n71, CA_n26-n71, CA_n28-n71, CA_n29-n71: no impact since BCS0 greatest UL n71 CBW remains 20MHz

  - CA_n5-n71, CA_n71-n85: 35MHz becomes the new greatest CBW due to BCS4/5 support, new MSD requirements are needed to replace first mandatory test point.

- Band n96 100MHz: no impact yet since the greatest supported UL CBW remains 80MHz for CA_n48-n96 in current BCS requests.

- Band n102 100MHz: Cross-band MSD requirements may be needed for CA_n77-n102 and CA_n78-n102 based on agreements for CA_n78-n104.

New smallest CBW:

- 3MHz in Bands n26, n28, n31, n85, n100: may impact several MSD requirements, not just only cross-band isolation.

- 5MHz in Bands n41 and n90 may impact several MSD requirements, not just only cross-band isolation. It is not clear if this CBW can be verified for conformance test due to footnote 11.

Observation 1: Rel-18 “optional CBW” should become “mandatory CBW” in Rel-19. This change introduces new smallest and new greatest CBW, which in turns may require revisiting several MSD requirements, not just only cross-band isolation.

Proposal 1: Remove optional CBW footnotes 4 and 12 for

- 3MHz CBW for Bands n26, n28, n31, n85, n100;

- 5MHz CBW for Bands n41, n90;

- 35MHz CBW for Band n71;

- 70MHz CBW for Band n79; and

- 100MHz CBW for Bands n46, n96 and Band n102.

Proposal 2: As an exception, do not consider the new smallest 3MHz CBW for MSD requirements, except in Band n106 where this is the only supported CBW. Update PRD rules accordingly.

Proposal 3: Companies to confirm if 5MHz can be configured for conformance test in Band n41 and Band n90. Consider revisiting all impacted Band n41/n90 MSD requirements: harmonic, cross-Band isolation, dual UL IMD.

Proposal 4: CA_n5A-n71A, consider introducing the following changes to the PC3 cross-Band isolation MSD requirements. Interested companies are invited to evaluate the new Band n5 “1st mandatory” MSD requirements.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Test point “ID” change | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |  |
| n71 | n5 | 688 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | 2.0 | becomes “2nd optional” | >ACLR2 |
| n71 | n5 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | [FFS] | becomes new “1st mandatory” | >ACLR2 |

Proposal 5: For PC3 CA_n71A-n85A, consider voiding footnotes 6 and 7 so that the second optional becomes the new “first mandatory”, and the “first mandatory” becomes the “second optional” test point.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Test point “ID” change | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2 | becomes “2nd optional” | >ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (RBstart=168) | 730.5 | 5 | 23 | becomes new “1st mandatory” | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-Band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting Band n78 with a n77 implementation. NOTE 4: Void NOTE 5: The MSD exceptions are applicable to the case that interference of UL Band 3rd order IMD product falls into the affected DL channels. NOTE 6: Void NOTE 7: Void |  |  |  |  |  |  |  |  |  |  |

For PC2 CA_n71A-n85A, for the same reasons, consider voiding note 4 and 5.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Test point “ID” change | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 10.96 15.97 | becomes “2nd optional” | >ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 266 32.37 | becomes new “1st mandatory” | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-Band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting Band n78 with a n77 implementation. NOTE 4: Void NOTE 5: Void NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx |  |  |  |  |  |  |  |  |  |  |

Proposal 6: Interested companies are invited to evaluate the PC3 cross-Band isolation MSD requirements for CA_n77A-n102A, CA_n78A-n102A.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n102 | n77 | 5975 | 100 | 30 | 270 (RBstart=0) | 4195 | 10 | [FFS] | >ACLR2 |
| n102 | n78 | 5975 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | [FFS] | >ACLR2 |

  - Non-compliant MSD test points

Table 2 summarizes the list of PC3 cross-Band isolation MSD test points that do not comply to the PRD rules [1], where we highlight in:

- Blue: non-compliance of the “ second optional” MSD test points,

- Yellow: non-compliance of the “first mandatory” MSD test points.

Table 2: List of non-compliant PC3 MSD requirements in Rel 19.0.0.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Test point “ID” | Greatest supported UL CBW by greatest BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |  |
| n1 | n3 | 1922.5 | 5 | 15 | 25 (RBstart=0) | 1877.5 | 5 | 3 | 2nd “optional” | n1: 50MHz, BCS4/5 |
| n1 | n3 | 1945 | 50 | 15 | 128 (RBstart=0) | 1877.5 | 5 | 19.7 | 1st “mandatory” |  |
| n1 | n40 | 1955 | 50 | 15 | 128 (RBstart=142) | 2302.5 | 5 | 6.6 | 1st “mandatory” | n1: 50MHz, BCS4/5 |
| n1 | n40 | 1970 | 20 | 15 | 100 (RBstart=6) | 2302.5 | 5 | 6.6 | 2nd “optional” |  |
| n1 | n41 | 1955 | 50 | 15 | 128 (RBstart=142) | 2501 | 10 | 6.1 | 1st “mandatory” | n1: 50MHz, BCS4/5 |
| n1 | n41 | 1970 | 20 | 15 | 100 (RBstart=6) | 2546 | 100 | 0.7 | 2nd “optional” |  |
| n3 | n74 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 2.6 | 1st “mandatory” | n3: 40MHz BCS0 |
| n3 | n75 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 4.3 | 1st “mandatory” | n3: 40MHz BCS0 |
| n66 | n2 | 1760 | 40 | 15 | 216 (RBstart=0) | 1932.5 | 5 | 1.2 | 2nd “optional” | n66: 45MHz BCS4/5 |
| n66 | n2 | 1757.5 | 45 | 15 | 240 (RBstart=2) | 1932.5 | 5 | 1.2 | 1st “mandatory” |  |
| n66 | n41 | 1760 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 0.4 | 1st “mandatory” | n66: 45MHz BCS4/5 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-Band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting Band n78 with a n77 implementation. NOTE 4: Void NOTE 5: The MSD exceptions are applicable to the case that interference of UL Band 3rd order IMD product falls into the affected DL channels. NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel Bandwidth NOTE 7: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel Bandwidth |  |  |  |  |  |  |  |  |  |  |

Observation 2:  Second optional MSD test points for CA_n1-n3, CA_n1-n40, CA_n1-n41 and CA_n2-n66 are not configured with the highest supported UL CBW. All other rules are met, e.g., the smallest UL/DL separation distance. We also observe that some of these test points have been agreed since Rel-17, i.e., somehow, they’ve become “legacy” requirements.

Observation 3: The UL CBW configurations specified for the following first mandatory MSD test point no longer comply with the PRD rules. For CA_n3-n74 and CA_n3-n75, the Band n3 UL CBW should be changed to 40MHz. We also observe that the DL Band n75 carrier frequency should be changed to 1514.5MHz. For CA_n41-n66, the Band n77 UL CBW should be changed to 45MHz.

Proposal 7: Relax the PRD rules regarding the “second optional” MSD for cross-Band isolation to allow configuring the UL CBW with other CBW than the maximum supported by the highest BCS.

Proposal 8: Interested companies are invited to evaluate the following “first mandatory” cross-Band isolation MSD requirements for CA_n3A-n74A, CA_n3A-n75A. Current MSD test points may be kept and changed to become “second optional” MSD test points as captured below.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Test point “ID” change | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |  |
| n3 | n74 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 2.6 | becomes “2nd optional” | >ACLR2 |
| n3 | n74 | 1730 | 40 | 15 | 50 (RBstart=0) | 1515.5 | 5 | [FFS] | becomes new “1st mandatory” | >ACLR2 |
| n3 | n75 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1514.5 | 5 | 4.3 | becomes “2nd optional” | >ACLR2 |
| n3 | n75 | 1730 | 40 | 15 | 50 (RBstart=0) | 1514.5 | 5 | [FFS] | becomes new “1st mandatory” | >ACLR2 |

Proposal 9: Consider adopting the following PC2 MSD corrections for CA_n2-n66 based on CA_n25-n66 agreements.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n2 | n66 | 1890 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 | 0.76 0.97 | >ACLR2 |

- Conclusion

This paper captures our views on simplifying the 1UL MSD requirements for Rel-19 HPUE NRCA inter-Band combinations.

Observation 1: Rel-18 “optional CBW” should become “mandatory CBW” in Rel-19. This change introduces new smallest and new greatest CBW, which in turns may require to revisit several MSD requirements, not just only cross-Band isolation.

Proposal 1: Remove optional CBW footnotes 3 and 12 for

- 3MHz CBW for Bands n26, n28, n31, n85, n100;

- 5MHz CBW for Bands n41, n90;

- 35MHz CBW for Band n71;

- 70MHz CBW for Band n79; and

- 100MHz CBW for Bands n46, n96 and Band n102.

Proposal 2: As an exception, do not consider the new smallest 3MHz CBW for MSD requirements except in Band n106 where this is the only supported CBW. Update PRD rules accordingly.

Proposal 3: Companies to confirm if 5MHz can be configured for conformance test in Band n41 and Band n90. Consider revisiting all impacted Band n41/n90 MSD requirements: harmonic, cross-Band isolation, dual UL IMD.

Proposal 4: CA_n5A-n71A, consider introducing the following changes to the PC3 cross-Band isolation MSD requirements. Interested companies are invited to evaluate the new Band n5 “1st mandatory” MSD requirements.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Test point “ID” change | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |  |
| n71 | n5 | 688 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | 2.0 | becomes “2nd optional” | >ACLR2 |
| n71 | n5 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | [FFS] | becomes new “1st mandatory” | >ACLR2 |

Proposal 5: For PC3 CA_n71A-n85A, consider voiding footnotes 6 and 7 so that the 2nd optional becomes the new “1st mandatory”, and the “1st mandatory” becomes the “2nd optional” test point.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Test point “ID” change | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2 | becomes “2nd optional” | >ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (RBstart=168) | 730.5 | 5 | 23 | becomes new “1st mandatory” | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-Band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting Band n78 with a n77 implementation. NOTE 4: Void NOTE 5: The MSD exceptions are applicable to the case that interference of UL Band 3rd order IMD product falls into the affected DL channels. NOTE 6: Void NOTE 7: Void |  |  |  |  |  |  |  |  |  |  |

For PC2 CA_n71A-n85A, for same reasons, consider voiding note 4 and 5.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Test point “ID” change | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 10.96 15.97 | becomes “2nd optional” | >ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 266 32.37 | becomes new “1st mandatory” | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-Band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting Band n78 with a n77 implementation. NOTE 4: Void NOTE 5: Void NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx |  |  |  |  |  |  |  |  |  |  |

Proposal 6: Interested companies are invited to evaluate the PC3 cross-Band isolation MSD requirements for CA_n77A-n102A, CA_n78A-n102A.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n102 | n77 | 5975 | 100 | 30 | 270 (RBstart=0) | 4195 | 10 | [FFS] | >ACLR2 |
| n102 | n78 | 5975 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | [FFS] | >ACLR2 |

Observation 2: Second optional MSD test points for CA_n1-n3, CA_n1-n40, CA_n1-n41 and CA_n2-n66 are not configured with the highest supported UL CBW. All other rules are met, e.g., the smallest UL/DL separation distance. We also observe that some of these test points have been agreed since Rel-17, i.e., somehow, they have become “legacy” requirements.

Observation 3: The UL CBW configurations specified for the following 1st mandatory MSD test point no longer comply with the PRD rules. For CA_n3-n74 and CA_n3-n75, the Band n3 UL CBW should be changed to 40MHz. We also observe that the DL Band n75 carrier frequency should be changed to 1514.5MHz. For CA_n41-n66, the Band n77 UL CBW should be changed to 45MHz.

Proposal 7: Relax the PRD rules regarding the “second optional” MSD for cross-Band isolation to allow configuring the UL CBW with other CBW than the maximum supported by the highest BCS.

Proposal 8: Interested companies are invited to evaluate the following “first mandatory” cross-Band isolation MSD requirements for CA_n3A-n74A, CA_n3A-n75A. Current MSD test points may be kept and changed to become “second optional” MSD test points as captured below.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Test point “ID” change | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |  |
| n3 | n74 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 2.6 | becomes “2nd optional” | >ACLR2 |
| n3 | n74 | 1730 | 40 | 15 | 50 (RBstart=0) | 1515.5 | 5 | [FFS] | becomes new “1st mandatory” | >ACLR2 |
| n3 | n75 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1514.5 | 5 | 4.3 | becomes “2nd optional” | >ACLR2 |
| n3 | n75 | 1730 | 40 | 15 | 50 (RBstart=0) | 1514.5 | 5 | [FFS] | becomes new “1st mandatory” | >ACLR2 |

Proposal 9: Consider adopting the following PC2 MSD corrections for CA_n2-n66 based on CA_n25-n66 agreements.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n2 | n66 | 1890 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 | 0.76 0.97 | >ACLR2 |

- References

- R4-2503022 Input to RAN4 PRD on rules for 1UL CC MSD requirements, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc., Murata.