3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503241

Wuhan, China, April 7th – 11th, 2025

# Title:	DC_(n)3CA MSD evaluation

# Agenda Item:	6.2.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

This contribution proposes a single one-uplink (1 UL) MSD test point for downlink (DL) DC_(n)3CA following the agreed Way Forward (WF) [1]. Proponents also clarified that BCS1 request for DC_(n)3AA is no longer required, hence there is no need to evaluate whether the need exists for an additional MSD requirement for DC_(n)3AA.

- Discussion

  - Background on Valid UL Configurations for DL DC_(n)3CA

To our understanding, DL DC_(n)3CA is requested with two different UL configurations: DC_(n)3AA and DC_3A_n3A. For both, only single-switched UL is supported, meaning that MSD requirements need to be assessed only for 1UL transmissions in band n3 with the following parameters:

- DL maximum aggregated BW of 50 MHz

- n3 DL maximum CBW of 30 MHz

- DL E-UTRA CA_3C BCS0 (i.e., 5, 10, 15, and 20 MHz CBW per CC).

  - Handling of E-UTRA CA_3C PCC/SCC MSD

The WF [1] agreements are reproduced below.

It was initially considered that both E-UTRA DL cells are secondary component carriers (SCC), hence it may be sufficient to specify the MSD of the first SCC, “SCC1” for example, and that this SCC1 MSD also applies by default to all subsequent SCCs (SCC2 in the case of CA_3C). However, for DC_(n)3CA, the E-UTRA DL cells of CA_3C may either be Pcell or Scell. MSD requirements are therefore needed for each DL CC.

Observation 1: According to the latest RAN4 PRD rules [2] for 1UL intra-band contiguous MSD, the E-UTRA MSD needs to be evaluated. Since both E-UTRA cells may either be Pcell or Scell, MSD requirements are needed on both DL CC. PRD rules need to reflect this new scenario.

  - MSD Test Point Selection

We reproduce in Table 1 the test points 1, 2, 3, and 4 from [3], to which we add:

- Two additional candidates: 5 and 6

- The legacy agreed MSD test point for DC_(n)3AA as baseline reference

- The approximate 1UL IMD order affecting both E-UTRA DL cells

Table 1: DC_(n)3CA MSD test point candidates.

| EN-DC configuration | E-UTRA/ NR band | FC (UL) | Channel bandwidth | UL allocation | FC (DL) | 1UL IMD order affecting E-UTRA | MSD | Candidate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (LCRB) | (MHz) |  | (dB) |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | 5+7 | [TBD] | #1 |
|  | 3 | N/A | 20 | N/A | 1845 | 7 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 50 (RBstart = 83) | 1867.5 | N/A | N/A |  |
| DC_(n)3CA | 3 | N/A | 20 | N/A | 1840 | mostly 7 / n-a | [TBD] | #2 |
|  | 3 | N/A | 5 | N/A | 1852.5 | mostly 7 / n-a | [TBD] |  |
|  | n3 | 1772.5 | 25 | 50 (RBstart = 83) | 1867.5 | N/A | N/A |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | 5 / -45 | [TBD] | #3 |
|  | 3 | N/A | 20 | N/A | 1845 | 7 / -50.4 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 6 (RBstart = 127) | 1867.5 | N/A | N/A |  |
| DC_(n)3CA | 3 | N/A | 20 | N/A | 1840 | 5 / n-a | [TBD] | #4 |
|  | 3 | N/A | 5 | N/A | 1852.5 | 7 / n-a | [TBD] |  |
|  | n3 | 1772.5 | 25 | 3 (RBstart = 130) | 1867.5 | N/A | N/A |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | 5+7 | [TBD] | #5 |
|  | 3 | N/A | 15 | N/A | 1842.5 | 7 | [6.2] |  |
|  | n3 | 1770.0 | 30 | 50 (RBstart = 110) | 1865.0 | N/A | N/A |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | partial 5 | [TBD] | #6 |
|  | 3 | N/A | 15 | N/A | 1842.5 | 5 | [6.2] |  |
|  | n3 | 1770.0 | 30 | 10 (RBstart = 150) | 1865.0 | N/A | N/A |  |
| DC_(n)3CA | 3 | N/A | 15 | N/A | 1842.5 | 5 | 6.2 | Legacy |
|  | n3 | 1770.0 | 30 | 10 (RBstart = 150) | 1865.0 | N/A | N/A |  |

To help better visualize the corresponding IMD landscape, we sketch an approximate representation of candidate #6 versus the legacy DC_(n)3AA test point.

For all candidates, it can be noted that:

- The lowest IMD order that may affect the E-UTRA DL cell closest to the UL band is IMD5. This is the same source of interference as that already specified for the legacy DC_(n)3AA requirements. Hence, it is expected that the MSD levels for each of these candidates will be very similar and fairly small.

- The DL E-UTRA cell adjacent to the NR DL cell is affected at worst by IMD7, hence a lower interference level is expected on that cell and a low MSD can be expected as well.

- As highlighted in green, test point #6 can be considered as a simple extension of the legacy DC_(n)3AA MSD test point, where the 5 MHz E-UTRA DL CC is just adjacent to the E-UTRA 15 MHz and NR 30 MHz DL CCs. This means that the legacy 6.2 dB MSD requirement of 15 MHz CBW E-UTRA CC can be re-used and does not need to be re-evaluated.

- Considering RAN4 is in the process of simplifying and cleaning MSD test points for intra-band CA/DC, it is preferable to consider adopting a single MSD test point.

Observation 2: The interference mechanism affecting the E-UTRA DL CCs is nearly the same for all test point candidates and is also identical to the legacy test point for DC_(n)3AA. At worst, the E-UTRA cell closest to the uplink band may be affected by a 1UL IMD5 product and the other adjacent E-UTRA cell by an IMD7 product. In other words, all candidates may lead to similar MSD requirements.

Observation 3: For test point #6, the MSD of the E-UTRA 15 MHz can be re-used from DC_(n)3AA. Only the MSD requirement of the E-UTRA 5 MHz CC needs to be evaluated.

Proposal 1: Consider adopting the following PC3 MSD test point for DC_(n)3CA. The 6.2 dB B3 15 MHz MSD can be copied from DC_(n)3AA.

| EN-DC configuration | E-UTRA/ NR band | FC (UL) | Channel bandwidth | UL allocation | FC (DL) | MSD | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (LCRB) | (MHz) | (dB) |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [TBD] | FDD |
|  | 3 | N/A | 15 | N/A | 1842.5 | 6.2 |  |
|  | n3 | 1770.0 | 30 | 10 (RBstart = 150) | 1865.0 | N/A |  |

  - MSD Evaluation

For PA transmitter (Tx) noise emission measurements in the receiver (Rx) band, we assume:

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB maximum power reduction (MPR)

- Power Class 3 (PC3) operation and 4 dB post PA loss

- Local Oscillator (LO) leakage: –28 dBc; IQ Image rejection: –28 dB

- C-IM3: –60 dBc, C-IM5: –70 dBc

For REFSENS/MSD estimations, we assume:

- 50 dB Tx to RX and 45 dB Tx to antenna duplexer rejection in Rx band

- 10 dB primary antenna to diversity antenna isolation

- 4 dB post PA loss and 4 dB antenna to LNA front-end losses

- 3 dB antenna diversity gain and –1 dB SNR

As background, we reproduce in Figure 1 the PA measurements performed to evaluate the DC_(n)3AA MSD evaluation for 15 MHz E-UTRA DL CC [4]. For LCRB = 10 (RBstart =150) we measured a noise of –41 dBm/MBW. This level results in a lower MSD than the agreed 6.2 dB MSD.

Figure 1: Band n3 30 MHz CBW PA noise levels affecting B3 15 MHz CBW vs UL RB allocation, with UL RB and NR UL carrier positioned closest to the DcL band.

In Table 2, the B3 5 MHz/15 MHz MSD is evaluated at 2.9 dB and 3.1 dB based on the updated measurements results below:

- PA noise affecting B3 15 MHz CC: –40.4 dBm/13.5 MHz, a level close to that plotted in Figure 1

- PA noise affecting B3 15 MHz CC: –45.8 dBm/4.5 MHz

Table 2: DC_(n)3CA MSD evaluation based on measurement results.

Since the B3 15 MHz CC MSD is smaller than the agreed 6.2 dB MSD, we evaluate in Table 3 the B3 5 MHz MSD by reverse engineering the PA interference level which corresponds to the agreed B3 15 MHz 6.2 dB MSD, and apply the difference of interference levels measured for 5 MHz CC versus 15 MHz CC to that level.

Table 3: DC_(n)3CA MSD evaluation scaled to reflect the legacy DC_(n)3AA MSD.

Proposal 2: Consider adopting the following PC3 MSD requirement for DC_(n)3CA.

| EN-DC configuration | E-UTRA/ NR band | FC (UL) | Channel bandwidth | UL allocation | FC (DL) | MSD | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (LCRB) | (MHz) | (dB) |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | 5.8 | FDD |
|  | 3 | N/A | 15 | N/A | 1842.5 | 6.2 |  |
|  | n3 | 1770.0 | 30 | 10 (RBstart = 150) | 1865.0 | N/A |  |

- Conclusion

This paper presents PA measurements to evaluate the DC_(n)3CA MSD requirements.

Observation 1: According to the latest RAN4 PRD rules [2] for 1UL intra-band contiguous MSD, the E-UTRA MSD needs to be evaluated. Since both E-UTRA cells may either be Pcell or Scell, MSD requirements are needed on both DL CC. PRD rules need to reflect this new scenario.

Observation 2: The interference mechanism affecting the E-UTRA DL CCs is nearly the same for all test point candidates and is also identical to the legacy test point for DC_(n)3AA. At worst, the E-UTRA cell closest to the uplink band may be affected by a 1UL IMD5 product and the other adjacent E-UTRA cell by an IMD7 product. In other words, all candidates may lead to similar MSD requirements.

Observation 3: For test point #6, the MSD of the E-UTRA 15 MHz can be re-used from DC_(n)3AA. Only the MSD requirement of the E-UTRA 5 MHz CC needs to be evaluated.

Proposal 1: Consider adopting the following PC3 MSD test point for DC_(n)3CA. The 6.2 dB B3 15 MHz MSD can be copied from DC_(n)3AA.

| EN-DC configuration | E-UTRA/ NR band | FC (UL) | Channel bandwidth | UL allocation | FC (DL) | MSD | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (LCRB) | (MHz) | (dB) |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [TBD] | FDD |
|  | 3 | N/A | 15 | N/A | 1842.5 | 6.2 |  |
|  | n3 | 1770.0 | 30 | 10 (RBstart = 150) | 1865.0 | N/A |  |

Proposal 2: Consider adopting the following PC3 MSD requirement for DC (n)3CA.

| EN-DC configuration | E-UTRA/ NR band | FC (UL) | Channel bandwidth | UL allocation | FC (DL) | MSD | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (LCRB) | (MHz) | (dB) |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | 5.8 | FDD |
|  | 3 | N/A | 15 | N/A | 1842.5 | 6.2 |  |
|  | n3 | 1770.0 | 30 | 10 (RBstart = 150) | 1865.0 | N/A |  |

- References

- R4-2502961 WF on DC_(n)3AA and DC_(n)3CA, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Murata, Skyworks Solutions, inc.

- R4-2503022 Input to RAN4 PRD on rules for 1UL CC MSD requirements, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, inc., Murata.

- R4-2501078 MSD test point for DC_(n)3CA, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Murata.