3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2503278

# Wuhan, China, 7 – 11 April, 2025

Title:	LS on Rel-19 signaling support for intra-band non-collocated NR-CA, EN-DC

Response to:

Release:	Rel-19

Work Item:	          NonCol_intraB_ENDC_NR_CA

Source:	RAN4

To:	RAN2

Cc:	-

Contact person:	Mohammad Abdi Abyaneh, mohammad.abdi.abyaneh@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

1) New UE capability

For UE to indicate the support of non-collocated deployment for FDD-FDD or TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping bands, the UE capabilities “interBandMRDC-WithOverlapDL-Bands-r16” and “requirementTypeIndication-r18” were introduced in Rel-16 and Rel-18, respectively,

UE capability “intraBandNR-CA-non-collocated-r18” was introduced in Rel-18 to support non-collocated deployment for TDD-TDD intra-band NR-CA.

In Rel-19 we would like you to consider the following requirements for the NR CA and the END-DC case as follows:

For TDD-TDD intra-band NR-CA

RAN4 concluded a separate new capability signalling is needed, which is to indicate the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4.1/Table 7.6.4-2 in 38.133 and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in 38.101-1 (Type 4 capability requirements, such as 4Rx/CC).

If the new UE capability and “intraBandNR-CA-non-collocated-r18” are absent, the UE supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1 (Type 1 capability requirement).

RAN4 also concluded the UE indicating the “New UE capability” can also support Type 1 and Type 2 capability requirements.

This new UE optional capability is expected to be per BC, and applicable to only band n77 and n78.

For TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands

RAN4 concluded a separate new capability signalling is needed, which is to indicate the UE supports TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping DL bands with an EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and inter-band RF requirements. (Type 4 capability requirements, such as 4Rx/CC).

To avoid confusion between type 2 and type 4 ”interBandMRDC-WithOverlapDL-Bands-r16” cannot be reused.

2) New BS signalling

RAN4 agreed to introduce new BS signalling by RRC to indicate whether Type 1 or Type 4 capability requirements will be applied by a UE configured by the network with the value of IE maxMIMO-Layers no more than 4 and task RAN2 to develop relevant signalling.

This new BS signalling is applied to the below cases.

For TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands

This new BS signalling is expected to be applicable for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands provided the UE has indicated “New UE capability”.

For TDD-TDD intra-band NR-CA

This new BS signalling is expected to be applicable for TDD-TDD intra-band NR-CA provided the UE has indicated “New UE capability”.

# 2	Actions

To RAN2:

ACTION: 	RAN4 respectfully requests RAN2 to specify, in Rel-19, the above UE capabilities reporting for intra-band non-collocated NR-CA and inter-band EN-DC with overlapping or partially overlapping bands. Moreover, the above new BS signallings by RRC to indicate whether Type 1 or Type 4 capability requirements will be applied by a UE configured by the network with the value of IE maxMIMO-Layers no more than 4 and task RAN2 to develop relevant signalling.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #115                 19 – 23 May 2025		Malta, Malta

TSG-RAN WG4 Meeting #116                 25 – 29 August 2025		Bengaluru, India