# 3GPP TSG-RAN WG4 Meeting #114bis  		R4-2503293

WuHan, China, 07 April – 11 April, 2025

Source: 	vivo

Title: 	Discussion on RRM requirements for on-demand SSB SCell operation in R19 NES

Agenda Item:	7.25.2.1

Document for:	Discussion

Introduction

In RAN4 #114, the WF [4] is agreed. Previous WF and LS to RAN1 can be found at [1], [2] and RAN1 reply LS can be found at [3].

Based on all above information, we provide our views on RRM requirement impacts for on-demand SSB SCell operation in R19 NES.

Discussion

- General On-demand SSB(OD-SSB) requirements

Issue 1-1-1: Additional Processing time for OD-SSB reception – scenario

- Proposal

  - Option 1: Nokia

      - The processing/preparation time for OD-SSB reception shall not apply at least in the following cases:

- If OD-SSB has been activated recently, or

- If OD-SSB is transmitted on the same frequency with always-on SSB

  - Option 2: CMCC

      - Only for the following cases if valid, additional processing time (Tprocessing) may be needed for UE to measure the OD-SSB:

- No AO-SSB on the SCell (Case 1)

- AO-SSB is transmitted on the SCell (Case 2), while OD-SSB and AO-SSB are within different frequencies and UE is expected to perform measurement on both OD-SSB and AO-SSB

  - Option 3: ZTE

      - For Case 1, the searcher re-scheduling could be performed within the duration of waiting for the first on-demand SSB occasion.

      - For Case 2, no need searcher re-scheduling at least for the case of same frequency between on-demand SSB and always-on SSB.

  - Option 4: Huawei

    - No additional UE processing/preparation for OD-SSB for SCell activation (i.e. Scenario 2A).

    - RAN4 to discuss the value of additional UE processing/preparation for OD-SSB only for deactivated SCell L3 measurement (i.e. Scenario 2).

Issue 1-1-2: Additional Processing time for OD-SSB reception - value

- Proposal

  - Option 1: CMCC

    - 1 slot

  - Option 2: vivo

    - 1 ms

  - Option 3: LG

    - If Time instance A is located within the first possible OD-SSB burst, start measurement at the next OD-SSB burst.

  - Option 4: Qualcomm

    - RAN4 to discuss whether a single margin is introduced to cover the worst-case preparation time or whether multiple margins are introduced that reflect different configuration scenarios.

In legacy L3 measurement requirements in connected mode, CSSF is defined for both measurement outside gaps and measurement within gaps. For measurement within gaps, as cited from Table 9.1.5.2.1-1 as follows, one searcher is assumed for PCC, and one searcher is shared between PSCC, SCC and inter-frequency SSB measurements without gaps, if any. Considering PCC and SCC, an illustration of UE measurement behaviour for outside gap case is illustrated in Figure 1. Note that UE may not always perform cell search between layers in alternative manner, i.e. search on another SCC layer after it collect enough samples on one frequency layer. A round robin manner would be assumed for UE measurement scheduler.

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required; CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required; CSSFoutside_gap,i for inter-frequency MO with no measurement gap; CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap [TBD]. FR1 only CA; 1+NPCC_CSIRS + NPCC_CCA_RSSI/CO; NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO; N/A; N/A; N/A; NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO; NSCC_SSB +Y+Z+2x NSCC_CSIRS. FR2 only intra band CA; N/A; N/A; 1+NPCC_CSIRS; N/A; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS. FR2 only inter band CA; N/A; N/A; 1+NPCC_CSIRS; 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5; 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM). FR1 +FR2 CA (FR1 PCell) Note 1; 1+NPCC_CSIRS; 2×( NSCC_SSB +Y+Z+2* NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); N/A; 2x(1+ NSCC_CSIRS_FR2_NCM) Note 3,5; 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM). FR1 +FR2 CA (FR2 PCell) Note 1; N/A; NSCC_SSB +Y+Z+2x NSCC_CSIRS; 1+NPCC_CSIRS; N/A; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS. Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. Note 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. Note 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. Note 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. Note 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured Note 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. Note 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. Note 10: NPCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. Note 11: Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0..

Figure 1  Illustration of connected mode CSSF for L3 measurement performed outside gaps. For SCC, UE performs cell search on each SCC in a round-robin manner

In this case UE would need some time to re-schedule its searcher, and prepare RF for this carrier. Such time would be different, depending on different cases. For example, for case 1, RF retuning is anyway needed. However, for case 2, when OD-SSB and AO-SSB(always-on SSB) have the same SSB frequency, and the spatial relation of the two kinds of SSBs are the same, then UE may not need to retune its RF, but still needs to perform cell search if the SCell is unknown, hence time for re-scheduling its searcher is needed. To deal with the worst case without complicating the spec, it is proposed to consider 1 ms for UE processing time.

Proposal 1: Additional UE processing time is needed after OD-SSB is activated by RRC or MAC CE. Prefer [1] ms for this delay.

Issue 4-2: L1 measurement after SCell activation

Agreement:

- RAN4 to support OD-SSB L1 measurement during and after SCell activation.

  - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB

- RAN4 to support OD-SSB L1 measurement during SCell activation.

  - The requirement when DRX is configured.

    - Based on OD-SSB periodicity

Wayforward:

- RAN4 to support OD-SSB L1 measurement after SCell activation.

  - FFS the requirement when DRX is configured.

    - Option 1: Follow legacy, based on DRX cycle (vivo, Apple, OPPO, Xiaomi)

    - Option 2: Based on OD-SSB periodicity (Nokia, ZTE, Samsung)

    - Option 3: FFS (HW, QC, Nokia, CMCC, Xiaomi, CATT)

    - Note: the OD-SSB is triggered before or together with the Scell activation command, the OD-SSB transmission kept after the SCell activation until the OD-SSB deactivation command received.

Proposal 2: For the requirement for OD-SSB L1 measurement after SCell activation when DRX is configured, requirements are based on DRX cycle, i.e., option 1.

Issue 1-1-6: Cell identification

The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios.

- Proposals

  - Option 1-1: Ericsson, LG, ZTE

    - UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [Xs] and then the subsequent OD-SSB is transmitted with the same spatial reception parameter.

  - Option 1-2: CMCC

    - The UE has sent a valid measurement report based on either AO-SSB or OD-SSB for the SCell being activated [5]s before,

    - The subsequent OD-SSB is transmitted with the same spatial reception parameter, same transmission power, and on the same frequency carrier as the OD-SSB or AO-SSB used for the valid measurement report,

      - provided the timing to this cell has not changed more than  3200/2µ Tc while L3 filtering has not been used, when L3 filtering is used, an additional delay can be expected

  - Option 2: Ericsson, LG

    - Case 2: Always-on SSB on the cell

      - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.

      - Scenario 3: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities.

  - Option 3: Ericsson, ZTE

    - OD-SSB Scell being measured is contiguous to an active serving cell in the same band.

  - Option 4: Apple, vivo

    - Extend referenceCell to OD-SSB SCell to skip cell identification

Proposal 3: The OD-SSB based cell identification before deactivated SCell measurement can be skipped based on option 4.

- OD-SSB based deactivated SCell L3 measurement requirement in FMW (Case 1)

- Sub-issue 1-2: What’s the expected minimum interval(T2, sage 4) between multiple FMWs?

Agreement:

RAN4 to define measurement requirement based on T2.

- When NW re-activates the OD-SSB within the T2,

  - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

  - Option 2: UE is required to perform “fast measurement” without cell search.

    - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition.

Option 3: UE is required to perform FMW after T2.

- When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW

  - The UE is assumed to meet fast cell identification and measurement.

- The start point of T2 is based on

  - Option 1: the time point Tend

  - Option 2: the latter point of L3 measurement reporting for FMW and Tend

  - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend

  - Option 4: the time point of OD-SSB deactivation

  - Other options are not precluded

Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3.

- The duration is

  - Option 1: 5*max(measCycleSCell,  DRX cycles) [– OD-SSB periodicity]

  - Option 2: 5* measCycle when measCycle =160 ms

  - Other options are not precluded

The activation and deactivation of OD-SSB is fully up to NW

Proposal 4：When NW re-activates the OD-SSB within the T2, prefer UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

Regarding the starting point of T2, the main purpose of T2 is to ensure UE leaves fast measurement mode and have power saving. Hence when the length of FMW is defined, the starting point of T2 is not related to the L3 reporting time and even not related to the OD-SSB deactivated point. The starting point of T2 can be the beginning of the FMW, option 4 is also acceptable.

Proposal 5: The start point of T2 can be the beginning of the FMW or based on option 4.

For the duration of T2, in principle either option 1 or option 2 works, considering the range of measCycle is from 160ms to 1280ms, which is similar to that of DRX. Option 2 is preferred for simplicity reason.

Proposal 6: The duration of T2 is 5* measCycle.

- OD-SSB based deactivated SCell L3 measurement requirement in FMW (Case 1)

Sub-issue 3-1: Lower bound for measurement in FMW

- Background

  - The agreement in last meeting is as follow.

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF.

Wayforward:

- Option 1: RAN4 not to define lower bound to measurement in FMW.

- Option 2: Define a lower bound with OD-SSB periodicity as [10ms].

  - The lower bound is only applied to measurement in FMW.

Proposal 7: For the FMW, support OD-SSB based deactivated SCell measurement and a lower bound to measurement in FMW may need further discussion.

Sub-issue 3-2: OD-SSB measurement in FMW(Case 1-1): single OD-SSB carrier with multiple legacy SSB carriers

Agreement:

When NW configures single OD-SSB carrier with multiple legacy SSB carriers, in FMW

- Option 1: The CSSF only counts OD-SSB layers

- Option 2: The CSSF of OD-SSB carrier counts OD-SSB layers and other layers

For the other case of CSSF, if the SCell with OD-SSB is known (for both case 1 and case 2), or if UE obtains rough timing of the SCell based on referenceCell indication, UE may perform OD-SSB based L3 measurement in parallel with other frequency layers, since cell search is not needed. Although it is not yet clear whether UE L3 reporting can be triggered the MAC CE based OD-SSB activation, it is somehow beneficial for the network power saving, if UE report the L3 results immediately after the MAC CE based OD-SSB activation, since it would extend the possibility of the known scenario. However, we do not see any issue if RAN4 re-use legacy L3 measurement requirements.

Proposal 8: For the other case of CSSF within FMW, OD-SSB based L3 measurement is prioritized over legacy L3 measurement on PSCC and other SCCs, i.e. RRM requirements on PSCC and other SCCs are not applicable during the time-window when UE performs OD-SSB based L3 measurement.

- This applies to both Case 1 and Case 2

- This only applies to SCell unknown case.

Proposal 9: If the SCell is known, or if UE obtains rough timing of the SCell based on referenceCell indication, UE performs OD-SSB based L3 measurement following legacy SCell L3 measurement requirements.

- UE may report L3 results of the SCell based on OD-SSB triggering MAC CE if RAN1/2 agrees, but it is up to UE whether/how to perform L3 measurement in this case. No additional impact to L3 measurement requirements.

Sub-issue 3-3: OD-SSB measurement in FMW(Case 1-2): Multiple active OD-SSB carriers

Agreement:

RAN4 to prioritize single OD-SSB carrier with multiple legacy SSB carriers in deactivated SCell measurement.

- FFS multiple OD-SSB carriers

Proposal 10: Multiple OD-SSB carriers can be discussed later.

- OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 2-1: (Case 2) OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered

Agreement:

When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2),

- Option 1: The same approach in Case 1 can be reused.

  - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell).

- Option 2:

  - If the serving cell is unknown, the same approach in Case 1 can be reused.

  - If the serving cell is known, UE follows legacy requirements based on measCycleSCell.

    - The known condition reuses the principle of legacy known condition.

    - This scenario applies at least when UE sends the meas. reporting.

  - FFS the case if the L3 meas. report cannot be triggered.

- The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

Proposal 11: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), prefer option 2.

In addition, at LS [R4-2503111], the following agreements are available:

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB,

- For the case when the center frequency locations of always-on SSB and on-demand SSB are same,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - From RAN1 perspective,

    - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification).

    - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported.

  - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2

  - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB

    - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB

- For the case when the center frequency locations of always-on SSB and on-demand SSB are different,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain.

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB

- NOTE: AO-SSB periodicity is not adapted

In addition RAN4 considers the following scenarios for always-on SSB and OD-SSB relation:

  - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.

  - Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities.

  - Scenario 3: OD-SSB and always-on SSB are within different frequencies.

  - Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities.

For the scenario 1 with Alt Time-C1, a few possibilities can be identified, for example AO-SSB is 160ms and OD-SSB is 160ms, 80ms, 40ms and 20ms. When the OD-SSB and AO-SSB periodicity is the same, the effective measurement periodicity is half of either the OD-SSB or AO-SSB periodicity. When OD-SSB and AO-SSB periodicity is different, the measurement periods could be always the minimum between AO-SSB and OD-SSB, i.e., min(AO-SSB, OD-SSB).

Proposal 12: For the scenario 1 with Alt Time-C1 under case 2, when the OD-SSB and AO-SSB periodicity is the same, the effective measurement periodicity is half of either the OD-SSB or AO-SSB periodicity. When OD-SSB and AO-SSB periodicity is different, the effective measurement periodicity could be always the minimum between AO-SSB and OD-SSB, i.e., min(AO-SSB, OD-SSB).

For Alt Time-C2, the OD-SSB is transmitted for a purpose hence logically requirements should be defined for the Alt Time-C2 case the the measurement periodicity could be based on the OD-SSB periodicity.

Proposal 13: For Alt Time-C2 under case 2, measurement periodicity could be based on the OD-SSB periodicity.

<On intra-band CA>

For intra-band co-located CA case, how much NW energy saving gain can be achieved for the OD-SSB case would be unclear, and such scenario significantly complicates the UE RRM requirements for SCell activation. As discussed in the SI phase, most NW energy comes from gNB TxRUs to support large antenna array, and with intra-band collocated active serving cell, such TxRUs shall be already activated. Since the power consumption is already high, periodic SSB transmission, if needed, on an intra-band collocated SCell, will not cause a significant power consumption issue. On the other hand, no matter SSB is transmitted or not for an intra-band collocated SCell, spec impacts would be quite large for both cases, at least in FR1. Therefore, RAN4 can consider deprioritize such scenario, since the necessity of such feature and the network energy saving gain are not clear, but big spec impact is foreseen.

Proposal 14: RAN4 to deprioritize the intra-band co-located CA for OD-SSB based SCell operation, at least in FR1.

For intra-band non-collocated CA in FR1, as discussed in R18, it shall be up to UE capability whether AGC can be adjusted for two intra-band CCs with non-collocated serving cells separately. For simplicity, it is possible to only consider the case UE have such capability.

Proposal 15: For intra-band non-collocated CA in FR1, only consider UE with similar behaviour as supporting intraBandNR-CA-non-collocated-r18

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1: Additional UE processing time is needed after OD-SSB is activated by RRC or MAC CE. Prefer [1] ms for this delay.

Proposal 2: For the requirement for OD-SSB L1 measurement after SCell activation when DRX is configured, requirements are based on DRX cycle, i.e., option 1.

Proposal 3: The OD-SSB based cell identification before deactivated SCell measurement can be skipped based on option 4.

Proposal 4：When NW re-activates the OD-SSB within the T2, prefer UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

Proposal 5: The start point of T2 can be the beginning of the FMW or based on option 4.

Proposal 6: The duration of T2 is 5* measCycle.

Proposal 7: For the FMW, support OD-SSB based deactivated SCell measurement and a lower bound to measurement in FMW may need further discussion.

Proposal 8: For the other case of CSSF within FMW, OD-SSB based L3 measurement is prioritized over legacy L3 measurement on PSCC and other SCCs, i.e. RRM requirements on PSCC and other SCCs are not applicable during the time-window when UE performs OD-SSB based L3 measurement.

- This applies to both Case 1 and Case 2

- This only applies to SCell unknown case.

Proposal 9: If the SCell is known, or if UE obtains rough timing of the SCell based on referenceCell indication, UE performs OD-SSB based L3 measurement following legacy SCell L3 measurement requirements.

- UE may report L3 results of the SCell based on OD-SSB triggering MAC CE if RAN1/2 agrees, but it is up to UE whether/how to perform L3 measurement in this case. No additional impact to L3 measurement requirements.

Proposal 10: Multiple OD-SSB carriers can be discussed later.

Proposal 11: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), prefer option 2.

Proposal 12: For the scenario 1 with Alt Time-C1 under case 2, when the OD-SSB and AO-SSB periodicity is the same, the effective measurement periodicity is half of either the OD-SSB or AO-SSB periodicity. When OD-SSB and AO-SSB periodicity is different, the effective measurement periodicity could be always the minimum between AO-SSB and OD-SSB, i.e., min(AO-SSB, OD-SSB).

Proposal 13: For Alt Time-C2 under case 2, measurement periodicity could be based on the OD-SSB periodicity.

Proposal 14: RAN4 to deprioritize the intra-band co-located CA for OD-SSB based SCell operation, at least in FR1.

Proposal 15: For intra-band non-collocated CA in FR1, only consider UE with similar behaviour as supporting intraBandNR-CA-non-collocated-r18

- References

[1]  R4-2420098,	WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4 #113

[2]  R4-2420192,	Reply LS on timeline for On-demand SSB operation on SCell, Ericsson, RAN4 #113

[3]  R4-2500010,	Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell, RAN1. RAN4 #114

[4]  R4-2502598,  WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4#114